| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0127 |
| --- | --- |

| Title: | CE2-related: Improved method for entropy coding MVD/BVD |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shawmin Lei MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shih-Ta Hsiang +886-3-5670766 ext. 24685 {shih-ta.hsiang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modified method for entropy coding the MVD/BVD vectors. The proposal attempts to further improve our former method in JCTVC-S0162 in coding efficiency. The modified method also reduces the maximal number of the context-coded bins and can meet the worst-case constraint on the number of context-coded bins compared with SCM-3.0. The proposed method reportedly achieves average Luma BD-rate savings of 0.8%, 1.5%, and 1.8% for lossy coding YUV, text & graphics with motion, 1080p & 720p sequences for the AI, RA, LB settings, respectively, under the common test conditions. When allowing a use of up to 4 context-coded bins per vector component, the proposed method reportedly achieves average Luma BD-rate savings of 1.1%, 1.9%, and 2.2% for lossy coding YUV, text & graphics with motion, 1080p & 720p sequences for the AI, RA, LB settings, respectively, under the common test conditions.

# Introduction

This contribution proposes a modified method for entropy coding the MVD/BVD vectors. The proposal attempts to further improve our former method [1] in coding efficiency. The modified method also reduces the maximal number of the context-coded bins and can meet the worst-case constraint on the number of context-coded bins compared with SCM-3.0.

# Proposed method

This contribution proposes to utilize a universal method for entropy coding the MVD/BVD vectors. This method represents an unsigned source sample value in a binary form by signaling its most significant bit (MSB) followed by its refinement bits. The MSB index for the sample value equal to 0 is assigned to -1. The prefix part of the resulting codeword represents the MSB index plus 1, denoted by msb_id_plus1. The prefix part msb_id_plus1 of a unsigned syntax x is given by

msb_id_plus1 = Floor ( Log2( x ) ) + 1,  	if x > 0; 
                                      0,   	otherwise.

The suffix part, denoted by refinement_bits, represents the value derived from the refinement bits and is present if msb_id_plus1 > 1. The decoded syntax value x is given by

x = ( 1 << ( msb_id_plus1 – 1 ) ) + refinement_bits,  if msb_id_plus1 > 1;

msb_id_plus1,                          otherwise.

For a signed source sample, the proposed method can be applied to coding the absolute value of the source sample together with the additionally coded sign bit.

In our former contribution in [1], this universal coding method was employed to code each of the MVD/BVD components. The prefix part, msb_id_plus1, was binarized by the unary code and the suffix part, refinement_bits, was binarized by the fixed length code, leading to exactly the same binarization as that of MVD coding specified in HEVC version 2. The method can use up to 7 context-coded bins for coding each component.

In this contribution, a new method has been developed for coding the resulting prefix part of the source sample value in order to reduce the number of the context-coded bins. The propose method first codes a syntax flag msb_idx_p1_gr0_flag to indicate if the prefix value is greater than 0. If the flag is true, then the remaining prefix part is tested against a threshold that further divides the remaining prefix part into the two sub-prefix parts. A second flag msb_idx_p1_grT_flag is then coded to indicate if the prefix value msb_id_plus1 is greater than the threshold msb_idx_thre. If the flag is true, then (msb_id_plus1 -1 - msb_idx_thre) is coded in the unary code, otherwise (msb_idx_thre - msb_id_plus1) is coded in the truncated unary code. The threshold value, msb_idx_thre, is coded in the slice header for each vector component.

Three contexts are allocated for entropy coding syntax flags msb_idx_p1_gr0_flag[c], c = 0, 1, in the CABAC mode, 1 context for msb_idx_p1_gr0_flag[0] and 2 contexts for msb_idx_p1_gr0_flag[1]. Context selection for entropy coding msb_idx_p1_gr0_flag[1] is conditioned on the coded value of msb_idx_p1_gr0_flag[0]. The same method was also independently developed in [4]. The syntax flag msb_idx_p1_grT_flag is coded in either the Bypass mode or the CABAC mode. The two sub-prefix parts can also be entropy coded with model selection conditioned on the bin index. The sub-prefix bin string is coded in the CABAC mode if the current bin index is less than a pre-determined threshold value, in the bypass mode, otherwise. Separate context sets are allocated for coding MVD and BVD.

When more context-coded bins are employed for coding the prefix value of the MVD/BVD component, the maximal number of context-coded bins can be increased in comparison with SCM-3.0. However, practical applications are mainly concerned about the worst-case parsing throughput rate for a processing unit, such as a CU or CTU. We can still easily meet the existing CABAC parsing throughput constraint for a processing unit despite an increase in the max number of the regular bins by the proposed method under the worst case. In our implementation, the encoder will track the number of regular bins consumed in the current CU subject to the rate-distortion optimization test. The encoder simply skips the CU configuration leading to the number of the regular bins greater than the max number of the regular bins subject to the specified throughput constraint. When no CU configuration is found to meet the constraint at the end of the RD-optimized mode decision process due to early skip of some CU modes, then the current CU will be coded in the Intra 2Nx2N mode.

# Experimental results

The proposed method has been integrated into the SCC reference software SCM-3.0 [2]. All the tests were conducted according to the common test conditions specified in [3].

Table 1 summarizes the lossy simulation results of the proposed method using up to 1 regular bin for BVD and 2 regular bins for MVD per component. In this way, the maximal number of the context-coded bins is not increased compared with the current SCM-3.0. Table 2 summarizes the lossy simulation results of the proposed method using up to 4 regular bins per BVD/MVD component.

The detailed simulation results can be found in the attached spreadsheets.

Table 1: Lossy BD bitrate results compared with the SCM anchor for lossy coding, up to 1 regular bin for BVD and 2 regular bins for MVD per component

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.8% | -0.7% | -0.7% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.8% | -0.8% | -0.7% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |
| Dec Time[%] | 114% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.3% | -1.3% | -1.3% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.1% |
| RGB, Animation, 720p | -0.2% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -1.5% | -1.5% | -1.6% |
| YUV, mixed content, 1440p & 1080p | -0.2% | -0.2% | -0.2% |
| YUV, Animation, 720p | -0.2% | -0.2% | 0.1% |
| YUV, camera captured, 1080p | -0.2% | -0.1% | -0.3% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 125% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.8% | -1.6% | -1.6% |
| RGB, mixed content, 1440p & 1080p | -0.2% | 0.0% | -0.4% |
| RGB, Animation, 720p | -0.2% | -0.1% | -0.2% |
| RGB, camera captured, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -1.8% | -1.7% | -1.8% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.6% | -0.6% |
| YUV, Animation, 720p | 0.0% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | -0.2% | -0.1% | -0.1% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 127% |  |  |

Table 2: Lossy BD bitrate results compared with the SCM anchor for lossy coding, up to 4 regular bins per vector component

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.9% | -0.9% | -0.9% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.3% | -0.3% |
| RGB, Animation, 720p | -0.1% | -0.1% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.1% | -1.0% | -1.0% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.4% | -0.3% |
| YUV, Animation, 720p | -0.1% | -0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 114% |  |  |
| Dec Time[%] | 115% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.7% | -1.7% | -1.7% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.4% | -0.2% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | -0.3% | -0.2% | -0.2% |
| YUV, text & graphics with motion, 1080p & 720p | -1.9% | -1.9% | -1.9% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.3% | -0.4% |
| YUV, Animation, 720p | -0.2% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | -0.3% | -0.2% | -0.3% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 110% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.0% | -1.9% | -1.8% |
| RGB, mixed content, 1440p & 1080p | -0.5% | -0.4% | -0.4% |
| RGB, Animation, 720p | -0.2% | -0.3% | -0.3% |
| RGB, camera captured, 1080p | -0.2% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -2.2% | -2.2% | -2.1% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.8% | -0.5% |
| YUV, Animation, 720p | 0.0% | 0.3% | 0.1% |
| YUV, camera captured, 1080p | -0.2% | 0.0% | 0.1% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 117% |  |  |

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S.-T. Hsiang, T.-D. Chuang, S. Lei, “CE1: Results of Tests 1.4, 2.2, & 3.3 on unified BVD & MVD coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-S0162, Strasbourg, FR, 17–24 Oct. 2014.

- SCM-3.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-3.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1015, Strasbourg, FR, 17–24 Oct. 2014.

- S.H. Kim, A. Segall, “Non-CE2: Adding a context for significance flag in BVD,” Document of Joint Collaborative Team on Video Coding, JCTVC-T0114, Geneva, CH, 10–18 Feb. 2015