The proposed modifications are based on HEVC Screen Content Coding: Draft 2 (JCTVC-S1005-v1). The inserted parts are marked in yellow. The deleted parts are marked in pink.

##### Sequence parameter set scc syntax

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_flag | u(1) |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| mvd_msb_idx_coding_enabled_flag | u(1) |
| } |  |

#### Prediction unit syntax

| prediction_unit( x0, y0, nPbW, nPbH ) { | Descriptor |
| --- | --- |
| if( cu_skip_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else if( intra_bc_flag[ x0 ][ y0 ] ) /* Intra BC*/ |  |
| bvd_coding( x0, y0, 2 ) |  |
| mvd_coding( x0, y0, 2 ) |  |
| else { /* MODE_INTER */ |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding( x0, y0, 0 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| MvdL1[ x0 ][ y0 ][ 0 ] = 0 |  |
| MvdL1[ x0 ][ y0 ][ 1 ] = 0 |  |
| } else |  |
| mvd_coding( x0, y0, 1 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

#### Block vector difference syntax

| bvd_coding ( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_bvd_greater0_flag[ 0 ] | ae(v) |
| abs_bvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_bvd_greater0_flag[ 0 ] ) { |  |
| abs_bvd_minus1[ 0 ] | ae(v) |
| bvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_bvd_greater0_flag[ 1 ] ) { |  |
| abs_bvd_minus1[ 1 ] | ae(v) |
| bvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

#### Motion vector difference syntax

| mvd_coding( x0, y0, refList ) { | Descriptor |
| --- | --- |
| if (mvd_msb_idx_coding_enabled_flag) { |  |
| signed_vector_msb_index_coding( x0, y0, refList ) |  |
| } |  |
| else { |  |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |
| } |  |

#### 7.3.8.12  Signed vector syntax

| signed_vector_msb_index_coding( x0, y0, refList ) { | Descriptor |
| --- | --- |
| abs_msb_idx_p1_gr0_flag[0] | ae(v) |
| abs_msb_idx_p1_gr0_flag[1] | ae(v) |
| if ( abs_msb_idx_p1_gr0_flag[0] = = 0 ) |  |
| abs_msb_idx_p1[0] = 0 |  |
| else { |  |
| abs_msb_idx_p1_grT_flag[0] | ae(v) |
| if ( abs_ msb_idx_p1_grT_flag[0] = = 0 ) { |  |
| thre _minus_abs_msb_idx[0] | ae(v) |
| abs_msb_idx_p1[0] = abs_msb_idx_thre[0] – thre _minus_abs_msb_idx[0] |  |
| } else { |  |
| abs_msb_idx_p1_minus_thre[0] | ae(v) |
| abs_msb_idx_p1[0] = abs_msb_idx_p1_minus_thre[0] + abs_ msb_idx_thre[0] + 1 |  |
| } |  |
| } |  |
| if ( abs_msb_idx_p1_gr0_flag[1] = = 0 ) |  |
| abs_msb_idx_p1[1] = 0 |  |
| else { |  |
| abs_msb_idx_p1_grT_flag[1] | ae(v) |
| if (abs_ msb_idx_p1_grT_flag[1] = = 0 ) { |  |
| thre _minus_abs_msb_idx[1] | ae(v) |
| abs_msb_idx_p1[0] = abs_msb_idx_thre[1] - thre _minus_abs_msb_idx[1] |  |
| } else { |  |
| abs_msb_idx_p1_minus_thre[1] | ae(v) |
| abs_msb_idx_p1[1] = abs_msb_idx_p1_minus_thre[1] + abs_ msb_idx_thre[1] + 1 |  |
| } |  |
| } |  |
| if(abs_msb_idx_p1 [0] > 0 ) { |  |
| if(abs_msb_idx_p1 [0 ] > 1) |  |
| abs_refinement_bits [0] | ae(v) |
| sign_flag [ 0] | ae(v) |
| } |  |
| if( abs_msb_idx_p1 [1] > 0 ) { |  |
| if( abs_msb_idx_p1 [1 ] > 1 ) |  |
| abs_refinement_bits [1] | ae(v) |
| sign_flag [ 1] | ae(v) |
| } |  |
| } |  |