| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0128 |
| --- | --- |

| Title: | CE-3: Results of Test B.5 on entropy coding ISC syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Shawmin Lei MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shih-Ta Hsiang +886-3-5670766 ext. 24685 {shih-ta.hsiang, peter.chuang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE3 Test B.5 on entropy coding using the proposed method in JCTVC-R0137 and JCTVC-S0165. The proposed method employed for coding the offset vectors and the string lengths of 2D matching reportedly achieves average Luma BD-rate savings of 0.5%, 0.2%, and 0.1% for lossy coding YUV, text & graphics with motion, 1080p & 720p sequences for the AI, RA, LB settings, respectively, compared with the SCM-3.0 anchor results under CE3 Test Condition 1. The proposed method reportedly achieves average Luma BD-rate savings of 0.8%, 0.4%, and 0.1% for lossy coding YUV, text & graphics with motion, 1080p & 720p sequences for the AI, RA, LB settings, respectively, compared with the SCM-3.0 anchor results under CE3 Test Condition 2.

# Introduction

This contribution reports the results of CE3 Test B.5 on entropy coding using the proposed method in JCTVC-R0137 [1] and JCTVC-S0165 [2]. For completeness, the proposed algorithm is provided in the next section.

# Proposed method

The proposal employs a unified method for coding new syntax elements resulting from the intra string copy (ISC) tools [3] by using a universal entropy coding scheme. This entropy coding method, previously employed in [3] for coding the IBC block vector difference, represents a syntax element in a binary value by sending its most significant bit (MSB) index followed by its refinement bits. The bin string for a binarized syntax value is thus a concatenation of a prefix bin string and a suffix bin string. The prefix string represents the MSB index plus 1, or msb_id_plus1, of a syntax value in a unary code, or a truncated unary code if the maximal syntax value is known. msb_id_plus1 of a unsigned syntax x is given by

msb_id_plus1 =  Floor ( Log2( x ) ) + 1,  	if x > 0; 
                                      0,   	otherwise.

The suffix bin string represents the refinement bits and is present if msb_id_plus1 > 1. The value of refinement bits, denoted by refinement_bits, is coded in a fixed-length binary code with the string length equal to msb_id_plus1 – 1. The decoded syntax value x is given by

x  =  (1 << ( msb_id_plus1-1)) + refinement_bits,  if msb_id_plus1 > 1;

msb_id_plus1,  otherwise.

This entropy coding method is employed for coding the syntax elements dictionary_pred_length_minus1, dictionay_pred_offset_minus1, and dictionay_pred_offsetX, dictionay_pred_offsetY for representing length, offset, and offset vector, respectively, of the matched string employed in [2]. For the 2-D offset vectors, the proposed method is applied to entropy coding the absolute value of each vector component. The mapping tables employed for context selection are provided in Tables below.

Specification of ctxIdx for coding dictionary_pred_length_minus1

Specification of ctxIdx for coding dictionay_pred_offset_minus1

Specification of ctxIdx for coding dictionay_pred_offsetX and dictionay_pred_offsetY

# Experimental results

The proposed method has been integrated into the CE3 ISC reference software [3]. All the tests were conducted according to CE3 test conditions specified in [3].

Tables 1 & 2 summarize the simulation results of Test B.5 using the proposed method for coding the offset vectors and the string length of 2D matching compared with the SCM-3.0 anchor results for lossy & lossless coding, respectively, under Test Condition 1 in [3]. Tables 3 & 4 summarize the simulation results of Test B.5 compared with the SCM-3.0 anchor results for lossy & lossless coding, respectively, under Test Condition 2 in [3].

The detailed simulation results can be found in the attached spreadsheets.

Table 1: Lossy BD bitrate results compared with the SCM-3.0 anchor for lossy coding, Test Condition 1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.6% | -0.6% | -0.6% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.6% | -0.5% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 139% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.4% | -0.4% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.3% | -0.5% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.2% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 112% |  |  |
| Dec Time[%] | 94% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.2% | -0.1% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.3% | -0.2% |
| YUV, Animation, 720p | 0.2% | 0.3% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 108% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2: Bit-rate savings compared with the SCM-3.0 anchor for lossless coding, Test Condition 1

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -2.7% | -3.1% | -8.4% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -0.4% | -0.8% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.7% | -3.0% | -8.3% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.4% | -0.7% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 154% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -1.1% | -1.6% | -5.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.0% | -1.5% | -4.9% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 112% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -1.0% | -1.1% | -3.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.9% | -1.1% | -3.0% | -0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

Table 3: Lossy BD bitrate results compared with the SCM-3.0 anchor for lossy coding, Test Condition 2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.0% | -1.1% | -1.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.8% | -0.9% | -0.9% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 133% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.6% | -0.7% | -0.7% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.6% | -0.7% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.2% | 0.1% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.2% | -0.2% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.0% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.2% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.3% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.3% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 104% |  |  |

Table 4: Bit-rate savings compared with the SCM-3.0 anchor for lossless coding, Test Condition 2

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -4.3% | -4.6% | -10.8% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.7% | -0.8% | -1.2% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -4.3% | -4.6% | -11.1% | -0.3% |
| YUV, mixed content, 1440p & 1080p | -0.7% | -0.8% | -1.3% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 147% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -2.3% | -2.6% | -6.8% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.0% | -2.6% | -6.9% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 111% |  |  |  |
| Dec Time[%] | 102% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -2.2% | -1.9% | -4.8% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.9% | -1.8% | -5.0% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 110% |  |  |  |
| Dec Time[%] | 102% |  |  |  |

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S.-T. Hsiang, T.-D. Chuang, S. Lei, “Non-SCCE4: Unified method for coding string matching syntax using a universal entropy coding scheme,” Document of Joint Collaborative Team on Video Coding, JCTVC-R0137, Sapporo, Japan, 30 June – 9 July 2014.

- S.-T. Hsiang, T.-D. Chuang, S. Lei, “CE10 Results of Test 6 on unified method for entropy coding intra string copy syntax elements,” Document of Joint Collaborative Team on Video Coding, JCTVC-S0165, Strasbourg, FR, 17–24 Oct. 2014.

- C.-C. Chen, et al, “Description of Core Experiment 3 (CE3): Intra Line Copy and Intra String Copy,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1103, Strasbourg, FR, 17–24 Oct. 2014.

# Text Modifications

The text provided in this section is based on JCTVC-S1005 & JCTVC-S0041.

The text supports both 2D matching dictionary mode (supporting adaptive horizontal and vertical scanning) and 1D matching dictionary mode.

##### 7.3.2.2.3	Sequence parameter set scc extensions syntax

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| dictionary_mode_enabled_flag | u(1) |
| } |  |

#### 7.3.8.5	Coding unit syntax

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag ) |  |
| cu_transquant_bypass_flag | ae(v) |
| if( slice_type != I ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| nCbS = ( 1 << log2CbSize ) |  |
| if( cu_skip_flag[ x0 ][ y0 ] ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else { |  |
| if( intra_block_copy_enabled_flag ) |  |
| intra_bc_flag[ x0 ][ y0 ] | ae(v) |
| if(!intra_bc_flag[ x0 ][ y0 ] && dictionary_mode_enabled_flag ) |  |
| dictonary_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( slice_type != I && !dictionary_mode_flag[ x0 ][ y0 ]) |  |
| pred_mode_flag | ae(v) |
| if( palette_mode_enabled_flag && ChromaArrayType = = 3 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) |  |
| palette_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( palette_mode_flag[ x0 ][ y0 ] ) |  |
| palette_coding( x0, y0, nCbS ) |  |
| else { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ log2CbSize = = MinCbLog2SizeY ) |  |
| part_mode | ae(v) |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && !intra_bc_flag[ x0 ][ y0 ] ] && !dictionary_mode_flag[ x0 ][ y0 ] &&) { |  |
| if( PartMode = = PART_2Nx2N && pcm_enabled_flag && log2CbSize >= Log2MinIpcmCbSizeY && log2CbSize <= Log2MaxIpcmCbSizeY ) |  |
| pcm_flag[ x0 ][ y0 ] | ae(v) |
| if( pcm_flag[ x0 ][ y0 ] ) { |  |
| while( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | f(1) |
| pcm_sample( x0, y0, log2CbSize ) |  |
| } else { |  |
| pbOffset = ( PartMode = = PART_NxN ) ? ( nCbS / 2 ) : nCbS |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] | ae(v) |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| if( prev_intra_luma_pred_flag[ x0 + i ][ y0 + j ] ) |  |
| mpm_idx[ x0 + i ][ y0 + j ] | ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| if( ChromaArrayType = = 3 ) |  |
| for( j = 0; j < nCbS; j = j + pbOffset ) |  |
| for( i = 0; i < nCbS; i = i + pbOffset ) |  |
| intra_chroma_pred_mode[ x0 + i ][ y0 + j ] | ae(v) |
| else if( ChromaArrayType != 0 ) |  |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| if( PartMode = = PART_2Nx2N ) |  |
| prediction_unit( x0, y0, nCbS, nCbS ) |  |
| else if( PartMode = = PART_2NxN ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS, nCbS / 2 ) |  |
| } else if( PartMode = = PART_Nx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS ) |  |
| } else if( PartMode = = PART_2NxnU ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 4 ), nCbS, nCbS * 3 / 4 ) |  |
| } else if( PartMode = = PART_2NxnD ) { |  |
| prediction_unit( x0, y0, nCbS, nCbS * 3 / 4 ) |  |
| prediction_unit( x0, y0 + ( nCbS * 3 / 4 ), nCbS, nCbS / 4 ) |  |
| } else if( PartMode = = PART_nLx2N ) { |  |
| prediction_unit( x0, y0, nCbS / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS / 4 ), y0, nCbS * 3 / 4, nCbS ) |  |
| } else if( PartMode = = PART_nRx2N ) { |  |
| prediction_unit( x0, y0, nCbS * 3 / 4, nCbS ) |  |
| prediction_unit( x0 + ( nCbS * 3 / 4 ), y0, nCbS / 4, nCbS ) |  |
| } else { /* PART_NxN */ |  |
| prediction_unit( x0, y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0, nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0, y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| prediction_unit( x0 + ( nCbS / 2 ), y0 + ( nCbS / 2 ), nCbS / 2, nCbS / 2 ) |  |
| } |  |
| } |  |
| } else if (dictionary_mode_flag[ x0 ][ y0 ] ) { |  |
| dictionary_mode( x0, y0, log2CbSize) |  |
| if( !pcm_flag[ x0 ][ y0 ] && !dictionary_mode_flag[ x0 ][ y0 ]) { |  |
| if( ( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && !( PartMode = = PART_2Nx2N && merge_flag[ x0 ][ y0 ] ) ) ¦ ¦ ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && intra_bc_flag[ x0 ][ y0 ] ) ) |  |
| rqt_root_cbf | ae(v) |
| if( rqt_root_cbf ) { |  |
| if( residual_adaptive_colour_transform_enabled_flag && ( CuPredMode[ x0 ][ y0 ] = = MODE_INTER ¦ ¦ intra_bc_flag[ x0 ][ y0 ] ¦ ¦ intra_chroma_pred_mode[ x0 ][ y0 ] = = 4 ) ) |  |
| cu_residual_act_flag |  |
| MaxTrafoDepth = ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ? ( max_transform_hierarchy_depth_intra + IntraSplitFlag ) : max_transform_hierarchy_depth_inter ) |  |
| transform_tree( x0, y0, x0, y0, log2CbSize, 0, 0 ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.8.X Dictionary mode syntax

| dictionary_mode( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| nCbS = ( 1 << log2CbSize ) |  |
| totalPixel = nCbS*nCbS |  |
| currPixel = 0 |  |
| currIndex = 0 |  |
| dictionary_type_flag[ x0 ][ y0 ] | ae(v) |
| dictionary_scan_flag[ x0 ][ y0 ] | ae(v) |
| while (currPixel < totalPixel ) { |  |
| dictionay_pred_mode_flag[ x0 ][ y0 ][ currIndex ] | ae(v) |
| if( dictionay_pred_mode_flag[ x0 ][ y0 ][ currIndex ] ) { |  |
| if( dictionary_type_flag[ x0 ][ y0 ] == 0 ) { |  |
| dictionay_pred_offset_minus1[ x0 ][ y0 ][ currIndex ] | ae(v) |
| dict_pred_offset_coding (dictionay_pred_offset_minus1[ x0 ][ y0 ][ currIndex ]) | ae(v) |
| } else { |  |
| dictionay_pred_offsetX[ x0 ][ y0 ][ currIndex ] | ae(v) |
| dictionay_pred_offsetY[ x0 ][ y0 ][ currIndex ] | ae(v) |
| dict_pred_offset_vector_coding (dictionay_pred_offsetX, dictionay_pred_offsetY) |  |
| } |  |
| dictionary_pred_length_minus1[ x0 ][ y0 ][ currIndex ] | ae(v) |
| dict_pred_length_coding(totalPixel – currPixel - 1 ) |  |
| currPixel += (dictionary_pred_length_minus1 [ x0 ][ y0 ][ currIndex ] + 1) |  |
| } else { |  |
| dictionary_direct_component[ x0 ][ y0 ][ currIndex ][ 0 ] | ae(v) |
| dictionary_direct_component[ x0 ][ y0 ][ currIndex ][ 1 ] | ae(v) |
| dictionary_direct_component[ x0 ][ y0 ][ currIndex ][ 2 ] | ae(v) |
| currPixel += 1 |  |
| } |  |
| currIndex += 1 |  |
| } |  |
| } |  |

#### Dictionary predion offset coding syntax

| dict_pred_offset_coding( x) { | Descriptor |
| --- | --- |
| abs_mvd_msb_idx_p1[ 0] | ae(v) |
| if(abs_mvd_msb_idx_p1 [0] > 0 ) { |  |
| if(abs_mvd_msb_idx_p1 [0] > 1) |  |
| abs_mvd_refinement_bits [0] | ae(v) |
| mvd_sign_flag [0] | ae(v) |
| } |  |
| } |  |

#### Dictionary predion offset vector coding syntax

| dict_pred_offset_vector_coding( x0, y0) { | Descriptor |
| --- | --- |
| abs_mvd_msb_idx_p1[ 0] | ae(v) |
| abs_ mvd_msb_idx_p1[1] | ae(v) |
| if(abs_mvd_msb_idx_p1 [0] > 0 ) { |  |
| if(abs_mvd_msb_idx_p1 [0] > 1) |  |
| abs_mvd_refinement_bits [0] | ae(v) |
| mvd_sign_flag [0] | ae(v) |
| } |  |
| if(abs_mvd_msb_idx_p1 [1]>0 ) { |  |
| if(abs_mvd_msb_idx_p1 [1 ] > 1) |  |
| abs_mvd_refinement_bits [1] | ae(v) |
| mvd_sign_flag [ 1] | ae(v) |
| } |  |
| } |  |

#### Dictionary predion length coding syntax

| dict_pred_length_coding( MaxIscRun) { | Descriptor |
| --- | --- |
| if (MaxIscRun > 0) { |  |
| isc_run_msb_id_plus1 | ae(v) |
| if (isc_run _msb_id_plus1 > 1 ) |  |
| isc_run_refinement_bits | ae(v) |
| dictionary_pred_length_minus1 = (isc_run _msb_id_plus1 > 1 ) ? ((1 << (isc_run _msb_id_plus1–1)) + isc_run_refinement_bits) : isc_run _msb_id_plus1 |  |
| } else |  |
| dictionary_pred_length_minus1 = 0 |  |
| } |  |