| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0129 |
| --- | --- |

| Title: | CE3-related: Improved method for entropy coding offset vectors of 2-D matching |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shawmin Lei MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Shih-Ta Hsiang +886-3-5670766 ext. 24685 {shih-ta.hsiang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modified method for coding the offset vectors of 2D matching. The proposal improves our former method CE3 TestB.5 in coding efficiency and reduces the worst-case number of the context-coded bins. The proposed method integrated with the CE3 ISC software reportedly achieves average Luma BD-rate savings of 0.4%, 0.2% & 0.0% for lossy coding YUV, text & graphics with motion, 1080p & 720p sequences for the AI, RA, & LB settings, respectively, compared with the SCM-3.0 anchor results under CE3 Test Condition 1. The proposed method integrated with the CE3 ISC software reportedly achieves average Luma BD-rate savings of 0.7%, 0.4% & 0.1% for lossy coding YUV, text & graphics with motion, 1080p & 720p sequences for the AI, RA, &LB settings, respectively, compared with the SCM-3.0 anchor results under CE3 Test Condition 2.

# Introduction

This contribution proposes a modified method for coding the offset vectors of 2D matching. The proposal attempts to further improve our former method [1, 2] currently under study of CE3 TestB.5 [3] in coding efficiency. The modified method also reduces the maximal number of the context-coded bins and does not increase the worst-case constraint on the number of context-coded compared with SCM-3.0.

# Proposed method

This contribution proposes to utilize a universal method for entropy coding the offset vectors of 2-D matching. This method represents an unsigned source sample value in a binary form by signaling its most significant bit (MSB) followed by its refinement bits. The MSB index for the sample value equal to 0 is assigned to -1. The prefix part of the resulting codeword represents the MSB index plus 1, denoted by msb_id_plus1. The prefix part msb_id_plus1 of a unsigned syntax x is given by

msb_id_plus1 = Floor ( Log2( x ) ) + 1,  	if x > 0; 
                                     0,   	otherwise.

The suffix part, denoted by refinement_bits, represents the value derived from the refinement bits and is present if msb_id_plus1 > 1. The decoded syntax value x is given by

x = ( 1 << ( msb_id_plus1 – 1 ) ) + refinement_bits,  if msb_id_plus1 > 1;

msb_id_plus1,                             otherwise.

For a signed source sample, the proposed method can be applied to coding the absolute value of the source sample together with the additionally coded sign bit.

In our former contribution in [2], this universal coding method was employed to code each of the offset vector components of 2-D matching. The prefix part, msb_id_plus1, was binarized by the unary code and the suffix part, refinement_bits, was binarized by the fixed length code, leading to exactly the same binarization as that of MVD coding specified in HEVC version 2. The method can use up to 7 context-coded bins for coding each component.

In this contribution, a new method is developed for coding the resulting prefix part of the source sample value in order to reduce the number of the context-coded bins. The propose method first codes a flag msb_idx_p1_gr0_flag to indicate if the prefix value is greater than 0. If the flag is true, then the remaining prefix part is tested against a threshold that further divides the remaining prefix part into the two sub-prefix parts. A second flag msb_idx_p1_grT_flag is then coded to indicate if the prefix value msb_id_plus1 is greater than the threshold msb_idx_thre. If the flag is true, then (msb_id_plus1 -1 - msb_idx_thre) is coded in the unary code, otherwise (msb_idx_thre - msb_id_plus1) is coded in the truncated unary code. The threshold value, msb_idx_thre, is coded in the slice header for each vector component.

Our coding method is further specially designed to exploit the fact that starting position of the reference string should be always located at the reconstructed picture region. The flags (msb_idx_p1_gr0_flag[0], msb_idx_p1_gr0_flag[1]) are jointly coded using a syntax element msb_idx_p1_gr0_flag_xy with the value equal to 0, 1, or 2, corresponding to (msb_idx_p1_gr0_flag[0] = 1, msb_idx_p1_gr0_flag[1] =0), (msb_idx_p1_gr0_flag[0] = 0, msb_idx_p1_gr0_flag[1] = 1), and (msb_idx_p1_gr0_flag[0] = 1, msb_idx_p1_gr0_flag[1] = 1), respectively. The syntax value is binarized by the truncated unary code with the maximum value equal to 2 and the resulting bin string is entropy coded in the CABAC mode. The two contexts are employed and context selection is determined by the bin index. The signs of the two offset vector components are coded only if (msb_idx_p1_gr0_flag[0] == 1 && msb_idx_p1_gr0_flag[1] == 1). Otherwise, the signs are inferred to be positive. We use a new syntax element dictionay_pred_offset_sign_xy with the value equal to 0, 1, or 2, corresponding to (dictionay_pred_offsetX > 0, dictionay_pred_offsetY > 0), (dictionay_pred_offsetX < 0, dictionay_pred_offsetY > 0), and (dictionay_pred_offsetX > 0, dictionay_pred_offsetY < 0), respectively. The syntax value is binarized by the truncated unary code with the maximum value equal to 2 and the resulting bin string is coded in the bypass mode.

# Experimental results

The proposed method has been integrated into the CE3 ISC reference software [3]. All the tests were conducted according to CE3 test conditions specified in [3].

For generating the experimental results listed in Tables 1-4, the proposed method codes the two syntax flags in the CABAC mode and the two sub-fix parts in the bypass mode and may use up to 2 context-coded bins for coding each vector component. Tables 1 & 2 summarize the simulation results of the proposed method for coding the offset vectors of 2D matching compared with the SCM-3.0 anchor results for lossy & lossless coding, respectively, under Test Condition 1 in [3]. Tables 3 & 4 summarize the simulation results of the proposed method for coding the offset vectors of 2D matching compared with the SCM-3.0 anchor results for lossy & lossless coding, respectively, under Test Condition 2 in [3].

The detailed simulation results can be found in the attached spreadsheets.

Table 1: Lossy BD bitrate results compared with the SCM-3.0 anchor for lossy coding, Test Condition 1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.5% | -0.5% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.4% | -0.4% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.1% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 139% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.1% | 0.2% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.3% | -0.4% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.0% | 0.2% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.3% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| Enc Time[%] | 113% |  |  |
| Dec Time[%] | 95% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.2% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.3% | 0.1% | 0.4% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2: Bit-rate savings compared with the SCM-3.0 anchor for lossless coding, Test Condition 1

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -2.7% | -3.1% | -8.5% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -0.4% | -0.7% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.6% | -2.9% | -8.4% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.3% | -0.4% | -0.7% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 158% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -1.0% | -1.6% | -5.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.9% | -1.5% | -5.0% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 112% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.9% | -1.1% | -3.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.8% | -1.0% | -2.8% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 108% |  |  |  |
| Dec Time[%] | 117% |  |  |  |

Table 3: Lossy BD bitrate results compared with the SCM-3.0 anchor for lossy coding, Test Condition 2

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.9% | -1.0% | -1.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.7% | -0.8% | -0.8% |
| YUV, mixed content, 1440p & 1080p | 0.0% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 131% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.6% | -0.7% | -0.7% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.1% | 0.2% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -0.4% | -0.5% | -0.6% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.3% | 0.1% |
| YUV, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 112% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.3% | -0.2% |
| RGB, mixed content, 1440p & 1080p | 0.2% | 0.2% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.1% | -0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.3% |
| YUV, Animation, 720p | 0.1% | 0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 105% |  |  |

Table 4: Bit-rate savings compared with the SCM-3.0 anchor for lossless coding, Test Condition 2

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -4.2% | -4.5% | -10.7% | -0.2% |
| RGB, mixed content, 1440p & 1080p | -0.7% | -0.8% | -1.2% | -0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -4.2% | -4.4% | -10.9% | -0.2% |
| YUV, mixed content, 1440p & 1080p | -0.7% | -0.8% | -1.2% | -0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 143% |  |  |  |
| Dec Time[%] | 98% |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -2.2% | -2.5% | -6.6% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.9% | -2.4% | -6.7% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 111% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -2.2% | -1.8% | -4.7% | -0.1% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.8% | -1.7% | -4.7% | -0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 109% |  |  |  |
| Dec Time[%] | 116% |  |  |  |

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- S.-T. Hsiang, T.-D. Chuang, S. Lei, “CE10 Results of Test 6 on unified method for entropy coding intra string copy syntax elements,” Document of Joint Collaborative Team on Video Coding, JCTVC-S0165, Strasbourg, FR, 17–24 Oct. 2014.

- S.-T. Hsiang, S. Lei, “CE-3: Results of Test B.5 on entropy coding ISC syntax elements,” Document of Joint Collaborative Team on Video Coding, JCTVC-T0128, Geneva, CH, 10–18 Feb. 2015.

- C.-C. Chen, “Description of Core Experiment 3 (CE3): Intra Line Copy and Intra String Copy,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1103, Strasbourg, FR, 17–24 Oct. 2014.