| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, Switzerland, 10 Feb – 18 Feb 2015 | Document: JCTVC-T0132 |
| --- | --- |

| Title: | Clipping for Cross Component Prediction and Adaptive Colour Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ted Hsieh Vadim Seregin, Jianle Chen Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | + 1-858-651-7357 thsieh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document proposes a clipping at various stages to prevent the overflow condition in screen content coding adaptive color transform and cross component prediction.

# Introduction

HEVC’s screen content coding (SCC) employs two coding tools to exploit the redundancy among three colour components to achieve higher compression ratio, namely adaptive color transform (ACT), and cross component prediction (CCP).

ACT is an in-loop colour-space transform [1], whose forward and inverse colour-space transforms for lossy coding use the YCoCg transform matrices, which are defined as follows:

Forward:  C'0C'1C'2=12120−2−12−1C0C1C24 	Inverse:	  C0C1C2=11−11011−1−1C'0C'1C'2

where the original color space (C0, C1, C2) may correspond to (R, G, B) or (Y, U, V).

CCP is a process to predict chroma (or 2nd and 3rd components) from the luma (or 1st component)[2]. Its equations are shown as follows:

Forward :

  - Y = Y

Inverse:

  - Y = Y

where α could be {}.

Both ACT and CCP are residual based operations, and their corresponding processing order within codec is shown in  Figure 1.

Figure 1. ACT, CCP processing order

The dynamic range analysis along HEVC data paths have been studied in detail in [3],[4]. As illustrated in Figure 2, up to 5 bits of bit-depth expansion can occur in the reconstructed residual where B-bit is the bit-depth of input encode pixels and prediction pixels.

Figure 2. HEVC codec bit depths

If ACT and CCP operations are considered, then the dynamic ranges expansion is shown in Figure 3, where IACT stands for Inverse ACT, ICCP stands for Inverse CCP.

Figure 3. Bitdepth information for screen content coding’s possible configurations

From the Figure 3, it’s clear to see that bit-depth after Inverse Transform (IT) is increased to (B+7) bits which is due to addition of forward CCP operation and forward ACT operations. This could increase chroma residual (residual delta) one more bit to (B+2) bits.

As can be seen from the bit depth analysis in Figure 3, bit depth increase at the input of the transform may have dynamic range impacts along the data path, like transpose buffer, ICCP, and IACT, which could result in higher implementation cost, and in general may not be desirable in the implementation.

# Proposed methods

## Normative encoder restriction to maintain dynamic range

In this contribution, it is proposed to keep bit depth unchanged when ACT and CCP tools are enable.  A clipping operation at the encoder is proposed to apply after forward CCP operation as Clip_A shown in Figure 4 (a) and (b). It is proposed to clip the dynamic range of the output of the CCP to B+1 bits, where B-bit is the bit-depth of input encode pixels and prediction pixels so its dynamic range can be brought back to the HEVC version 2 limits.

- CCP only

(b) ACT + CCP

Figure 2. Proposed clipping after forward CCP and its dynamic range after clipping

## Normative decoder restriction to maintain dynamic range

In this alternative solution, it is proposed to clip the dynamic range of the input to IACT to B+n1 bits, where B-bit is the bit-depth of input encode pixels and prediction pixels. The clipping location is as Clip_B shown in Figure 5.

Figure 3. Proposed clipping locations at decoder side

# Simulation results

Two sets of coding performance comparisons are reported. One is the results for clipping at forward CCP output (before transform), the other one is for clipping at the input of inverse ACT.

## Coding performance with encoder restriction

In this section, the experimental results of the proposed method with encoder restriction are presented.

## Coding performance with normative decoder restriction

In this section, the experimental results of the proposed method with normative decoder restriction are presented.

# References

- L. Zhang, J. Chen, J. Sole, M. Karczewicz, X. Xiu, Y. He, Y. Ye, SCCE5 Test 3.2.1: In-loop color-space transform, JCTVC-R0147, August 2014, Sapporo, Japan.

- R. Joshi, J. Xu, HEVC Screen Content Coding Draft Text 2, JCTVC-S1005, Oct. 2014

- C. Yeo ,Y.H. Tan, and Z. Li “Dynamic Range Analysis in High Efficiency Video Coding Residual Coding and Reconstruction,” IEEE Trans. Circuits Syst. Video Technol., vol. 23, no. 7, pp. 1131–1136, July. 2013.

- M. Zhou, AHG7: IDCT Output Range After T+Q+IQ+IT With Valid Residual Inputs, document JCTVC-G856, Joint Collaborative Team on Video Coding (JCT-VC), Nov. 2011.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).