| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0134 |
| --- | --- |

| Title: | Palette SPS syntax cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Jie Zhao, Seung-Hwan Kim, Andrew Segall 5750 NW Pacific Rim Blvd Camas WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com jzhao@sharplabs.com kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

It is asserted that the current palette design results in maximum palette table predictor sizes that are always greater than or equal to the maximum palette table size. The SPS syntax elements corresponding to maximum palette size and maximum palette predictor size do not take this fact into consideration. A cleanup of the SPS syntax elements and their semantics is therefore proposed that does not allow the maximum palette predictor size to be smaller than maximum palette table size is proposed.

# Introduction

In the existing palette coding design the palette predictor size is always greater than or equal to the palette size. However, the current draft JCTVC-S1005 [1] does not prevent syntax elements palette_max_predictor_size from taking on values smaller than palette_max_size. When such values are signalled then:

- In the decoding process for palette mode (clause 8.4.5.2.8 in [1]); the palette table predictor is allowed to contain as many as palette_max_size entries violating the current semantics of palette_max_predictor_size.

- Moreover, the palette predictor stuffing operation is always bypassed when current palette size is greater than palette_max_predictor_size.

See excerpt from clause 8.4.5.2.8 of [1] below illustrating the above points:

The variable PreviousPaletteSize is set equal to the CurrentPaletteSize. Variables PredictorPaletteSize and PredictorPaletteEntries are modified as follows:

for( i = 0; i < currentPaletteSize; i++ ) 
	for( cIdx = 0; cIdx < 3; cIdx++ ) 
		newPredictorPaletteEntries[ cIdx ][ i ] = CurrentPaletteEntries[ cIdx ][ i ]
newPredictorPaletteSize = currentPaletteSize 
for( i = 0; i < PredictorPaletteSize  &&  newPredictorPaletteSize < palette_max_predictor_size; i++ )
	if( PalettePredictorEntryReuseFlag[ i ]  ! =  1 ) {
		for( cIdx = 0; cIdx < 3; cIdx++ ) 		(8-1)
			newPredictorPaletteEntries[ cIdx ][ newPredictorPaletteSize ] = PredictorPaletteEntries[ cIdx ][ i ]
		newPredictorPaletteSize ++ 
	}
for( cIdx = 0; cIdx < 3; cIdx++ ) 
	for( i = 0; i < newPredictorPaletteSize; i++ ) 
		PredictorPaletteEntries[ cIdx ][ i ] = newPredictorPaletteEntries[ cIdx ][ i ]
PredictorPaletteSize = newPredictorPaletteSize

Consequently, the current signaling in SPS of these palette related syntax elements needs cleanup.

# Proposal

Following two cleanup options are proposed (changes marked in yellow):

## Cleanup Option 1

| sps_scc_extensions( ) { | Descriptor |
| --- | --- |
| intra_block_copy_enabled_flag | u(1) |
| palette_mode_enabled_flag | u(1) |
| if( palette_mode_enabled_flag ) { |  |
| palette_max_size | ue(v) |
| delta_palette_max_predictor_size | ue(v) |
| } |  |
| adaptive_mv_resolution_enabled_flag | u(1) |
| intra_boundary_filtering_disabled_flag | u(1) |
| } |  |

delta_palette_max_predictor_size specifies the difference between the maximum predictor palette size and the maximum allowed palette size “palette_max_size”. When not present, the value of delta_palette_max_predictor_size is inferred to be 0.

The variable PaletteMaxPredictorSize is derived as follows:

PaletteMaxPredictorSize = palette_max_size + delta_palette_max_predictor_size

All references in [1] to palette_max_predictor_size should be replaced with PaletteMaxPredictorSize.

## Cleanup Option 2

palette_max_predictor_size specifies the maximum predictor palette size. When not present, the value of palette_max_predictor_size is inferred to be 0. The value of syntax element palette_max_predictor_size shall be greater than or equal to value of syntax element palette_max_size.

# Conclusion

The proposal has identified the need for cleanup of SPS palette syntax elements. Two options are proposed. We recommend that cleanup option 1 be adopted since (a) it is more bit efficient compared to cleanup option 2 (b) it fixes the identified problem and (c) does not allow a rogue encoder to signal incorrect entries.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Rajan Joshi, Jizheng Xu, “High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 2,”, JCTVC-S1005, 18th Meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Sapporo, JP, 30 June – 9 July 2014