| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10 Feb. – 18 Feb. 2015 | Document: JCTVC-T0135 |
| --- | --- |

| Title: | CE3: Results of Test B.2 on Intra String Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kailun Zhou, Liping Zhao, Tao Lin 1239 Siping Ave, Shanghai 200092, China | Tel: Email: | lintao@tongji.edu.cn |
| Source: | Tongji University |  |  |

_____________________________

# Abstract

In the 19th JCT-VC meeting, CE3 (JCTVC-S1103) was formed to study intra string copy (ISC) techniques. This contribution is based on JCTVC-S0250 to evaluate the coding performance of ISC under two configurations of search range: (TC1) FF IBC vs 2CTU ISC and (TC2) 4CTU IBC vs 4CTU ISC. Using SCM30 as anchor, their coding results for YUV TGM and MC AI lossy and lossless are reported respectively as follows:

|  |  | G/Y | B/U | R/V |  |
| --- | --- | --- | --- | --- | --- |
| TC1 | YUV, text & graphics with motion, 1080p & 720p | -1.4% | -1.4% |  | -1.5% |
| Lossy | YUV, mixed content, 1440p & 1080p | -0.6% | -1.0% |  | -1.0% |
| TC2 | YUV, text & graphics with motion, 1080p & 720p | -2.6% | -2.8% |  | -3.0% |
| Lossy | YUV, mixed content, 1440p & 1080p | -1.0% | -1.6% |  | -1.6% |

|  |  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| --- | --- | --- | --- | --- | --- |
| TC1 | YUV, text & graphics with motion, 1080p & 720p | -3.3% | -3.8% | -11.8% | -0.3% |
| Lossless | YUV, mixed content, 1440p & 1080p | -0.6% | -0.6% | -1.2% | -0.1% |
| TC2 | YUV, text & graphics with motion, 1080p & 720p | -5.3% | -5.9% | -16.0% | -0.4% |
| Lossless | YUV, mixed content, 1440p & 1080p | -1.1% | -1.1% | -1.8% | -0.3% |

# Introduction

The Intra String Copy (ISC) technique is a natural extension of both Intra Block Copy (IBC) and Palette. It provides flexible coding features to fit a variety of picture contents. This document shows that a variation of ISC proposed in JCTVC-S0250 can provide quite good coding performance improvement using SCM30 as anchor.

# Experimental Results

The experiments were conducted using HM-16.2+SCM-3.0 software [1] and the screen content common test conditions [2] to collect the BD-rate/bitrate-saving and the encoding and decoding times of the proposed technique under the All-Intra, Random Access, and Low Delay B test conditions. Two different settings to the search range are tested as listed in Table 1.

Table 1. Search range of each tests.

|  | IBC | ISC |
| --- | --- | --- |
| Test condition 1 | Full-frame | 1x2-CTU |
| Test condition 2 | 1x4-CTU | 1x4-CTU |

The following tables show the coding performance.

Table 2. BD-rate reduction of TC1 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.4% | -1.6% | -1.6% | -0.9% | -1.0% | -1.0% | -0.4% | -0.5% | -0.5% |
| RGB, mixed content, 1440p & 1080p | -0.6% | -0.7% | -0.8% | -0.4% | -0.5% | -0.6% | -0.3% | -0.4% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.4% | -1.4% | -1.5% | -0.8% | -0.9% | -1.1% | -0.3% | -0.5% | -0.6% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -1.0% | -1.0% | -0.4% | -0.9% | -0.9% | -0.5% | -1.3% | -1.1% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | -0.2% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 218% |  |  | 141% |  |  | 121% |  |  |
| Dec Time[%] | 107% |  |  | 109% |  |  | 110% |  |  |

Table 3. Bitrate saving of TC1 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving Average | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, TGM, 1080p & 720p | -3.3% | -3.8% | -11.7% | -0.3% | -1.5% | -2.1% | -7.1% | -0.2% | -1.3% | -1.4% | -3.6% | -0.2% |
| RGB, MC, 1440p & 1080p | -0.5% | -0.6% | -1.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -3.3% | -3.8% | -11.8% | -0.3% | -1.4% | -2.1% | -7.1% | -0.2% | -1.2% | -1.3% | -3.6% | -0.2% |
| YUV, MC, 1440p & 1080p | -0.6% | -0.6% | -1.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 278% |  |  |  | 135% |  |  |  | 125% |  |  |  |
| Dec Time[%] | 109% |  |  |  | 106% |  |  |  | 107% |  |  |  |

Table 4. BD-rate reduction of TC2 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.8% | -3.2% | -3.1% | -1.9% | -2.2% | -2.2% | -0.9% | -1.0% | -1.0% |
| RGB, mixed content, 1440p & 1080p | -1.0% | -1.2% | -1.2% | -0.7% | -0.9% | -0.9% | -0.2% | -0.5% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.6% | -2.8% | -3.0% | -1.6% | -2.0% | -2.3% | -0.6% | -1.0% | -0.9% |
| YUV, mixed content, 1440p & 1080p | -1.0% | -1.6% | -1.6% | -0.6% | -1.5% | -1.4% | -0.3% | -1.8% | -1.0% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.0% | 0.0% | 0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% | 0.2% |
| Enc Time[%] | 195% |  |  | 139% |  |  | 120% |  |  |
| Dec Time[%] | 103% |  |  | 107% |  |  | 106% |  |  |

Table 5. Bitrate saving of TC2 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving Average | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, TGM, 1080p & 720p | -5.2% | -5.8% | -15.4% | -0.4% | -3.1% | -3.5% | -9.9% | -0.2% | -3.0% | -2.4% | -5.8% | -0.2% |
| RGB, MC, 1440p & 1080p | -1.0% | -1.1% | -1.7% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -5.3% | -5.9% | -16.0% | -0.4% | -2.9% | -3.6% | -10.2% | -0.2% | -2.7% | -2.3% | -5.4% | -0.2% |
| YUV, MC, 1440p & 1080p | -1.1% | -1.1% | -1.8% | -0.3% | -0.2% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 239% |  |  |  | 124% |  |  |  | 119% |  |  |  |
| Dec Time[%] | 93% |  |  |  | 91% |  |  |  | 99% |  |  |  |

The worst-case memory bandwidth is the same as CE3 base software..

# Conclusion

In this contribution, the coding performance of ISC under two different configurations of the search range is tested. Up to 3.2% average BD-rate reduction in lossy coding and 5.9% average bit-rate reduction in lossless coding are observed in the All-Intra configuration in TC2 (1x4-CTU). Based on the quite good coding performance improvement, we would recommend the committee to adopt this coding mode in SCC draft standard.

# References

- JCT-VC, “HM-16.2+SCM-3.0,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-3.0

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-S1015, Strasbourg, Oct. 2014.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).