| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10 Feb. – 18 Feb. 2015 | Document: JCTVC-T0136 |
| --- | --- |

| Title: | CE3: Results of Test B.4.2 (minimum string length of 20 pixels) on Intra String Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liping Zhao, Kailun Zhou, Tao Lin 1239 Siping Ave, Shanghai 200092, China | Tel: Email: | lintao@tongji.edu.cn |
| Source: | Tongji University |  |  |

_____________________________

# Abstract

One major concern on ISC is the theoretical worst case DDR bandwidth due to small strings. By only allowing strings with length not smaller than 20 pixels outside of 2-CTU reference window, the theoretical worse case DDR memory bandwidth of ISC is between 40%~147% of IBC, and is between 15%~55% of ISC without the constraint, depending on memory access patterns. This contribution is to evaluate the impact of minimum string length of 20 pixels on coding performance of ISC technique proposed in JCTVC-S0250 under two configurations of search range: (TC1) FF IBC vs 2CTU ISC and (TC2) 4CTU IBC vs 4CTU ISC. Using SCM30 as anchor, their coding results for YUV TGM and MC AI lossy coding are reported respectively as follows, and the BD-rate increase is less than 0.1% (comparison with the results of Test B.2 which does not have the minimum string length constraint).

|  |  | G/Y | B/U | R/V |  |
| --- | --- | --- | --- | --- | --- |
| TC1 | YUV, text & graphics with motion, 1080p & 720p | -1.4% | -1.4% |  | -1.5% |
| Lossy | YUV, mixed content, 1440p & 1080p | -0.6% | -1.0% |  | -1.0% |
| TC2 | YUV, text & graphics with motion, 1080p & 720p | -2.5% | -2.7% |  | -2.9% |
| Lossy | YUV, mixed content, 1440p & 1080p | -1.0% | -1.6% |  | -1.5% |

# Introduction

The Intra String Copy (ISC) technique is a natural extension of both Intra Block Copy (IBC) and Palette. It provides flexible coding features to fit a variety of picture contents. One major concern on ISC is DDR bandwidth due to small strings. By only allowing strings with length equal to or greater than 20 pixels outside of 2-CTU reference window, the theoretical worse case memory bandwidth of ISC can be significantly reduced and is between 40%~147% of IBC depending on memory access patterns. This document evaluates the impact of minimum string length of 20 pixels on coding performance of ISC technique proposed in JCTVC-S0250. The experimental results show that the minimum string length constraint of 20 pixels has very little impact on ISC coding performance.

# Experimental Results

The experiments were conducted using HM-16.2+SCM-3.0 software [1] and the screen content common test conditions [2] to collect the BD-rate/bitrate-saving and the encoding and decoding times of the proposed technique under the All-Intra, Random Access, and Low Delay B test conditions. Two different settings to the search range are tested as listed in Table 1.

Table 1. Search range of each tests.

|  | IBC | ISC |
| --- | --- | --- |
| Test condition 1 | Full-frame | 1x2-CTU |
| Test condition 2 | 1x4-CTU | 1x4-CTU |

The following tables show the coding performance.

Table 2. BD-rate reduction of TC1 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.4% | -1.6% | -1.6% | -0.9% | -1.0% | -1.0% | -0.4% | -0.5% | -0.5% |
| RGB, mixed content, 1440p & 1080p | -0.6% | -0.7% | -0.8% | -0.4% | -0.5% | -0.6% | -0.3% | -0.4% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.4% | -1.4% | -1.5% | -0.8% | -0.9% | -1.1% | -0.3% | -0.5% | -0.6% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -1.0% | -1.0% | -0.4% | -0.9% | -0.9% | -0.5% | -1.3% | -1.1% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | 0.1% | 0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | -0.2% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 187% |  |  | 121% |  |  | 115% |  |  |
| Dec Time[%] | 105% |  |  | 101% |  |  | 103% |  |  |

Table 3. Bitrate saving of TC1 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving Average | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, TGM, 1080p & 720p | -3.3% | -3.8% | -11.7% | -0.3% | -1.5% | -2.1% | -7.1% | -0.2% | -1.3% | -1.4% | -3.6% | -0.2% |
| RGB, MC, 1440p & 1080p | -0.5% | -0.6% | -1.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -3.3% | -3.8% | -11.8% | -0.3% | -1.4% | -2.1% | -7.1% | -0.2% | -1.2% | -1.3% | -3.6% | -0.2% |
| YUV, MC, 1440p & 1080p | -0.6% | -0.6% | -1.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 230% |  |  |  | 124% |  |  |  | 118% |  |  |  |
| Dec Time[%] | 105% |  |  |  | 99% |  |  |  | 101% |  |  |  |

Table 4. BD-rate reduction of TC2 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.7% | -3.0% | -2.9% | -1.8% | -2.1% | -2.1% | -0.8% | -0.9% | -0.9% |
| RGB, mixed content, 1440p & 1080p | -0.9% | -1.2% | -1.2% | -0.6% | -0.9% | -0.9% | -0.3% | -0.5% | -0.6% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.5% | -2.7% | -2.9% | -1.6% | -1.9% | -2.2% | -0.6% | -1.0% | -0.8% |
| YUV, mixed content, 1440p & 1080p | -1.0% | -1.6% | -1.5% | -0.6% | -1.5% | -1.3% | -0.4% | -1.4% | -1.4% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.0% | 0.1% | 0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% |
| Enc Time[%] | 199% |  |  | 121% |  |  | 114% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  | 100% |  |  |

Table 5. Bitrate saving of TC2 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving (Average) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving (Total) | Bit-rate saving Average | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, TGM, 1080p & 720p | -4.6% | -5.1% | -14.5% | -0.3% | -2.7% | -3.1% | -9.3% | -0.2% | -2.6% | -2.2% | -5.2% | -0.2% |
| RGB, MC, 1440p & 1080p | -0.9% | -1.0% | -1.5% | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -4.7% | -5.3% | -15.0% | -0.3% | -2.5% | -3.2% | -9.6% | -0.2% | -2.4% | -2.0% | -5.1% | -0.2% |
| YUV, MC, 1440p & 1080p | -0.9% | -1.0% | -1.6% | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 222% |  |  |  | 124% |  |  |  | 113% |  |  |  |
| Dec Time[%] | 77% |  |  |  | 87% |  |  |  | 90% |  |  |  |

The theoretical worst-case DDR memory bandwidth is significantly reduced by 45%~75% as shown in the following tables.

Memory pattern 8x2 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |  |
| --- | --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 6 | 18 |  |
| Intra LC | 1 | 1 | 4 | 12 | 36 |  |
|  | 1 | 4 | 1 | 8 | 24 |  |
| Inter 8x8 | 8/6 | 8 | 8 | 12/10.5 | 33 |  |
| Intra String Copy | 1 | 1 | 1 | 16 | 48 |  |
| Intra String Copy (Min String Length = 20) | 1 | 20 | 1 | (3*8*2)/20=2.4 | 7.2 | Horizontal scan case |
| Intra String Copy (Min String Length = 20) | 1 | 1 | 20 | (11*8*2)/20=8.8 | 26.4 | Vertical scan case |

Memory pattern 8x4 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |  |
| --- | --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 8 | 24 |  |
| Intra LC | 1 | 1 | 4 | 16 | 48 |  |
|  | 1 | 4 | 1 | 16 | 48 |  |
| Inter 8x8 | 8/6 | 8 | 8 | 16/12 | 39 |  |
| Intra String Copy | 1 | 1 | 1 | 32 | 96 |  |
| Intra String Copy (Min String Length = 20) | 1 | 20 | 1 | (3*8*4)/20=4.8 | 14.4 | Horizontal scan case |
| Intra String Copy (Min String Length = 20) | 1 | 1 | 20 | (7*8*4)/20=11.2 | 33.6 | Vertical scan case |

Memory pattern 4x2 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |  |
| --- | --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 3 | 9 |  |
| Intra LC | 1 | 1 | 4 | 6 | 18 |  |
|  | 1 | 4 | 1 | 4 | 12 |  |
| Inter 8x8 | 8/6 | 8 | 8 | 10/7 | 24 |  |
| Intra String Copy | 1 | 1 | 1 | 8 | 24 |  |
| Intra String Copy (Min String Length = 20) | 1 | 20 | 1 | (7*4*2)/20=2.8 | 8.4 | Horizontal scan case |
| Intra String Copy (Min String Length = 20) | 1 | 1 | 20 | (11*4*2)/20=4.4 | 13.2 | Vertical scan case |

Memory pattern 4x4 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |  |
| --- | --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 4 | 12 |  |
| Intra LC | 1 | 1 | 4 | 8 | 24 |  |
|  | 1 | 4 | 1 | 8 | 12 |  |
| Inter 8x8 | 8/6 | 8 | 8 | 12.5/8 | 28.5 |  |
| Intra String Copy | 1 | 1 | 1 | 16 | 48 |  |
| Intra String Copy (Min String Length = 20) | 1 | 20 | 1 | (7*4*4)/20=5.6 | 16.8 | Horizontal scan case |
| Intra String Copy (Min String Length = 20) | 1 | 1 | 20 | (7*4*4)/20=5.6 | 16.8 | Vertical scan case |

Memory pattern 8x8 per pixel

|  | L | M | N | Y/ (U or V) | 3 component |  |
| --- | --- | --- | --- | --- | --- | --- |
| Intra BC (4x4) | 1 | 4 | 4 | 16 | 48 |  |
| Intra LC | 1 | 1 | 4 | 16 | 96 |  |
|  | 1 | 4 | 1 | 32 | 96 |  |
| Inter 8x8 | 8/6 | 8 | 8 | 18/18 | 54 |  |
| Intra String Copy | 1 | 1 | 1 | 64 | 192 |  |
| Intra String Copy (Min String Length = 20) | 1 | 20 | 1 | (3*8*8)/20=9.6 | 28.8 | Horizontal scan case |
| Intra String Copy (Min String Length = 20) | 1 | 1 | 20 | (3*8*8)/20=9.6 | 28.8 | Vertical scan case |

# Conclusion

In this contribution, the impact of minimum string length constraint of 20 pixels on coding performance of ISC is investigated. The BD-rate and bit-rate increase is less than 0.1% and 0.6%, respectively for lossy and lossless coding (comparison with the results of Test B.2 which does not have the minimum string length constraint).

Based on the quite good coding performance improvement and low DDR worst-case bandwidth, we would recommend the committee to adopt this ISC coding mode in SCC draft standard.

# References

- JCT-VC, “HM-16.2+SCM-3.0,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-3.0

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-S1015, Strasbourg, Oct. 2014.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).