| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10 Feb. – 18 Feb. 2015 | Document: JCTVC-T0139 |
| --- | --- |

| Title: | Non-CE3: Improvement on Intra String Copy |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liping Zhao, Kailun Zhou, Shuhui Wang, Tao Lin 1239 Siping Ave, Shanghai 200092, China | Tel: Email: | lintao@tongji.edu.cn |
| Source: | Tongji University |  |  |

_____________________________

# Abstract

This contribution reports a few improvements on Intra String Copy (ISC) technique. The coding performance is evaluated under three configurations of search/reference range: (TC1) FF IBC vs 2CTU ISC, (TC2) 4CTU IBC vs 4CTU ISC, and (TC3) FF IBC vs FF ISC. Using SCM30 as anchor, their coding results for Y component of YUV TGM AI lossy coding are reported as -3.3%, -5.4%, -6.8%, respectively for TC1, TC2, TC3. The minimum string length is 20 pixels outside of 2CTU (current and left) reference range to reduce theoretical worst case DDR bandwidth.

# Introduction

The Intra String Copy (ISC) technique is a natural extension of both Intra Block Copy (IBC) and Palette. It provides flexible coding features to fit a variety of picture contents. This document presents a few improvements on ISC including lossy matching, lossy hash, travScan, coding parameters (offset and len) coding optimization. Experiments show that the improved ISC can provide quite good coding performance improvement using SCM30 as anchor. Also, the minimum string length is 20 pixels outside of 2CTU (current and left) reference range to reduce theoretical worst case DDR bandwidth.

# Proposed methods

## ISC string offset distribution (2CTU lossy coding, all qp and depths)

H scan case

offsetY

offsetX

64

-64

-64

128

ABOVE: 24%

ABOVE: Ext. 10%

LEFT 2%

LEFT Ext 22%

X<0,Y>0: 8%

X>0,Y>0: 17%

X>0,Y<0: 17%

V scan case

offsetY

offsetX

64

-64

-64

128

LEFT: 1%

LEFT: Ext. 3%

ABOVE 24%

ABOVE Ext 43%

X<0,Y>0: 3%

X>0,Y>0: 13%

X>0,Y<0: 13%

From the above figure and data, we can see that ABOVE plus LEFT strings are about 25%, and ABOVE Ext strings are about 26%. There are 49% strings other than LEFT, ABOVE, and ABOVE Ext. By allowing pixel copy from any location, ISC can provide significant room for coding performance improvement.

## Improvements

Based on CE3 ISC software, we made the following improvements:

- lossy string matching in the encoder

- lossy hash by using 4 MSB of each sample to generate the hash value

- travScan in both horizontal and vertical scan directions

- redundancy removal in string offset coding

- TB coding of string length

- better binarization of offset and length

- minimum string length of 20 pixels outside of 2CTU (current and left) reference range to significantly reduce theoretical worst case DDR bandwidth as shown in the following tables.

|  | Configurations |  |  |  |  |  |  | Memory Bandwidth (P) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Block Size |  | IF Length |  | Bi Pred | Memory Pattern |  | Y Component |  |  |  |  |  |
|  |  |  | Y | U/V |  |  | m= | 2 | 2 | 4 | 4 | 8 | 8 |
| Description | M | N | L | L | Y or N |  | n= | 2 | 4 | 2 | 8 | 4 | 8 |
| 8x8 Bi-prediction | 8 | 8 | 8 | 6 | Y |  |  | 8 | 10 | 10 | 15 | 15 | 18 |
| 4x8 Uni-prediction | 4 | 8 | 8 | 6 | N |  |  | 6 | 7.5 | 8 | 12 | 15 | 18 |
| 8x4 Uni-prediction | 8 | 4 | 8 | 6 | N |  |  | 6 | 8 | 7.5 | 15 | 12 | 18 |
| 4x4 IBC PU | 4 | 4 | 1 | 1 | N |  |  | 2.3 | 3 | 3 | 8 | 8 | 16 |
| 1x4 ILC | 1 | 4 | 1 | 1 | N |  |  | 3 | 4 | 6 | 16 | 16 | 32 |
| 4x1 ILC | 4 | 1 | 1 | 1 | N |  |  | 3 | 6 | 4 | 16 | 16 | 32 |
| 1x1 ISC | 1 | 1 | 1 | 1 | N |  |  | 4 | 8 | 8 |  |  |  |
| S0139 H scan | 20 | 1 | 1 | 1 | N |  |  |  |  |  | 9.6 | 6.4 | 12.8 |
| S0139 V scan | 1 | 20 | 1 | 1 | N |  |  |  |  |  | 6.4 | 9.6 | 12.8 |

Note 1: In typical SRAM implementation, the memory pattern size is 2x2 pixels with a memory bus width of 2x2x24=96 bits. In this case, ISC has a P-value of 4, which is very reasonable and acceptable.

Note 2: In typical DDR implementation, the memory pattern size is 8x8 pixel samples (bytes) with a memory bus width of 64 bits and burst size of 8. In this case, ISC has a P-value of 12.8, which is very reasonable and acceptable.

Because ISC technique searches longest strings in shortest distance, it always finds big strings in long distance by nature. So the minimum string length constraint has almost no impact on ISC coding performance.

CABAC throughput is shown in the following tables. No residual is coded in S0139.

| Prediction Mode | JCTVC-T0139 |  |
| --- | --- | --- |
| (Note: CU Size = 8x8) | Syntax Elements | Number |
| Mode Flag | cu_transquant_bypass_flag | 1 |
|  | cu_skip_flag | 1 |
|  | intra_bc_flag | 1 |
|  | intra_sc_flag | 1 |
|  | isc_srb_mode_flag | 1 |
|  | isc_srb_sharing_flag | 1 |
| Partition |  |  |
| MV or BV | isc_no_ref_pixel_flag | 64 |
|  | isc_offsetX_zero_flag | 64 |
|  | isc_offsetX_sign_flag | 64 |
|  | isc_offsetY_zero_flag | 64 |
|  | isc_offsetY_sign_flag | 64 |
|  | isc_length_prefix | 104 |
| Residual (4 4x4 TBs, 3 Color Components) | N/A |  |
| Number of Context-coded Bins |  | 430 |
| Bins per Pixel |  | 6.71875 |

# Experimental Results

The experiments were conducted using HM-16.2+SCM-3.0 software [1] and the screen content common test conditions [2] to collect the BD-rate/bitrate-saving and the encoding and decoding times of the proposed technique under the All-Intra, Random Access, and Low Delay B test conditions. Three different settings to the search range are tested as listed in Table 1.

Table 1. Search range of each tests.

|  | IBC | ISC |
| --- | --- | --- |
| Test condition 1 | Full-frame | 1x2-CTU |
| Test condition 2 | 1x4-CTU | 1x4-CTU |
| Test condition 3 | Full-frame | Full-frame |

The following tables show the coding performance.

Table 2. BD-rate reduction of TC1 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.3% | -3.9% | -3.8% | -2.3% | -2.7% | -2.7% | -1.6% | -1.9% | -1.8% |
| RGB, mixed content, 1440p & 1080p | -1.3% | -1.8% | -1.8% | -0.8% | -1.4% | -1.4% | -0.4% | -1.2% | -1.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -3.3% | -3.5% | -3.5% | -2.2% | -2.6% | -2.8% | -1.5% | -1.7% | -2.1% |
| YUV, mixed content, 1440p & 1080p | -1.3% | -2.7% | -2.6% | -0.9% | -2.2% | -2.2% | -0.3% | -2.5% | -2.3% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | -0.2% | 0.1% | 0.1% | 0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% |
| Enc Time[%] | 428% |  |  | 156% |  |  | 143% |  |  |
| Dec Time[%] | 113% |  |  | 104% |  |  | 105% |  |  |

Table 3. Bitrate saving of TC1 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving Total | Bit-rate saving (Avg) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving Total | Bit-rate saving (Avg) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving Total | Bit-rate saving (Avg) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, TGM, 1080p & 720p | -4.6% | -5.5% | -15.8% | -0.5% | -2.4% | -3.3% | -10.4% | -0.3% | -2.3% | -2.5% | -6.3% | -0.3% |
| RGB, MC, 1440p & 1080p | -0.8% | -0.9% | -1.6% | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, CC, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -4.9% | -5.7% | -16.0% | -0.5% | -2.4% | -3.5% | -10.5% | -0.3% | -2.2% | -2.6% | -6.5% | -0.2% |
| YUV, MC, 1440p & 1080p | -0.9% | -0.9% | -1.7% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, CC, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 387% |  |  |  | 148% |  |  |  | 134% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  | 102% |  |  |  |

Table 4. BD-rate reduction of TC2 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p |  |  |  |  |  |  |  |  |  |
| RGB, mixed content, 1440p & 1080p |  |  |  |  |  |  |  |  |  |
| RGB, Animation, 720p |  |  |  |  |  |  |  |  |  |
| RGB, camera captured, 1080p |  |  |  |  |  |  |  |  |  |
| YUV, text & graphics with motion, 1080p & 720p | -5.4% | -6.1% | -6.3% | -3.9% | -4.6% | -4.9% | -2.2% | -2.8% | -3.0% |
| YUV, mixed content, 1440p & 1080p | -2.3% | -3.9% | -3.9% | -1.5% | -3.3% | -3.3% | -0.5% | -2.4% | -2.3% |
| YUV, Animation, 720p |  |  |  |  |  |  |  |  |  |
| YUV, camera captured, 1080p |  |  |  |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |

Table 5. Bitrate saving of TC2 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving Total | Bit-rate saving (Avg) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving Total | Bit-rate saving (Avg) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving Total | Bit-rate saving (Avg) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, TGM, 1080p & 720p |  |  |  |  |  |  |  |  |  |  |  |  |
| RGB, MC, 1440p & 1080p |  |  |  |  |  |  |  |  |  |  |  |  |
| RGB, Animation, 720p |  |  |  |  |  |  |  |  |  |  |  |  |
| RGB, CC, 1080p |  |  |  |  |  |  |  |  |  |  |  |  |
| YUV, TGM, 1080p & 720p | -7.1% | -7.9% | -20.2% | -0.6% | -4.1% | -5.2% | -14.1% | -0.4% | -3.9% | -3.7% | -8.9% | -0.3% |
| YUV, MC, 1440p & 1080p | -1.5% | -1.6% | -2.6% | -0.4% | -0.3% | -0.3% | -0.3% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
| YUV, Animation, 720p |  |  |  |  |  |  |  |  |  |  |  |  |
| YUV, CC, 1080p |  |  |  |  |  |  |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6. BD-rate reduction of TC3 in lossy coding.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p |  |  |  | -4.9% | -5.8% | -5.6% | -3.3% | -3.9% | -3.9% |
| RGB, mixed content, 1440p & 1080p |  |  |  | -1.4% | -2.3% | -2.3% | -0.7% | -1.7% | -1.7% |
| RGB, Animation, 720p |  |  |  | 0.1% | 0.1% | 0.0% | 0.1% | 0.1% | -0.1% |
| RGB, camera captured, 1080p |  |  |  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -6.8% | -7.3% | -7.6% | -4.6% | -5.4% | -5.8% | -2.8% | -3.6% | -3.9% |
| YUV, mixed content, 1440p & 1080p | -2.8% | -4.5% | -4.5% | -1.8% | -3.5% | -3.6% | -0.8% | -3.3% | -3.2% |
| YUV, Animation, 720p |  |  |  | 0.0% | -0.2% | 0.2% | 0.0% | 0.1% | 0.0% |
| YUV, camera captured, 1080p |  |  |  | 0.0% | 0.0% | 0.1% | 0.0% | 0.2% | 0.1% |
| Enc Time[%] |  |  |  | 168% |  |  | 152% |  |  |
| Dec Time[%] |  |  |  | 103% |  |  | 105% |  |  |

Table 7. Bitrate saving of TC3 in lossless coding.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate saving Total | Bit-rate saving (Avg) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving Total | Bit-rate saving (Avg) | Bit-rate saving (Min) | Bit-rate saving (Max) | Bit-rate saving Total | Bit-rate saving (Avg) | Bit-rate saving (Min) | Bit-rate saving (Max) |
| RGB, TGM, 1080p & 720p | -6.7% | -8.1% | -22.1% | -0.6% | -4.4% | -5.3% | -14.6% | -0.4% | -4.3% | -4.1% | -9.0% | -0.3% |
| RGB, MC, 1440p & 1080p | -1.7% | -1.8% | -2.4% | -1.0% | -0.3% | -0.3% | -0.3% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, CC, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -7.2% | -8.5% | -22.9% | -0.6% | -4.4% | -5.6% | -15.1% | -0.4% | -4.1% | -4.3% | -9.3% | -0.3% |
| YUV, MC, 1440p & 1080p | -1.8% | -1.9% | -2.6% | -0.9% | -0.3% | -0.3% | -0.3% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, CC, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 429% |  |  |  | 154% |  |  |  | 139% |  |  |  |
| Dec Time[%] | 99% |  |  |  | 102% |  |  |  | 103% |  |  |  |

# Conclusion

In this contribution, the coding performance of ISC improvement under three different configurations of the search range is tested. Up to -7.6% average BD-rate reduction in lossy coding and -8.5% average bit-rate reduction in lossless coding are observed in the All-Intra configuration in TC3 (FF IBC and FF ISC). Based on the quite good coding performance improvement, we would recommend the committee to adopt this coding mode in SCC draft standard.

# References

- JCT-VC, “HM-16.2+SCM-3.0,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.2+SCM-3.0

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content coding tests,” JCTVC-S1015, Strasbourg, Oct. 2014.

# Patent rights declaration(s)

Tongji University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).