| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0140-r3 |
| --- | --- |

| Title: | Enhanced QP offset signalling for adaptive cross-component transform in SCC extensions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN Krishna Rapaka Rajan Joshi Vadim Seregin Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA | Tel: Email: | +81 44 431 7659 chono@ct.jp.nec.com 1-858-658-2042 krapaka@qti.qualcomm.com |
| Source: | NEC Corporation, Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents an enhanced QP offset signalling method for correctly switching QP offset values between RGB color space and YCoCr color space residuals when adaptive cross-component transform is used. This contribution also provides the software patch to HM-16.2+SCM-3.0. The patch increases the number of bits of PPS SCC extension by 4 bits and does not change the other encoding results under the current common test conditions, which is reportedly shown by simulation results with the current common test conditions. This contribution also reportedly presents additional simulation results using non-default qp offset values and shows gains by the proposal. It is recommended that the proposal is considered for SCC test model adoption or studied in AHG toward the next meeting.

# Introduction and Problem Statement

In the 19th JCTVC meeting, JCTVC-S0040, JCTVC-S0094, JCTVC-S0144, and JCTVC-S0300 were proposed to signal additional qp offset values and switch them based on color space indicated by CU-level flag, namely cu_residual_act_flag, when adaptive cross-component transform is enabled.

The proposals claim that the current HEVC Screen Content Coding (SCC) design is not able to exploit the qp offset functionality that signals and controls offsets to be used for the quantization parameter (QP) of each of the Chroma components relative to the QP used for Luma. The reason is that the current HEVC SCC design signals only single set of chroma QP offset values and applies it to different color space prediction residuals eve if adaptive cross-component transform is enabled.

It is asserted that since the qp offset functionality is used in AVC and HEVC version 1 based products, some appropriate fix should be adopted in the final HEVC SCC design in order to allow HEVC SCC users to use them with non-zero values. This contribution proposes a solution on top of JCTVC-S0300 (which merges JCTVC-S0040, JCTVC-S0094, and JCTVC-S0144) in the following section.

# Proposal

This proposal consists of the following schemes:

- PPS/slice-level signaling of additional QP offset values for CUs associated with the second color space when adaptive_color_trans_flag at PPS is equal to 1.

- CU-level switching of QP offset values for reconstructing residuals according to the value of cu_residual_act_flag.

Notice that since the QP switching is applied only to reconstruction process, other processes such as QP prediction, QP derivation for deblocking, etc., are not changed. Therefore when Qp offset values are set to zero (i.e., current common test condition), the proposal increases the number of bits of PPS SCC extension by 4 bits and does not change the other encoding results.

The proposed modification to the current HEVC SCC draft is as follows:

[In the extension to the PPS:]

| pps_scc_extension ( ) { | Descriptor |
| --- | --- |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| if(residual_adaptive_colour_transform_enabled_flag){ |  |
| pps_slice_act_qp_offsets_present_flag | u(1) |
| pps_act_y_qp_offset_plus5 | se(v) |
| pps_act_cb_qp_offset_plus5 | se(v) |
| pps_ act_cr_qp_offset_plus3 | se(v) |
| } |  |
| } |  |

[In the slice segment header]

| slice_segment_header( ) { | Descriptor |
| --- | --- |
| first_slice_segment_in_pic_flag | u(1) |
| …………….. |  |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( chroma_qp_offset_list_enabled_flag ) |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if(pps_slice_act_qp_offsets_present_flag){ |  |
| slice_act_y_qp_offset | se(v) |
| slice_act_cb_qp_offset | se(v) |
| slice_act_cr_qp_offset | se(v) |
| } |  |
| …… |  |

residual_adaptive_colour_transform_enabled_flag equal to 1 specifies that an adaptive colour transform may be applied to the residual in the decoding process for the pictures referring to the PPS. residual_adaptive_colour_transform_enabled_flag equal to 0 specifies that adaptive colour transform is not applied to the residual for the pictures referring to the PPS. When not present, the value of residual_adaptive_colour_transform_enabled_flag is inferred to be equal to 0.

When chroma_format_idc is not equal to 3, residual_adaptive_colour_transform_enabled_flag shall be equal to 0.

pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_plus5 and pps_act_cr_qp_offset_plus3 specify offsets to the luma, cb and cr quantization parameter qP derived in section 8.6.2, respectively. The values of pps_act_y_qp_offset, pps_act_cb_qp_offset and pps_act_cr_qp_offset are derived as pps_act_y_qp_offset_plus5 - 5, pps_act_cb_qp_offset_plus5 - 5 and pps_act_cr_qp_offset_plus3 -3 respectively and shall be in the range of −12 to +12, inclusive. When ChromaArrayType is equal to 0, pps_act_cb_qp_offset and pps_act_cr_qp_offset are not used in the decoding process and decoders shall ignore their value. When not present, the values of pps_act_y_qp_offset_plus5, pps_act_cb_qp_offset_plus5 and pps_act_cr_qp_offset_plus3 are inferred to be equal to 0.

pps_slice_act_qp_offsets_present_flag equal to 1 specifies that slice_act_y_qp_offset , slice_ act _cb_qp_offset, slice_ act _cr_qp_offset are present in the slice header. pps_slice_act_qp_offsets_present_flag equal to 0 specifies that slice_act_y_qp_offset , slice_ act _cb_qp_offset, slice_ act _cr_qp_offset are not present in the slice header. When not present, the value of pps_slice_act_qp_offsets_present_flag is inferred to be equal to 0.

slice_act_y_qp_offset, slice_act_cb_qp_offset and slice_act_cr_qp_offset specify offsets to the luma, cb and cr quantization parameter qP derived in section 8.6.2, respectively. The values of slice_act_y_qp_offset, slice_act_cb_qp_offset and slice_act_cr_qp_offset shall be in the range of −12 to +12, inclusive. When ChromaArrayType is equal to 0, slice_act_cb_qp_offset and slice_act_cr_qp_offset are not used in the decoding process and decoders shall ignore their value. When not present, the values of slice_act_y_qp_offset, slice_act_cb_qp_offset and slice_act_cr_qp_offset are inferred to be equal to 0. The value of pps_act_y_qp_offset + slice_act_y_qp_offset shall be in the range of -12 to +12, inclusive. The value of pps_act_cb_qp_offset + slice_act_cb_qp_offset shall be in the range of -12 to +12, inclusive. The value of pps_act_cr_qp_offset + slice_act_cr_qp_offset shall be in the range of -12 to +12, inclusive.

[In the derivation process for quantization parameters (Section 8.6.1):]

When ChromaArrayType is not equal to 0, the following applies.

–	If cu_residual_act_flag[ xTbY ][ yTbY ] is equal to 0, the variables qPCb and qPCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 57, QpY + pps_cb_qp_offset + slice_cb_qp_offset + CuQpOffsetCb )

qPiCr = Clip3( −QpBdOffsetC, 57, QpY + pps_cr_qp_offset + slice_cr_qp_offset + CuQpOffsetCr )

–	Otherwise (cu_residual_act_flag[ xTbY ][ yTbY ] is equal to 1), the variables qPCb and qPCr are derived as follows:

qPiCb = Clip3( −QpBdOffsetC, 57, QpY + pps_act_cb_qp_offset + slice_act_cb_qp_offset + CuQpOffsetCb )

qPiCr = Clip3( −QpBdOffsetC, 57, QpY + pps_act_cr_qp_offset + slice_act_cr_qp_offset  + CuQpOffsetCr )

[In the scaling and transformation process (Section 8.6.2)]

The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = Clip3(0, 51 + QpBdOffsetY, Qp′Y + (cu_residual_act_flag [ xTbY ][ yTbY ] ? pps_act_y_qp_offset + slice_act_y_qp_offset : 0)

–	Otherwise, if cIdx is equal to 1,

qP = max( 0, Qp′Cb + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −5 : 0 ) )

–	Otherwise (cIdx is equal to 2),

qP = max(0, Qp′Cr + (cu_residual_act_flag[ xTbY ][ yTbY ] ? −3 : 0 ) )

# Simulation results

3.1. Test1: Common test condition test

In this subsection, the following are tested.

Reference: Anchor (SCM-3.0) with --CbQpOffset=0 --CrQpOffset=0

Tested: T0140 with --CbQpOffset=0 --CrQpOffset=0 --ActQpYOffset=-5 --ActQpCbOffset=-5 --ActQpCrOffset=-3, where ActQpYOffset, ActQpCbOffset, and ActQpCrOffset are command options to signal the pps_act_y_qp_offset, pps_act_cb_qp_offset and pps_act_cr_qp_offset values.

Table 1 shows the results. (Note that the author found that the reference an tested results of RGB sc_programming_1280x720_60_8bit are different from that of “SCM-3 0_lossy_template_r1.xlsm.” The reason is being investigated.) Notice that the proposal does not change R-D results since the data increase associated with the additional PPS SCC extension 4 bits is hidden by byte alignment.

Table1: Common test condition results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

3.2. Test2: Non-common test condition test

In order to see the behaviour of the proposal when using non-default qp offset values, the following are tested for RGB contents.

Reference:	Anchor (SCM-3.0) with –CbQpOffset-=-6 --CrQpOffset=-6

Tested:	T0140 with --CbQpOffset=0 --CrQpOffset=0 --ActQpYOffset=-5 --ActQpCbOffset=-11--ActQpCrOffset=-9.

In this setting, it is intended that color fidelity of G component is improved by using the negative qp value of -6 in the region coded by using YCoCr color space, i.e., cu_residual_act_flag equal to 1. Notice that the current HEVC SCC design does not allow setting qp offset values separately for RGB and YCoCr color space regions, it can spend too much bits in B and R information of the regions coded by cu_residual_act_flag equal to 0. Table 2 shows the results.

Table2: Non-common test condition results

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -10.3% | 2.4% | 2.6% |
| RGB, mixed content, 1440p & 1080p | -9.8% | 4.8% | 5.4% |
| RGB, Animation, 720p | -12.9% | 10.7% | 9.0% |
| RGB, camera captured, 1080p | -37.9% | 9.7% | 2.9% |
| YUV, text & graphics with motion, 1080p & 720p | NA | NA | NA |
| YUV, mixed content, 1440p & 1080p | NA | NA | NA |
| YUV, Animation, 720p | NA | NA | NA |
| YUV, camera captured, 1080p | NA | NA | NA |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -12.8% | 4.1% | 4.2% |
| RGB, mixed content, 1440p & 1080p | -20.2% | 5.1% | 5.6% |
| RGB, Animation, 720p | -19.8% | 9.0% | 8.9% |
| RGB, camera captured, 1080p | -37.0% | 7.0% | -3.4% |
| YUV, text & graphics with motion, 1080p & 720p | NA | NA | NA |
| YUV, mixed content, 1440p & 1080p | NA | NA | NA |
| YUV, Animation, 720p | NA | NA | NA |
| YUV, camera captured, 1080p | NA | NA | NA |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -11.8% | 6.3% | 6.1% |
| RGB, mixed content, 1440p & 1080p | -21.1% | 8.2% | 8.8% |
| RGB, Animation, 720p | -24.3% | 16.3% | 17.4% |
| RGB, camera captured, 1080p | -43.0% | 13.7% | 1.8% |
| YUV, text & graphics with motion, 1080p & 720p | NA | NA | NA |
| YUV, mixed content, 1440p & 1080p | NA | NA | NA |
| YUV, Animation, 720p | NA | NA | NA |
| YUV, camera captured, 1080p | NA | NA | NA |
| Enc Time[%] | 95% |  |  |

The proposal shows gains in G component in RGB mixed contents and camera captured contents since it does not spend too much bits in B and R information of the regions coded by cu_residual_act_flag equal to 0.

- Conclusion

This contribution has proposed the enhanced QP offset signaling method for fixing the current HEVC SCC design bug. This contribution also provides the software patch to HM-16.2+SCM-3.0 in which changes are indicated by macros “SCM_T0140_ACT_QP_OFFSET.” The patch increases the number of bits of PPS SCC extension by 4 bits and does not change the other encoding results under the common test condition. It is recommended that the proposal is considered for SCC test model adoption or studied in AHG toward the next meeting.

# Reference

- K. Chono, "Enhanced chroma QP signalling for adaptive cross-component transform in SCC extensions", JCTVC-S0040.

- R. Sjoberg, J. Samuelsson, "QP offset for Adaptive Colour Transform", JCTVC-S0094.

- K. Rapaka, L. Zhang, R. Joshi, M. Karczewicz, "Qp derivation for adaptive color transform", JCTVC-S0144.

- K. Rapaka, L. Zhang, R. Joshi, M. Karczewicz, K. Chono, J. Xu, R. Sjoberg, K. Misra, S. H. Kim, A. Segall, "Qp derivation and offsets signaling for adaptive color transform", JCTVC-S0300.

- R. Joshi, J. Xu, “HEVC Screen Content Coding Draft Text 2”, JCTVC-S1005.

# Patent rights declaration

NEC Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).