| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0169 |
| --- | --- |

| Title: | CE1-related: Fix of CE1 Test D.1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | 1Yao-Jen Chang, Chao-Hsiung Hung, 2Chun-Lung Lin, 3Ching-Chieh Lin, Jih-Sheng Tu 195, Sec. 4, Chung Hsing Rd., Chutung, Hsinchu 31040, Taiwan | Tel: Email: | +886-3-591-8774 1britpablo@itri.org.tw 2Chunlung@itri.org.tw 3JackLin@itri.org.tw |
| Source: | ITRI International |  |  |

_____________________________

# Abstract

This proposal proposes a fixed encoder modification in CE1 test D1 based on SCM 3.0 reference software. The fixed encoder modification disables the palette-removal method which is originally integrated in the CE1 Test D1 and provides improved coding performance. It also includes two versions, with and without one additional RD checks compared to SCM 3.0. Compared with SCM 3.0 reference software under the conditions of AI-lossy coding, the first version and the second version can, respectively, achieve 0.0~0.1% and 0.1~0.3% BD-rate saving for the class “text & graphics with motion, 1080p & 720p”. Note that the two versions of the fast algorithm have no loss on encoding time. This contribution also combines the proposed methods with JCTVC-T0087. The results show that under the conditions of AI-lossy coding, the first version and the second version can, respectively, achieve 0.3~0.5% and 0.5~0.6% BD-rate saving for the class “text & graphics with motion, 1080p & 720p”.

# Fix of encoder modification

This proposal provides a fixed version of CE1 test D1 [1]. There are two modifications:

- S + TH < M is used to execute a fast algorithm. The threshold TH = 2 is replaced by TH = 3 in the fixed version.

- It also replaces the palette-removal method in the provided source code in JCTVC-T0044 by the original palette-removal method in SCM 3.0. It can be seen as a bugfix of our source code due to large BD-rate loss in RA and LB conditions. The detail can be found in the macro #define ITRI_BUGFIXED.

The fixed version also includes two versions, where the first one has no additional RD check, and the second one has one additional RD check.

- Simulation results

Two versions of the proposed algorithm are simulated based on SCM 3.0 reference software under the common test condition [2]. Table 1 and Table 2 show the lossy results for Version 1 and Version 2, where the anchor is the results of SCM 3.0. Version 1 and Version 2 can achieve up to 0.1% and 0.3% BD-rate saving for the class “text & graphics with motion, 1080p & 720p”. Note that there is no coding time loss for both versions. Table 3 and Table 4 show the lossless results. Combing the proposed methods with the initial palette generation method proposed in JCTVC-T0087, Table 5 and Table 6 show there is no conflict between two proposals in AI conditions, where 0.3~0.5% gain and 0.5~0.6% gain are achieved, respectively.

Table 1. Lossy results for Version 1.

Table 2. Lossy results for Version 2.

Table 3. Lossless results for Version 1.

Table 4. Lossless results for Version 2.

Table 5. AI lossy results for Version 1 combined with JCTC-T0087.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.3% | -0.3% | -0.3% |
| RGB, mixed content, 1440p & 1080p | -0.2% | -0.3% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.5% | -0.4% | -0.4% |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.5% | -0.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 87% |  |  |
| Dec Time[%] | 98% |  |  |

Table 6. AI lossy results for Version 2 combined with JCTC-T0087.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.5% | -0.4% | -0.4% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | -0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.6% | -0.6% | -0.7% |
| YUV, mixed content, 1440p & 1080p | -0.6% | -0.7% | -0.5% |
| YUV, Animation, 720p | 0.0% | -0.2% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 89% |  |  |
| Dec Time[%] | 98% |  |  |

# Conclusion

This proposal proposes a fixed encoder modification in CE1 test D1. It replaces palette size difference threshold to 3. It also disables a smoothing algorithm. The fixed version provides improved coding gain. Compared with SCM 3.0 reference software under the conditions of AI-lossy coding, the proposed algorithm with no additional RD check can be up to 0.1% BD-rate saving for the class “text & graphics with motion, 1080p & 720p”, and the proposed algorithm with one additional RD check can be up to 0.3% BD-rate saving for the class “text & graphics with motion, 1080p & 720p”. This contribution also combines the proposed methods with JCTVC-T0087. The results show that under the conditions of AI-lossy coding, the first version and the second version can, respectively, achieve 0.3~0.5% and 0.5~0.6% BD-rate saving for the class “text & graphics with motion, 1080p & 720p”.

# Reference

- Y. J. Chang, C. H. Hung, C. L. Lin, C. C. Lin, J. S. Tu, “CE6 Test B.2: Encoder modification of palette coding for escape pixels,” 19th JCT-VC meeting, JCTVC-S0048, Strasbourg, France, Oct. 2014.

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding,” 19th JCT-VC meeting, JCTVC-S1015, Strasbourg, France, Oct. 2014

# Patent rights declaration(s)

ITRI international may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)