| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0174 |
| --- | --- |

| Title: | CE1-related: Cross-check of JCTVC-T0119 on improved palette run-length coding with palette flipping, and additional tests |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | PoLin Lai, Jungsun Kim, Shan Liu 2860 Junction Ave, San Jose, CA, USA | Email: | { polin.lai, jungsun.kim,shan.liu, } @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents cross-check report for JCTVC-T0119 on improved palette run-length coding with palette flipping. Two flipping modes are presented in JCTVC-T01119: Vertical flipping (e.g. flip first row with the last, etc.) is utilized for horizontal scan, and horizontal flipping (e.g. flip first column with the last, etc.) is utilized for vertical scan. Software study was conducted and confirms that the proposed palette run-length coding modification and palette index flipping are implemented as described in the contribution. The cross-check’s simulation results match the data provided by the proponent.

In addition, this contribution also presents tests that replace the two palette index flipping modes in JCTVC-T0119 with a single index rotation. It is asserted that index rotation is an operation that already exists in HEVC RExt as residue rotation. Simulation results report similar or better coding efficiency when changing both palette index flipping modes in JCTVC-T0119 to a palette index rotation.

# Introduction

## Index map flipping, from JCTVC-T0119

In JCTVC-T0119 [1], improved palette run-coding as proposed in CE1 Test B.1 [2] was tested together with a proposed method of palette index flipping. Two flipping modes are presented in JCTVC-T01119:

- Vertical flipping (e.g. flip first row with the last, etc.) is utilized for horizontal scan.

- Horizontal flipping (e.g. flip first column with the last, etc.) is utilized for vertical scan.

Figure 1 below depicts the flipping of an 8x8 index map.

Figure 1. Palette index flipping on an 8x8 index map

The flipping operation was introduced in JCTVC-T0119 such that the efficient run-to-the-end signalling in CE1Test B.1 can also be used for the first run in the index map (after flipping, the first run become the last in scanning order).

## Index map rotation, additional tests by cross-checker

The cross-checker noticed that, the residue rotation operation in HEVC RExt, which rotates the scan by 180 degree, also achieves the same purpose of putting the first run to the last.

In addition, rotation operations of 180 degree for horizontal and vertical scans are the same. Figure 2 below depict rotation is a 4x4 block, with pixels marked with numbers to identify their positions.

Figure 2. 180 degree rotation (numbers are for identification of individual pixels)

This contribution also tests replacing the two flipping modes in JCTVC-T0119 [1] with a single index rotation. The signalling and other aspects are the same as in JCTVC T0119 [1].

# Simulation results

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- X. Xiu, Y. Ye, Y. He (InterDigital), W. Pu, R. Joshi, M. Karczewicz, F. Zou, V.Seregin (Qualcomm), “Non-CE1: improved palette run-length coding with palette flipping,” Document of Joint Collaborative Team on Video Coding, JCTVC-T0119, Feb. 2015

- W. Pu, R. Joshi, M. Karczewicz, F. Zou, V. Seregin (Qualcomm), ”CE1 Test B.1: Modified Palette Run Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-T0034, Feb. 2015