| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0182 |
| --- | --- |

| Title: | Non-CE2: Cross-check of JCTVC-T0098 on zero merging candidates derivation for Intra BC/Inter signaling unification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Akira Minezawa Kazuyuki Miyazawa Shun-ichi Sekiguchi 5-1-1, Ofuna, Kamakura, Kanagawa, JAPAN | Tel: Email: | +81 467 41 2463 Minezawa.Akira@ds.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results on zero merging candidates derivation for Intra BC/Inter signaling unification described in JCTVC-T0098. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that the proposed scheme achieves up to 5.8% coding improvement compared with CE2 anchor for lossy case. On the other hand, up to 3.3% coding loss is observed for lossless configurations. These objective results perfectly match the results from the proponent. Similar tendency on encoding and decoding time was also observed.

Since the proposed scheme in JCTVC-T0098 is implemented on top of CE2 Test 1, the coding performance of the proposed scheme compared with CE2 Test 1 is also evaluated. The simulation results indicate that the coding gain of the proposed scheme relative to CE2 Test 1 shows -0.7% to 1.3% for lossy case and -1.2% to 1.7% for lossless case.

# Introduction

Performance verification on modification of zero merging candidates derivation for Intra BC/Inter signaling unification proposed in JCTVC-T0098 [1] was conducted. In CE2 Test 1, Intra BC can use merge mode by unifying Intra BC and Inter signaling. On top of this scheme, JCTVC-T0098 modifies the zero merging candidates derivation in order to have high affinity with the specification of Intra BC. This work was conducted using modified SCM-3.0 software provided by the proponent.

# Simulation results

Verification tests are conducted on 64bit Linux operating system. The performance evaluation has been conducted under the SCC common test conditions [2]. All the simulation results are perfectly matched the results from the proponent. The results are also shown that the tendency of encoding and decoding time is approximately the same compared with the results provided by the proponent.

Table 1. Test result of RGB/YUV 4:4:4 contents for lossy configuration

Table 2. Test result of YUV 4:2:0 contents for lossy configuration

Table 3. Test result of RGB/YUV 4:4:4 contents for lossless configuration

Table 4. Test result of YUV 4:2:0 contents for lossless configuration

Since the proposed scheme to be crosscheck is integrated on top of CE2 Test 1, the coding performance relative to CE2 Test 1 is also tested in this contribution. The simulation results are shown as follows.

Table 5. Test result of RGB/YUV 4:4:4 contents for lossy configuration

Table 6. Test result of YUV 4:2:0 contents for lossy configuration

Table 7. Test result of RGB/YUV 4:4:4 contents for lossless configuration

Table 8. Test result of YUV 4:2:0 contents for lossless configuration

# Conclusions

This contribution reports cross-verification results of JCTVC-T0098 that evaluates zero merging candidates derivation for Intra BC/Inter signaling unification. The verification work was performed using the software provided by the proponent. Reported performance results were obtained by building and running the provided software on 64bit Linux platform. It has been confirmed that the objective R-D performance produced in this work perfectly matches the results from the proponent. It has been also confirmed that the coding gain of the proposed scheme relative to CE2 Test 1 shows -0.7% to 1.3% for lossy case and -1.2% to 1.7% for lossless case.

# References

[1] C. Pang, V. Seregin and M. Karczewicz, “Non-CE2: Zero merging candidates derivation for Intra BC/Inter signaling unification”, JCTVC- T0098, Geneva, CH, Feb. 2015.

[2] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common conditions for screen content tests”, JCTVC-S1015, Strasbourg, FR, Oct. 2014.