| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0189 |
| --- | --- |

| Title: | Cross check of T0109 -- Non-CE1: Extension of palette mode to non-4:4:4 colour formats |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jie Zhao Seung-Hwan Kim 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 kimse@sharplabs.com |
| Source: | SHARP Laboratories of America, Inc. |  |  |

_____________________________

# Abstract

This document reports the crosscheck results for JCTVC-T0109 – “Non-CE1: Extension of palette mode to non-4:4:4 colour formats” by Qualcomm. In JCTVC_T0109, it proposes to extend palette mode to non-4:4:4 colour format. The source code provided by the proponents was verified to be consistent with the description in JCTVC-T0109. The RD performance was evaluated for CE1 common test conditions [1] and matches those provided in JCTVC-T0109.

# Introduction

In JCTVC-T0109, for 4:2:0 colour format, palette coding is enabled and a single palette is used where each palette entry consists of three components. The syntax is identical to the 4:4:4 case except for escape samples. The chroma escape pixels are only signaled at the sample position where both row and column indices are even.

# Simulation Results

Simulation is performed according to CE1 test conditions Error: Reference source not found for 420 sequences and based on the software provide by proponents. The detailed results can be found in the attached excel datasheets. Results match those provided by the component in JCTVC-T0109_v2.zip.

Table 1.1: Results with respect to the SCM3.0 4:2:0 anchor (Lossy) ,

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | -3.7% | -2.9% | -3.2% | -3.5% | -3.1% | -3.3% | -2.1% | -3.0% | -4.6% |
| Mixed content, 480p | -0.9% | -2.9% | -3.0% | -0.3% | -1.2% | -1.3% | 0.2% | 0.0% | 0.0% |
| Animation, 768p | -4.6% | -12.0% | -7.9% | -2.5% | -14.1% | -7.5% | -1.4% | -10.7% | -4.9% |
| Average of all sequences | -3.2% | -5.2% | -4.3% | -2.4% | -5.3% | -3.9% | -1.4% | -4.2% | -3.5% |

Table 1.2: Results with respect to the SCM3.0 4;2:0 anchor (Lossless)

|  | All Intra |  |  |  |  |  |  |  | Random Access |  |  |  | Low delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit-rate Savings |  |  |  |  |  |  |  | Bit-rate Savings |  |  |  | Bit-rate Savings |  |  |  |
|  | Total |  | Ave. |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | Min |  |  | Max |  | Total | Ave. | Min | Max | Total | Ave. | Min | Max |
| Text & graphics with motion, 720p | -0.1% | -0.1% |  |  | -0.3% | 0.0% |  |  | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% | 0.1% |
| Mixed content, 480p | -0.1% | -0.1% |  |  | -0.1% | -0.1% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Animation, 768p | -0.6% | -0.6% |  |  | -0.6% | -0.6% |  |  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Average of all sequences | -0.3% | -0.2% |  |  | -0.6% | 0.0% |  |  | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | -0.1% | 0.1% |

# Conclusion

JCTVC-T0109 is cross-checked according to CE1 test conditions Error: Reference source not found and based on the software provide by proponents. It is reported that these results perfectly match those provided by the proponents.

# References

- P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, Z. Ma, “Description of Core Experiment 1 (CE1): Improvements of palette mode,” 19th JCT-VC meeting, JCTVC-S1101, Strasbourg, France, Oct. 2014.