| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0192 |
| --- | --- |

| Title: | Non-CE1: 2-D Index Map Coding in HEVC SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Wang, Meng Xu, Zhan Ma, H. Yu 2330 Central Expy Santa Clara, CA 95050 USA | Tel: Email: | +1 408-330-5142 wei.wang.sc@huawei.com |
| Source: | Huawei R&D USA |  |  |

_____________________________

# Abstract

This contribution presents a hybrid 1-D and 2-D string copy method for index map coding. This method is designed for improving the performance of the palette mode in HEVC SCC WD and SCM 3.0. The performance reported in this contribution was evaluated under common test conditions with various search range configurations.

# Proposed Method

The index map coding of palette mode in the current HEVC SCC draft uses RUN based 1D string copy method. For any given location, in addition to the RUN based 1D string match, a 2D string copy with flexible width and height is also performed in this proposal. Basically if an area within the current CU covered by the 2D-match has more indices than the total number of indices represented by a single 1D RUN length, the 2D string copy mode is selected and 4 parameters, mvx, mvy, width, height, are encoded into bit-stream. Otherwise, the current index map coding mode is selected and its associated syntax elements are encoded into bit-stream. In this proposal, residual is generated and then coded using the HEVC residual coding method.

The differences between this contribution and CE1 Test-C.1 are:

| CE1 Test-C.1 | Non-CE1 |
| --- | --- |
| Both 1D and 2D string search operation are done in the Palette index domain | Both 1D and 2D string search operation are done in the original pixel domain. |
| 2D string search is limited to CTU level | 2D string search is extended to CU level |
| In 1D/2D copy mode, the CU prediction values are generated using RGB value retrieved from palette table. | In this proposal, the CU prediction values are generated using reconstruction pixel from reference match location. |
| Residual is not generated so no residual coding involved | Residual is coded using HEVC residual coding method |

Technical Details:

A one-bit context coded “2D_flag” is added to the syntax to indicate if the 2D-string copy mode is selected.

- If 2D_flag = 1:

  - a one-bit context coded mvx0_flag is added to indicate whether its match vector mvx is equal to zero; and a one-bit context coded mvy0_flag is added to indicate whether its match vector mvy is equal to 0.

    - When a 2D block is a single color block, both mvx and mvy are set to 0 and its color index is encoded.

    - Prediction value for index mode is generated using RGB value retrieved from palette table; Prediction value for copy mode is generated using reconstruction value from reference match location.

  - a one-bit context is used to encode the sign of non-zero mvx, followed by the absolute value of mvx, which is encoded using truncated binary coding (TBC) method. The max value of TBC input parameter can be obtained from the width of search range.

  - a one-bit context is used to encode the sign of non-zero mvy, followed by the absolute value of mvy, which is encoded using truncated binary coding (TBC) method. The max value of TBC input parameter can be obtained from the height of search range.

  - both width and height are encoded using TBC method. The max value of TBC input parameter can be obtained from the current location, reference location and width/height of search range.

- If 2D_flag = 0, a modified version of the current index map coding is applied where the COPY_ABOVE flag is encoded using only one context (2 contexts are used in the SCM-3.0 software).

In this proposal, residuals are generated by subtracting the original pixels from the prediction pixels represented by the match map described above. Residual coding is also performed using HEVC residual coding method.

# Simulations

Experiments are performed on top of the SCM 3.0 with the common test condition [2]. In addition, we also performed tests using various restricted search areas (e.g. 1x4 CTUs) for intra block copy (IBC). Results for lossy and lossless are summarized as follows.

Below figure are for 1 second AI. We will update with full sequence results as soon as they become available.

Table 1 : Lossy coding efficiency (Anchor SCM3.0 IBC 1x4)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -2.5% | -2.9% | -2.7% |
| RGB, mixed content, 1440p & 1080p | -0.3% | -0.4% | -0.5% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -2.4% | -2.7% | -2.7% |
| YUV, mixed content, 1440p & 1080p | -0.4% | -0.5% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.0% |
| Enc Time[%] | 123% |  |  |
| Dec Time[%] | 106% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | #VALUE! | #VALUE! | #VALUE! |
| RGB, mixed content, 1440p & 1080p | #VALUE! | #VALUE! | #VALUE! |
| RGB, Animation, 720p | #VALUE! | #VALUE! | #VALUE! |
| RGB, camera captured, 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, text & graphics with motion, 1080p & 720p | #VALUE! | #VALUE! | #VALUE! |
| YUV, mixed content, 1440p & 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, Animation, 720p | #VALUE! | #VALUE! | #VALUE! |
| YUV, camera captured, 1080p | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #VALUE! |  |  |
| Dec Time[%] | #VALUE! |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | #VALUE! | #VALUE! | #VALUE! |
| RGB, mixed content, 1440p & 1080p | #VALUE! | #VALUE! | #VALUE! |
| RGB, Animation, 720p | #VALUE! | #VALUE! | #VALUE! |
| RGB, camera captured, 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, text & graphics with motion, 1080p & 720p | #VALUE! | #VALUE! | #VALUE! |
| YUV, mixed content, 1440p & 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, Animation, 720p | #VALUE! | #VALUE! | #VALUE! |
| YUV, camera captured, 1080p | #VALUE! | #VALUE! | #VALUE! |
| Enc Time[%] | #VALUE! |  |  |
| Dec Time[%] | #VALUE! |  |  |

Table 2 : Lossless coding efficiency (Anchor SCM3.0 IBC 1x4)

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |
|  | Random Access |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |
|  | Low Delay B |  |  |  |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #NUM! |  |  |  |
| Dec Time[%] | #NUM! |  |  |  |

# Recommendation

It is recommended to further study 2-D string copy method in CE.

# References

[1] W. Wang, Z. Ma, M. Xu, H. Yu, “Non-CE6: 2-D Index Map Coding of Palette Mode in HEVC SCC”, Doc. JCT-VC S0151, Strasbourg, FR, Oct. 2014.

[2] H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common Test Conditions for Screen Content Coding”, Doc. JCT-VC S1015, Strasbourg, FR, Oct. 2014.

# Patent rights declaration(s)

Huawei R&D USA may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).