| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0195 |
| --- | --- |

| Title: | Non-CE2: Improved binarization for BVD/MVD |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim, Kiran Misra, Jie Zhao, Andrew Segall 5750 NW Pacific Rim Blvd, Camas, WA 98607 USA | Tel: Email: | +1 360 817 8451 kimse@sharplabs.com misrak@sharplabs.com jzhao@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution proposes an improved binarization for BVD and MVD. Specifically three regular bins are coded to indicate if the BVD (or MVD) value is greater than 0, 4, and 20, respectively. The remaining bins are bypass coded using a Exp-Golomb(EG) or fixed length code. The proposed binarization is tested on top of on top of unification framework CE2 test 8.3. It is reported that the proposed binarization method provides objective luma bit rate reductions of (1.3 %, 1.5%), (3.4%, 3.9%), (4.0%, 4.3%) for All Intra, Random access and Low Delay-B text and graphics RGB and YUV categories, respectively over SCM-3.0 anchor.

# Introduction

In the current HEVC screen content coding draft [1], block vector difference (BVD) is coded with the following syntax elements shown in Table1. Also the conventional motion vector difference (MVD) is coded with syntax elements shown in Table 2.

Table 1. Syntax elements for BVD

bvd_coding ( x0, y0, refList ) {

abs_bvd_greater0_flag[ 0 ]

abs_bvd_greater0_flag[ 1 ]

if( abs_bvd_greater0_flag[ 0 ] ) {

abs_bvd_minus1[ 0 ]

bvd_sign_flag[ 0 ]

}

if( abs_bvd_greater0_flag[ 1 ] ) {

abs_bvd_minus1[ 1 ]

bvd_sign_flag[ 1 ]

}

}

Table 2. Syntax elements for MVD

mvd_coding( x0, y0, refList ) {

abs_mvd_greater0_flag[ 0 ]

abs_mvd_greater0_flag[ 1 ]

if( abs_mvd_greater0_flag[ 0 ] )

abs_mvd_greater1_flag[ 0 ]

if( abs_mvd_greater0_flag[ 1 ] )

abs_mvd_greater1_flag[ 1 ]

if( abs_mvd_greater0_flag[ 0 ] ) {

if( abs_mvd_greater1_flag[ 0 ] )

abs_mvd_minus2[ 0 ]

mvd_sign_flag[ 0 ]

}

if( abs_mvd_greater0_flag[ 1 ] ) {

if( abs_mvd_greater1_flag[ 1 ] )

abs_mvd_minus2[ 1 ]

mvd_sign_flag[ 1 ]

}

}

# Proposed Method

In this contribution, unified binarization is considered for MVD and BVD.  Specifically three following flags are introduced and context coded.

- Greater0 flag: greater than zero flag

- Greater4 flag: greater than four flag

- Greater20 flag: greater than twenty flag

In Table 3 the unified MVD and BVD coding based on the proposed binarization is shown.

Xvd_coding( x0, y0, refList ) {

abs_Xvd_greater0_flag[ 0 ]

abs_Xvd_greater0_flag[ 1 ]

if( abs_xvd_greater0_flag[ 0 ] ){

abs_Xvd_greater4_flag[ 0 ]

if( abs_Xvd_greater4_flag[ 0 ] ==0)

abs_Xvd_minus1[ 0 ]

else{

abs_Xvd_greater20_flag[ 0 ]

if( abs_xvd_greater20_flag[ 0 ] ==0)

abs_Xvd_minus5[ 0 ]

else

abs_Xvd_minus21[ 0 ]

}

bvd_sign_flag[ 0 ]

}

if( abs_Xvd_greater0_flag[ 1 ] ){

abs_Xvd_greater4_flag[ 1 ]

if( abs_Xvd_greater4_flag[ 1 ] ==0)

abs_Xvd_minus1[ 1 ]

else{

abs_Xvd_greater20_flag[ 1 ]

if( abs_Xvd_greater20_flag[ 1 ] ==0)

abs_Xvd_minus5[ 1 ]

else

abs_Xvd_minus21[ 1 ]

}

bvd_sign_flag[ 1 ]

}

}

# Experimental Results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM3.0, and the common test conditions specified in Error: Reference source not found are followed. The proposed method is implemented on top of CE2 test 8.3 [2]. The compression performance is measured using BD rate compared with SCM-3.0 anchor.

Lossy:

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.3% | -2.7% | -2.6% |
| RGB, mixed content, 1440p & 1080p | -1.0% | -1.7% | -1.9% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.5% | -2.7% | -2.9% |
| YUV, mixed content, 1440p & 1080p | -1.0% | -1.9% | -2.0% |
| YUV, Animation, 720p | -0.1% | -0.1% | -0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -3.4% | -5.2% | -5.1% |
| RGB, mixed content, 1440p & 1080p | -1.0% | -2.6% | -2.6% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.2% |
| RGB, camera captured, 1080p | -0.2% | 0.0% | -0.2% |
| YUV, text & graphics with motion, 1080p & 720p | -3.9% | -5.5% | -5.6% |
| YUV, mixed content, 1440p & 1080p | -1.3% | -3.1% | -3.3% |
| YUV, Animation, 720p | -0.3% | -0.5% | -0.2% |
| YUV, camera captured, 1080p | -0.2% | -0.1% | -0.3% |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -4.0% | -5.3% | -5.2% |
| RGB, mixed content, 1440p & 1080p | -1.3% | -2.8% | -3.1% |
| RGB, Animation, 720p | -0.1% | -0.2% | -0.3% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | -4.3% | -5.5% | -5.9% |
| YUV, mixed content, 1440p & 1080p | -1.7% | -4.0% | -4.6% |
| YUV, Animation, 720p | -0.1% | -0.4% | -0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.2% | 0.1% |

Lossless:

|  | All Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | 0.2% | 0.1% | -1.5% | 1.7% |
| RGB, mixed content, 1440p & 1080p | -1.0% | -0.8% | -2.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | ### | ### | ### | ### |
| YUV, mixed content, 1440p & 1080p | -0.5% | -0.4% | -1.6% | 0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |

|  | Random Access |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | -0.7% | -1.2% | -3.6% | 0.0% |
| RGB, mixed content, 1440p & 1080p | ### | ### | ### | ### |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.2% | -0.8% | -3.3% | 0.2% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.4% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |

|  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit-rate change (Total) | Bit-rate change (Average) | Bit-rate change (Min) | Bit-rate change (Max) |
| RGB, text & graphics with motion, 1080p & 720p | ### | ### | ### | ### |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.3% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -0.3% | -1.2% | -6.0% | 0.1% |
| YUV, mixed content, 1440p & 1080p | -0.1% | -0.1% | -0.2% | 0.0% |
| YUV, Animation, 720p | ### | ### | ### | ### |
| YUV, camera captured, 1080p | ### | ### | ### | ### |

# Conclusion

This contribution proposes improved entropy coding for BVD and MVD. Specifically three regular bins are coded to indicate if the BVD (or MVD) value is greater than 0, 4, and 20, respectively. The remaining bins are bypass coded using a Exp-Golomb(EG) or fixed length code.

# Patent rights declaration(s)

SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Rajan Joshi, Jizheng Xu, “High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 2,” 19th meeting of Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Strasbourg, FR, 17 Oct. – 24 Oct. 2014

[2] S. Liu, C. Pang, and J. Xu, “Description of Core Experiment 2 (CE2): Intra block copy relationship to inter coding”, JCTVC-S1102, Strasbourg, FR, October 2014.

# Draft specification changes

| Table 9-xx – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| mvd_coding ( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater4_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater20_flag[ ] | FL | cMax=1 |
|  | abs_mvd_minus1 [ ] | FL | cMax =4 |
|  | abs_mvd_minus5[] | FL | cMax=16 |
|  | abs_mvd_minus5 | EG5 |  |
|  | mvd_sign_flag [ ] | FL | cMax = 1 |