| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0196 |
| --- | --- |

| Title: | On selective RDOQ |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Tel: Email: | [phone contact number] [email address(es)] |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This document proposes a fast encoding algorithm to bypass the RDOQ process for selected blocks to speed up the encoding process. The experimental results show that the proposed fast encoding algorithm saves 3%~4% encoding time without noticeable performance drop under SCC CTC.

# Introduction

This document proposes a fast encoding algorithm to bypass the RDOQ process for selected blocks to speed up the encoding process. The experimental results show that the proposed fast encoding algorithm saves 3%~4% encoding time without noticeable performance drop under SCC CTC.

# Solution

The proposed algorithm uses a pre-analysis method to determine bypass the RDOQ process or not. The pre-analysis is just simple quantization. If all the coefficients are quantized to 0, the RDOQ process is bypassed. Otherwise, the RDOQ process is performed as usual.

In the original quantization process of HM, an offset of 1/6 is applied to P or B slices and an offset of 1/3 is applied to I slices. In the proposed pre-analysis step, an offset of 1/3 is applied to luma component and an offset of 1/2 applied to chroma components.

# Experimental results

HM-16.2+SCM-3.0 default coding structure is used as the anchor. The common test conditions specified in [1] are followed. The summarized experimental results are shown in the following tables. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for lossy coding (444 sequences)

Table 2 Experimental results for lossy coding (420 sequences)

We also performed tests under HM CTC and RExt CTC. The experimental results are shown in the following tables.

Table 3 Experimental results for lossy coding (HM CTC)

Table 4 Experimental results for lossy coding (RExt CTC)

# References

- JCTVC-S1015, “Common conditions for screen content coding tests,” Strasbourg, FR, 17–24 Oct. 2014.

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).