| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0209 |
| --- | --- |

| Title: | A HEVC SEI Message for Green Metadata |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiangtao Wen Tsinghua University Beijing, China Spencer Cheng Nanjing Yunyan Technology Ltd. Nanjing, China | Email: | jtwen@tsinghua.edu.cn spencer@morphbius.com |
| Source: | Tsinghua University and Nanjing Yunyan Information Technology Ltd. |  |  |

_____________________________

# Abstract

An HEVC SEI message is proposed for the carriage of Green Metadata (ISO/IEC 23001-11). The metadata enables cross-segment decoding for quality recovery after low-power encoding.

# Introduction

An HEVC SEI message is proposed for the carriage of Green Metadata (ISO/IEC 23001-11). The metadata enables cross-segment decoding for quality recovery after low-power encoding.

# Proposed SEI Message Syntax for Green Metadata

The proposed text changes for the general HEVC SEI message syntax are shown in Table 1. The proposed syntax for the Green Metadata SEI message is shown in Table 2.

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| … |  |
| else if ( payloadType == xx) |  |
| green_metadata( payloadSize ) |  |
| Else |  |
| reserved_sei_message( payloadSize ) |  |

Table 1: Proposed text changes for general HEVC SEI message syntax

|  | C | Descriptor |
| --- | --- | --- |
| green_metadata( payload_size ) |  |  |
| green_metadata_type | 5 | u(8) |
| switch (green_metadata_type) { |  |  |
| case 0: |  |  |
| // reserved |  |  |
| case 1: |  |  |
| xsd_metric_type | 5 | u(8) |
| xsd_metric_value | 5 | u(16) |
| break; |  |  |
| default: |  |  |
| } |  |  |

Table 2: Proposed Green Metadata SEI Message Syntax in HEVC

xsd_metric_type -- indicates the type of the objective quality  metric as shown in Table 3. PSNR is the only type currently supported. Please see ISO/IEC 23001-10 [3] for definition of PSNR.

xsd_metric_value - contains the metric value of the last picture of the Segment. When xsd_metric_type is 0, then the stored 16-bit unsigned integer xsd_metric_value, is interpreted as a floating-point PSNR value (in dB) as follows:

| Value | Description |
| --- | --- |
| 0x00 | PSNR |
| 0x01-0xFF | User-defined |

Table 3: xsd_metric_type values

# Patent Rights Declaration(s)

Tsinghua University and Nanjing Yunyan Information Technology Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).