| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0212 |
| --- | --- |

| Title: | Cross-check of JCTVC-T0051 On IBC memory reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krishna Rapaka 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-2042 krapaka@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports cross checking results of JCTVC-T0051 on IBC memory reduction for screen content. It was confirmed that the cross checking results exactly match the results provided by the proponents and the software implementation matches the proposed method description in JCTVC-T0051.

# Introduction

The document JCTVC-T0051 asserts that the current IBC increases the memory needed for some implementations due to additional storage of non-filtered blocks. To avoid this additional storage, this JCTVC-T0051 proposes to signal the CTBs which are available for IBC prediction. For these CTBs, the DBF and SAO are disabled.

The cross checking simulations were performed using x64 Linux cluster. The software was provided by the proponent and compiled with GCC 4.1.2. The run time information may not be accurate.

# Simulation Results

The results were tested under the SCC common test conditions specified in [2].  Complete results can be found in the attached spreadsheets.

Table I. JCTVC-T0051 Lossy 444 configuration

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.5% | -1.0% | -0.8% |
| RGB, mixed content, 1440p & 1080p | 0.9% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.2% | 0.6% | 0.2% |
| RGB, camera captured, 1080p | 0.3% | 0.2% | 0.7% |
| YUV, text & graphics with motion, 1080p & 720p | 0.9% | 0.0% | -0.2% |
| YUV, mixed content, 1440p & 1080p | 1.2% | 1.1% | 1.1% |
| YUV, Animation, 720p | 0.6% | 1.2% | 0.7% |
| YUV, camera captured, 1080p | 0.7% | 0.9% | 0.7% |
| Enc Time[%] | 114% |  |  |
| Dec Time[%] | #VALUE! |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.4% | -1.4% | -1.3% |
| RGB, mixed content, 1440p & 1080p | 0.3% | -1.3% | -1.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.8% | -0.9% | -1.2% |
| YUV, mixed content, 1440p & 1080p | 0.7% | -1.1% | -1.0% |
| YUV, Animation, 720p | 0.2% | 0.1% | 0.2% |
| YUV, camera captured, 1080p | 0.2% | 0.3% | 0.1% |
| Enc Time[%] | 111% |  |  |
| Dec Time[%] | #VALUE! |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.6% | -0.7% | -0.6% |
| RGB, mixed content, 1440p & 1080p | 0.3% | -1.0% | -1.1% |
| RGB, Animation, 720p | 0.0% | 0.1% | -0.2% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.9% | -0.6% | -0.9% |
| YUV, mixed content, 1440p & 1080p | 0.7% | -1.7% | -1.7% |
| YUV, Animation, 720p | 0.1% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | #VALUE! |  |  |

Table II. JCTVC-T0051 Lossy 420 configuration

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | 1.9% | 3.5% | 3.4% |
| Mixed content, 480p | 1.2% | 3.6% | 3.7% |
| Animation, 768p | 1.2% | 2.7% | 4.4% |
| Average of all sequences | 1.5% | 3.3% | 3.7% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | 1.2% | 1.2% | 1.6% |
| Mixed content, 480p | 0.2% | 0.4% | 0.3% |
| Animation, 768p | 0.7% | 1.9% | 2.6% |
| Average of all sequences | 0.8% | 1.2% | 1.5% |
| Enc Time[%] | 114% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| Text & graphics with motion, 720p | 0.9% | 1.3% | -0.8% |
| Mixed content, 480p | 0.2% | 0.0% | -0.1% |
| Animation, 768p | 0.3% | 0.4% | 0.0% |
| Average of all sequences | 0.6% | 0.7% | -0.5% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 100% |  |  |

# Reference

- G. Laroche, G. Malard, T. Poirier, C. Gisquet, P. Onno (Canon)“ AHG10: On IBC memory reduction,” JCTVC-T0051.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” JCTVC-S1015.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).