| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0218 |
| --- | --- |

| Title: | CE1-related: Improvements of draft text related to palette sharing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | PoLin Lai, Shan Liu, and Shawmin Lei 2860 Junction Ave, San Jose, CA, USA | Email: | {polin.lai, shan.liu, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents observations on the draft text related to palette share. It is asserted that there exists redundant text parts, and this contribution presents modifications to remove the redundant parts.

# Observation and proposed changes in draft text

It is observed that there exists redundant text in the draft text on palette. When palette_share_flag is 1, palette_num_signalled_entries is not present, it is inferred to be equal to 0. Thus, there is no need to separate the derivation of CurrentPaletteEntries into two cases for palette_share_flag, there is also no need to separate the derivation of CurrentPaletteSize into two cases for palette_share_flag, as currently specified in the draft text.

The following is extracted from HEVC Screen Content Coding Draft Text 2 [1]. The proposed changes are highlighted.

-----

[MOVED FROM BELOW TO HERE] The variable PalettePredictorEntryReuseFlag[ i ] equal to 1 specifies that the i-th entry in the predictor palette is reused in the current palette. PalettePredictorEntryReuseFlag[ i ] equal to 0 specifies that the i-th entry in the predictor palette is an entry in the current palette. All elements of the array PalettePredictorEntryReuseFlag[ i ] are initialized to be equal to zero.

palette_share_flag[ x0 ][ y0 ] equal to 1 specifies that the palette for the current coding unit is derived by copying the first PreviousPaletteSize entries from the predictor palette. The variable PreviousPaletteSize is derived as specified in subclause Error: Reference source not found. palette_share_flag [ x0 ][ y0 ] equal to 0 specifies the palette for the current coding unit is specified as a combination of palette entries from previous coding units and new palette entries which are explicitly signalled.

[MOVED TO ABOVE] The variable PalettePredictorEntryReuseFlag[ i ] equal to 1 specifies that the i-th entry in the predictor palette is reused in the current palette. PalettePredictorEntryReuseFlag[ i ] equal to 0 specifies that the i-th entry in the predictor palette is an entry in the current palette. All elements of the array PalettePredictorEntryReuseFlag[ i ] are initialized to be equal to zero.

If palette_share_flag [ x0 ][ y0 ] is equal to 1, the array PalettePredictorEntryReuseFlag[ i ] is further modified as follows:

for( i = 0; i < PreviousPaletteSize; i++ )		(7-1)
		PalettePredictorEntryReuseFlag[ i ] = 1

palette_predictor_run specifies the number of zeros that precede a non-zero entry in the array predictor_palette_entry_reuse_flag. The value of palette_prediction_run shall be in the range of 0 to palette_max_predictor_size, inclusive.

The variable paletteNumPredictedEntries specifies the number of entries in the current palette that are reused from the predictor palette. The value of paletteNumPredictedEntries shall be in the range of 0 to max_palette_ size, inclusive.

palette_num_signalled_entries specifies the number of entries in the current palette that are explicitly signalled.

When palette_num_signalled_ entries is not present, it is inferred to be equal to 0.

The variable CurrentPaletteSize specifies the size of the current palette and is derived as follows:

- If palette_share_flag [ x0 ][ y0 ] is equal to 1,

CurrentPaletteSize = PreviousPaletteSize		(7-2)

- Otherwise (palette_share_flag [ x0 ][ y0 ] is equal to 0)

CurrentPaletteSize = paletteNumPredictedEntries + palette_num_signalled_entries	(7-3)

The value of CurrentPaletteSize shall be in the range of 0 to palette_max_size, inclusive.

palette_entry specifies the value of a component in a palette entry for the current palette.

The variable PredictorPaletteEntries[ cIdx ][ i ] specifies the i-th element in the predictor palette for the colour component cIdx.

The variable CurrentPaletteEntries[ cIdx ][ i ] specifies the i-th element in the current palette for the colour component cIdx and is derived as follows:

When palette_share_flag [ x0 ][ y0 ] is equal to 1, CurrentPaletteEntries is derived as follows:

- If palette_share_flag [ x0 ][ y0 ] is equal to 1,

for( cIdx = 0; cIdx < 3; cIdx++ )
	for( i = 0; i < PreviousPaletteSize; i++ )		(7-4)
		CurrentPaletteEntries[ cIdx ][ i ] = PredictorPaletteEntries[ cIdx ][ i ]

- Otherwise (palette_share_flag [ x0 ][ y0 ] is equal to 0)

numPredictedPaletteEntries = 0
for( i = 0; i < PredictorPaletteSize; i++ ) 
	if( PalettePredictorEntryReuseFlag[ i ] = = 1 ) {
		for( cIdx = 0; cIdx < 3; cIdx++ ) 
			CurrentPaletteEntries[ cIdx ][ numPredictedPaletteEntries ] = PredictorPaletteEntries[ cIdx ][ i ]
		numPredictedPaletteEntries++ 
	}
for( cIdx = 0; cIdx < 3; cIdx++ ) 		(7-5)
	for( i = 0; i < num_signalled_palette_entries; i++ ) 
		CurrentPaletteEntries[ cIdx ][ numPredictedPaletteEntries + i ] = palette_entry

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “HEVC Screen Content Coding Draft Text 2,” Document of Joint Collaborative Team on Video Coding, JCTVC-S1005, Oct. 2014