| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10 Feb. – 18 Feb. 2015 | Document: JCTVC-T0224 |
| --- | --- |

| Title: | CE3-related: Worst case SRAM and DDR bandwidth analysis and solutions for ISC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tao Lin 1239 Siping Ave, Shanghai 200092, China | Tel: Email: | lintao@tongji.edu.cn |
| Source: | Tongji University |  |  |

_____________________________

# Abstract

This contribution provides worst case SRAM and DDR bandwidth analysis and solutions for ISC.

# Worst case SRAM bandwidth analysis and solutions

All real-time video decoder chips including general-purpose one or ASIC or one embedded in SoC use at least 187MHz system clock frequency for full HD (1920x1080x60=124.42M-pixel/second) in single processor unit architecture. A great majority of full HD video decoder chips with single processor unit architecture operate at a system clock frequency between 200MHz and 400MHz.

Nobody will design a decoder chip at a very low frequency and then make a throughput of multiple pixels per clock cycle to keep real-time operation. It is a basic design principle to select a system clock frequency that is high enough to have at least 1.5 cycles to decode and output a pixel for real-time operation. If it is impossible to do so, then multi-processor or parallel-processing architecture will be used. This principle is always followed to achieve cost-effective chip and system design. If this principle is not followed, then the resulting chip and system will need to waste a lot more buffering space in every stage of the pipeline. No chip design engineer will design a video decoder chip in such an inefficient way.

When at least one clock is available per pixel, then an operation will never have SRAM bandwidth problem as long as the operation is simple enough to need at most only one SRAM access per pixel to complete.

ISC decoding is just such a simple operation. It only needs at most one SRAM read to copy a reference pixel. No index conversion, no interpolation, no filtering, nothing extra is needed. Therefore, regardless of SRAM memory pattern size, ISC decoding will not have SRAM bandwidth problem in any cases.

Even from P-value perspective, using 2x2-pixels or 4x2-pixels or 2x4-pixels memory pattern size, the worst case P-values are 4, 8, 8 vs 6, 7.5, 8 for inter 4x8 Uni-prediction.

Therefore, from any point of view, SRAM bandwidth is not a problem at all.

SRAM cost is also not a problem. There are 2 options:

- Reuse the existing 2CTU SRAM for any other modes. It is free for ISC.

- Use dedicated SRAM, the cost is 0.4 cents at 28 nano-meter semiconductor process technology as calculated below.

2CTU SRAM bits:		2x64x64x24=196608

Silicon area at 28nm:		0.075 mm2

12 inch (300mm) wafer cost:	3000 USD

Wafer utilization:		80%

The cost for 2CTU:		3000*0.075/(3.14*150*150*0.8)=0.00398 USD=0.4 cents

Therefore, both SRAM bandwidth and cost are not a problem at all in ISC.

# Worst case DDR bandwidth analysis and solutions

In all video decoder chips supporting 4K, the DDR bus width is at least 64-bit. In all video decoder chips supporting full HD, the DDR bus width is at least either 32-bit or 64-bit. In DDR-3, DDR burst size is 8. Therefore, the minimum DDR memory pattern size is either 4x8 or 8x4 or 8x8.

The minimum string length of ISC can be limited to 20 pixels for depth＝3 and 36 pixels for depth=0~2 outside of 2CTU . Therefore, the worst case P-value is illustrated below:

8x4

8x4

8x4

8x4

4x8

4x8

4x8

4x8

4x8

4x8

8x8

8x8

8x8

8x8

8x4

8x4

8x4

8x8

8x8

8x8

8x4

8x8

4x8

4x8

4x8

4x8

4x8

4x8

8x4

8x4

P = 12x4x8/36 = 10.7

P = 8x8x8/36 = 14.2

8x4

8x4

P = 12x8x4/36 = 10.7

Depth = 0~2

8x8

8x8

4x8

4x8

8x8

8x8

P = 4x8x8/20 = 12.8

4x8

4x8

4x8

8x4

8x4

8x4

8x4

8x4

8x4

4x8

P = 6x8x4/20= 9.6

P = 6x4x8/20 = 9.6

Depth = 3

In ISC, the worst case P-value for 8x4, 4x8, 8x8 memory patterns are 10.7, 10,7, 14.2 vs 8, 8, 16 for IBC and 12, 15, 18 for inter 4x8 Uni-prediction.

ISC uses IBC buffer and no extra buffer or dictionary is needed.

Therefore, both DDR bandwidth and cost are not a problem at all in ISC.

# Conclusions

- ISC does not have SRAM and DDR bandwidth problem.

- SRAM cost for ISC is either zero or 0.4 penny.

- ISC operation is extremely simple because only simple and direct copy (memory read and write) is involved. The complete ISC decoding logic takes less than 10K gates which cost less than 0.1 penny.

- Using SCM3.0 FF IBC as anchor, ISC can deliver about -7% BD-rate reduction in lossy coding and about -10% bit-rate saving in lossless coding at almost no silicon implementation cost.

.