| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0227 |
| --- | --- |

| Title: | Non-CE2 Test1: Intra block copy and inter signalling unification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chao Pang, Ye-Kui Wang, Vadim Seregin, Krishna Rapaka, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA Xiaozhong Xu, Shan Liu, Shawmin Lei MediaTek Bin Li, Jizheng Xu Microsoft Corp. | Tel: Email: | + 1-858-658-3779 cpang@qti.qualcomm.com xiaozhong.xu@mediatek.com shan.liu@mediatek.com shawmin.lei@mediatek.com libin@microsoft.com jzxu@microsoft.com |
| Source: | Qualcomm Inc., MediaTek, Microsoft Corp. |  |  |

_____________________________

# Abstract

This proposal reports results of using HEVC AMVP and MVD on top of CE2 Test 1. The performance is reported to be an average BD-rate of -1.0% and -1.3%, respectively for 1080p & 720p YUV SC text & graphics with motion sequences under RA and LB configuration, respectively.

# Introduction

In CE2 Test 1 [1], the Intra BC mode is signalled reusing inter signalling by adding the current picture (without in-loop filtering) into the reference picture list. With the shared signalling, Intra BC can be differentiated from the conventional Inter mode by checking reference index of the reference picture used in the prediction. If the reference picture is the current picture, then the prediction mode is Intra BC mode. Otherwise, it is conventional Inter mode.

# Proposed method

In this proposal, different from CE2 Test 1, HEVC AMVP and MVD are used for both Intra BC mode and conventional inter mode.

# Experimental results

In this section, the experimental results of the proposed methods are presented. All the simulations are conducted on top of SCM3.0, and the common test conditions specified in Error: Reference source not found are followed. Moreover, the results of signalling block vector (BV) for Intra BC in the units of 1-pixel and ¼-pixel are provided. The summarized experimental results of 4:4:4 format sequences for lossy with different BV precision are shown in Table 1 and Error: Reference source not found, respectively. More detailed experimental results could be found in the attached spreadsheets.

Table 1 Experimental results for lossy coding with 1-pixel BV precision

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.6% | -0.8% | -0.8% | -1.0% | -2.7% | -2.7% | -1.3% | -2.7% | -2.7% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.8% | -0.9% | -0.1% | -1.8% | -1.8% | -0.5% | -2.0% | -2.1% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.0% | 0.1% | -0.1% | 0.1% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.1% | -0.2% | -0.1% | -0.1% | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 1.0% | -0.3% | -0.5% | -1.0% | -2.6% | -2.9% | -1.3% | -2.8% | -3.1% |
| YUV, mixed content, 1440p & 1080p | 0.3% | -0.8% | -0.9% | -0.2% | -2.1% | -2.2% | -0.8% | -3.3% | -3.2% |
| YUV, Animation, 720p | 0.1% | 0.1% | 0.1% | 0.0% | -0.2% | 0.1% | 0.1% | -0.1% | 0.4% |
| YUV, camera captured, 1080p | 0.1% | 0.2% | 0.2% | -0.2% | 0.0% | 0.0% | 0.0% | 0.1% | 0.2% |
| Enc Time[%] | 117% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 104% |  |  | 94% |  |  | 100% |  |  |

Table 2 Experimental results for lossy coding with 1/4-pixel BV precision

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | G/Y | B/U | R/V | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 3.4% | 1.8% | 1.9% | 0.4% | -1.5% | -1.5% | -0.7% | -2.1% | -2.1% |
| RGB, mixed content, 1440p & 1080p | 1.5% | 0.5% | 0.5% | 0.7% | -0.9% | -1.0% | -0.1% | -1.6% | -1.9% |
| RGB, Animation, 720p | 0.4% | 0.5% | 0.4% | 0.2% | 0.2% | 0.1% | 0.2% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.1% | 0.1% | 0.2% | -0.1% | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 4.2% | 2.9% | 2.5% | 0.5% | -1.2% | -1.6% | -0.6% | -2.2% | -2.3% |
| YUV, mixed content, 1440p & 1080p | 2.3% | 0.9% | 0.8% | 1.0% | -1.2% | -1.3% | -0.3% | -3.2% | -3.3% |
| YUV, Animation, 720p | 0.7% | 0.8% | 0.6% | 0.2% | 0.1% | 0.3% | 0.2% | 0.3% | 0.0% |
| YUV, camera captured, 1080p | 0.2% | 0.4% | 0.3% | -0.1% | 0.1% | 0.0% | 0.0% | 0.2% | 0.3% |
| Enc Time[%] | 118% |  |  | 105% |  |  | 104% |  |  |
| Dec Time[%] | 106% |  |  | 97% |  |  | 101% |  |  |

# References

[1] S. Liu, C. Pang, and J. Xu, “Description of Core Experiment 2 (CE2): Intra block copy relationship to inter coding”, JCTVC-S1102, Strasbourg, FR, October 2014.

[2] H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding”, JCTVC-S1015, Strasbourg, FR, October 2014.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).