| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0230 |
| --- | --- |

| Title: | BoG report on IBC memory Bandwidth reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Minhua Zhou | Email: | krapaka@qti.qualcomm.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This document provides a report of the BoG on intra block copy (IBC) memory bandwidth reduction.

# Introduction

A BoG was held on Saturday February 14th, 2014, 1:00 pm to 3:00 pm to discuss methods related to IBC memory bandwidth reduction. The mandates of the BoG were:

- Discuss and detail methods/configurations to be tested related to IBC memory bandwidth reduction.

- Discuss issues related to IBC memory bandwidth in the current SCC test model

# List of Proposals

- JCTVC-T0045, AHG10: Memory bandwidth reduction for intra block copy, J. Lainema, M. M. Hannuksela (Nokia)

- JCTVC-T0051, AHG10: On IBC memory reduction, G. Laroche, G. Malard, T. Poirier, C. Gisquet, P. Onno (Canon)

- JCTVC-S0145, Bandwidth reduction method for intra block copy, K. Rapaka, T. Hsieh, C. Pang, V. Seregin, M. Karczewicz(Qualcomm)

# BoG suggestions

## Bandwidth Analysis

It was suggested to investigate the IBC memory bandwidth analysis under the full frame search area configuration. The bandwidth analysis includes the worst-case and average case memory bandwidth.

## Methods to be investigated

### JCTVC-T0045/ JCTVC-T0051: signal the CTBs which are available for IBC prediction. For these CTBs, the filtering is disabled.

### Combination Test: The encoder method of JCTVC-T0045 is added over JCTVC-T0051

### Signal the CTBs which are available for IBC prediction to indicate for decoder and not disabled filtering operation (JCTVC-S0145).

### Restrict that the any reference pixel for Intra BC cannot be from bi-predicted blocks. This method will have same worst case bandwidth as HEVC.

### Sequence and picture level control: - The filtering is disabled sequence level and adaptively at picture level.

### Under CTC, further reduce the number of inter reference pictures for RA/LD-B by 1 to compensate for the IBC reference picture.

## Suggested test configuration for the CE related to IBC memory reduction

### The IBC search range configurations to be tested are specified in the following table:

| Configuration | 8x4 | 4x8/8x8/16x16/32x32 |
| --- | --- | --- |
| Config 1 | 2x1-CTU | Full-frame |
| Config2 | 4x1-CTU | 4x1-CTU |

### Additional test configuration: CTU size of 16x16 and local search area size in terms of samples is kept same as config 1 (128x64)

### Expert subjective analysis to be performed for the methods that change filtering operations (Deblock/SAO)

It was suggested to use lower QP (in the range of QP=37) for the above test.

It was also suggested to test under LD-P configuration.

# BoG Discussion and Recommendation

# It was agreed that the methods 3.1.1, 3.1.2, 3.1.5 are reasonable only with use of local cache.

BoG recommends that there be a CE on the topic