| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0231r1 |
| --- | --- |

| Title: | Non-CE1: Harmonization of grouping palette indices at front (JCTVC-T0065) and extended copy above mode to the first line Test A.1.5 (JCTVC-T0036) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Wei Pu, Rajan Joshi Feng Zou, Vadim Seregin 5775 Morehouse Drive San Diego, CA 92121, USA Yu-Chen Sun1, Jungsun Kim2, Tzu-Der Chuang1, Yi-Wen Chen1, Shan Liu2, Yu-Wen Huang1, Shawmin Lei1 1 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2 MediaTek San Jose: 2860 Junction Ave, San Jose, CA 95134, US | Tel: Email: | 1-858-651-7749 martak@qti.qualcomm.com Wei.Pu@qti.qualcomm.com 886-3-5670766 ext. 24572 {yc.sun, jungsun.kim, peter.chuang, yiwen.chen, shan.liu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Qualcomm Incorporated MediaTek Inc. |  |  |

_____________________________

# Abstract

This document proposes a harmonization method of grouping palette indices at front (JCTVC-T0065) and extended copy above mode to the first line Test A.1.5 (JCTVC-T0036). Compared with SCM3.0 anchor under SCC common test condition, the proposed harmonization results 1.3% BDBR for “YUV, text & graphics with motion, 1080p & 720p sequences” under AI, RA, and LB, respectively.

# Introduction

In the current palette mode design Error: Reference source not found in screen content coding standard, the syntax elements of palette_index_idc which are CABAC bypass coded are interleaved with other syntax elements such palette_run_msb_id_plus1 which are CABAC context coded. JCTVC-T0065 [2] proposes a method of grouping all of the occurrences of palette_index_idc at front.

JCTVC-T0036 [3] Test A.1.5 proposes to enable Copy Above mode by directly using a neighboring pixel value outside the current CU to represent a current pixel instead of converting the outside pixels into palette indices.

# Proposed harmonization method of JCTVC-T0065 and JCTVC-T0036

In JCTVC-T0036, during index coding, conditional redundancy removal is used to reduce the dynamic range of the possible palette indices for truncated binary coding. Specifically, if the previous pixel uses Copy Above, and the pixel above the current pixel is from outside the block, the redundancy removal is disabled.

Due to this, when index grouping is enabled, the maximum value for the indices is not known as a priori. Note that Indices are sent with truncated binary coding. To encode or decode indices using truncated binary coding, it is necessary to know the maximum value that an index may take. Let Cmax denote the maximum index value plus 1. For example, if there are 7 entries in the palette and escape pixels are present in the palette block, then Cmax is equal to 8 and the indices range from 0 to 7, inclusive. Similarly, if there are 5 entries in the palette and escape pixels are not present, then Cmax is equal to 5 and the indices range from 0 to 4, inclusive.

Therefore, to harmonize JCTVC-T0036 and JCTVC-T0065, it is proposed to always apply the index redundancy removal by reducing the palette index range by 1. First, let us denote the palette size as (indexMax + 1). And the number of Index run mode is coded using coefficient remaining codeword. Specifically, the difference between this value and indexMax is coded into the bitstream using HEVC coefficient coding method, i.e. by calling the function of xWriteCoefRemainExGolomb(.) in HM with parameter equal to 2+indexMax/6. If negative value occurs, they are mapped to positive values with equal spacing of 16 in non-palette share mode., n palet

When the previously coded pixel is Copy Above, and the pixel above the current pixel is from outside, the following procedures are applied to code the current pixel palette index K:

- If the current pixel palette index K is smaller than uiMaxSymbol, K is coded using truncated binary codeword with the range uiMaxSymbol;

- Otherwise, (the current pixel palette index K is equal to uiMaxSymbol), (K – 1) is coded using truncated binary codeword with the range uiMaxSymbol, followed by a flag to indicate whether it is the current palette index is real (K – 1) or K.

Simulation Results

Simulations are performed under the test condition specified in CE1 description [6] and SCC common test condition [2]. Only AI lossy results are presented in this section. The complete test results are attached in the proposal package.

Table 1. Proposed harmonization method v.s. SCM3.0 Anchor

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -1.3% | -1.3% | -1.4% |
| RGB, mixed content, 1440p & 1080p | #VALUE! | #VALUE! | #VALUE! |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | -1.5% | -1.5% | -1.4% |
| YUV, mixed content, 1440p & 1080p | #VALUE! | #VALUE! | #VALUE! |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | #VALUE! |  |  |
| Dec Time[%] | #VALUE! |  |  |

# Conclusion

In this document, a method of harmonizing JCTVC-T0036 and JCTVC-T0065 is proposed to solve the index redundancy removal issue. Simulation results indicate that this method does not affect coding efficiency while improving throughput.

# References

- R. Joshi and J. Xu, “HEVC screen content coding draft text 2,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting, JCTVC-S1005, Strasbourg, FR, 17–24 Oct. 2014.

- M. Karczewicz, W. Pu, R. Joshi, V. Seregin, “Non CE1: Grouping Palette Indices At Front,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting, JCTVC-T0065, Geneva, CH, 10–18 Feb. 2015

- Y.-C. Sun, J. Kim, S. Liu, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei, V. Seregin, F. Zou, W. Pu, R. Joshi, M. Karczewicz, “CE1 : Tests A.1 : Extended copy above mode to the first line (1.1-1.5),” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting, JCTVC-T0036, Geneva, CH, 10–18 Feb. 2015

- P. Lai, P. Onno, R. Cohen, V. Seregin, X. Xiu, and Z. Ma, “Description of core experiment 1 (CE1): Improvements of palette mode,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting, JCTVC-S1101, Strasbourg, FR, 17–24 Oct. 2014.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu, “Common test conditions for screen content coding,” in Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting, JCTVC-S1015, Strasbourg, FR, 17–24 Oct. 2014.

- Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Mediatek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).