| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T0233 |
| --- | --- |

| Title: | CE1-related: Harmonization between JCTVC-T0065 Non CE1: Grouping Palette Indices At Front and CE1 Test A.1.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun1, PoLin Lei2, Jungsun Kim2, Tzu-Der Chuang1, Yi-Wen Chen1, Shan Liu2, Yu-Wen Huang1, Shawmin Lei1 1 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2 MediaTek San Jose: 2860 Junction Ave, San Jose, CA 95134, USA | Tel: Email: | Yu-Chen Sun +886-3-5670766 ext. 24572 {yc.sun, jungsun.kim, peter.chuang, yiwen.chen, shan.liu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents modifications to harmonize JCTVC-T0065 Grouping palette indices at front and CE1 Test A.1.5. The following are applied to achieve harmonized syntax design: 1. Grouping palette run_type flags at front (followed by the grouping of palette indices as in JCTVC-T0065). 2. run_type context does not depend on the position above. 3. Index adjustment does not depend on the above position. The results reported show that the BD-rate loss of the harmonization is about 0.0-0.2%.

# Introduction

In CE1 Test A.1.5 [1,2], for a parsed palette index after a COPY_ABOVE run, if its above position is directly copied from sample outside current CU, the parsed index will not be adjusted (such that the reconstruction of palette indices does not depends on the reconstruction of samples outside current CU).

In JCTVC-T0065 [3], palette indices are grouped at front, before palette run_type flags and palette runs. When combined with CE1 Test A.1.5, “the range of allowed index values is unknown until other syntax elements are parsed, and the binarization depends on knowledge of the range.”

# Proposed methods

To achieve harmonized syntax design, the following are applied to achieve harmonized syntax design:

- Grouping palette run_type flags at front (followed by the grouping of palette indices as in JCTVC-T0065).

Since run_type flags are grouped at front (without knowing the run length), the run_type of a given position is unknown when parsing run_type flags and parsing palette indices.

- run_type context cannot depend on above position: This is taken care of by applying the adoption of JCTVC-T0078 Part I [4], such that run_type context does not depend on the position above.

- Index adjustments cannot depend on the above position: No index adjustment that is conditioned based on the position above.

## JCTVC-T0036: CE1 Test A.1.5 with palette index redundancy removal for partial coded indices

In this simplification, we propose to directly use a NCP value to represent a current pixel instead of the derived index of the NCP. Therefore, the conversion process, Eq. (1), can be totally removed. Fig. 4 shows an example. To reconstruct the colour indices of the first row of the current CU, if the indices are encoded by a copy-above run, the decoder reconstructs them from the NCP values instead of the derived indices of NCPs. Note that the decoder does not derive indices of NCPs, so the palette index redundancy removal can be applied only for partial coded indices. If the reference index for a copy-index run mode is from an NCP, the redundant index removal will be disabled.

Fig.4.  Test A.1.5 method: copy pixel value

## JCTVC-T0065: Grouping Palette Indices at Front

With the proposed method of grouping palette indices at the front, the palette mode syntax table is updated as:

| ... |  |
| --- | --- |
| scanPos = 0 |  |
| indices_idc_coding() (Relocate all occurrence of palette_index_idc to here) |  |
| while( scanPos < nCbS * nCbS ) { |  |
| … |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE && adjustedIndexMax > 0 ) |  |
| palette_index_idc | ae(v) |
| … |  |

The function of indices_idc_coding() can be specified as:

| indices_idc_coding() { |  |
| --- | --- |
| num_palette_index | ae(v) |
| for (i = 0; i < num_palette_index; i++) |  |
| palette_index_idc | ae(v) |
| palette_last_run_type_flag | ae(v) |
| } |  |

num_palette_index specifies the number of palette index signalled in the current block. The difference between this value and indexMax Error: Reference source not found is coded into the bitstream using HEVC coefficient coding method, i.e. by calling the function of xWriteCoefRemainExGolomb() in HM with parameter equal to 2+indexMax/6. If negative value occures, they are mapped to positive values with equal spacing of 32 in non palette share mode, and 8 in palette share mode.

palette_last_run_type_flag specifies the value of the last occurence of palette_last_run_type_flag.

The proposed method is only enabled when the variable indexMax is greater than 0 because otherwise there is no necessarity of grouping. The coding of last paletteRun can be bypassed when the run type equals to palette_last_run_type_flag and the number of occurred palette_index_idc equals to num_palette_index.

Also, the coding of palette_run_type_flag can be bypassed when the number of occurred palette_index_idc equals to num_palette_index or the current position is the last one in the block.

## T0078 Part-1: Fixed Context for run_type coding

It is proposed to fix the context of run_type to 0 to avoid referring above line’s run_type every time. For example, the context index for run_type is fixed to 0. The spec is changed as follows:

| palette_run_type_flag | 0, 1 (subclause Error: Reference source not found) | na | Na | na | na | na |
| --- | --- | --- | --- | --- | --- | --- |

| Syntax element | condL | condA | ctxInc |
| --- | --- | --- | --- |
| split_cu_flag[ x0 ][ y0 ] | CtDepth[ xNbL ][ yNbL ] > cqtDepth | CtDepth[ xNbA ][ yNbA ] > cqtDepth | ( condL && availableL ) + ( condA && availableA ) |
| cu_skip_flag[ x0 ][ y0 ] | cu_skip_flag[ xNbL ][ yNbL ] | cu_skip_flag[ xNbA ][ yNbA ] | ( condL && availableL ) + ( condA && availableA ) |
| palette_transpose_flag [ x0 ][ y0 ] | palette_transpose_flag [ xNbL ][ yNbL ] | na | ( condL && availableL ) |
| palette_run_type_flag [ x0 ][ y0 ] | na | palette_run_type_flag [ xNbA ][ yNbA ] | ( condA && availableA ) |

## Harmonization of Test A.1.5, JCTVC-T0065, and JCTVC-T0078

In order to harmonize the Test A.1.5 and JCTVC-T0065, the palette_run_type_flag are moved in front of the grouped indices. When signalling palette_run_type_flag, we also integrate the adopted method in T0078 Method-1, where the context formation of palette_run_type_flag doesn’t depend on the palette_run_type_flag of the above sample. Therefore, in this combined software, the syntax of the run types, the palette indices, the run lengths, the escape pixels are grouped respectively. No interleaved syntax signaling is applied. The run types, which are all coded in context coded bins, are further grouped together to improve the CABAC decoding throughput. When decoding the index group, the previous run mode of each index is known. When the previous run mode is not copy index run mode, the redundant index removal is not applied. The conditional check in A.1.5 (checking whether the previous coded mode is copy above mode and the above sample is copied from above CU) can be simplified (only checking whether the previous coded mode is copy above mode).

| palette_coding( x0, y0, nCbS ) { | Descriptor |
| --- | --- |
| … |  |
| if( indexMax > 0) { |  |
| palette_transpose_flag | ae(v) |
| palette_num_indices |  |
| prev_palette_run_type_flag = COPY_INDEX_MODE |  |
| for( i = 0; i < palette_num_indices; i += (prev_palette_run_type_flag = = COPY_INDEX_MODE){ |  |
| if(prev_palette_run_type_flag = = COPY_INDEX_MODE) |  |
| palette_run_type_flag[ xC ][ yC ] | ae(v) |
| prev_palette_run_type_flag = palette_run_type_flag[ xC ][ yC ] |  |
| ) |  |
| } |  |
| for( i = 0; i < palette_num_indices; i++ ) | ae(v) |
| palette_index_idc |  |
| last_palette_run_type_flag | ae(v) |
| indicesLeft = palette_num_indices | ae(v) |
| } |  |
| scanPos = 0 |  |
| while( scanPos < nCbS * nCbS ) { |  |
| … |  |
| if( indexMax > 0 && scanPos > = nCbS && palette_run_type_flag[ xcPrev ][ ycPrev ] ! = COPY_ABOVE_MODE && indicesLeft && scanPos < nCbS * nCbS − 1) { |  |
| palette_run_type_flag[ xC ][ yC ] | ae(v) |
| } |  |
| if( palette_run_type_flag[ xC ][ yC ] = = COPY_INDEX_MODE && adjustedIndexMax > 0) |  |
| palette_index_idc | ae(v) |
| indicesLeft –= 1 |  |
| if( indexMax > 0 && (indicesLeft ¦ ¦ palette_run_type_flag != last_palette_run_type_flag)) { |  |
| maxPaletteRun = nCbS * nCbS – scanPos – 1 |  |
| … |  |
| } |  |
| } |  |

# Experimental results

The proposed method is integrated on SCM-3.0 [3]. All tests were conducted under the common test condition (CTC) [4] for SCC with the full picture IBC search range. Table 1 show the results of the proposed harmonization.

# Conclusions

The proposed method is to harmonize the adopted tools in JCTVC-T0036 CE1 Test-A.1.5, JCTVC-T0065, and JCTVC-T0078 method-1. The results reported show that the BD-rate loss of the harmonization is about 0.0-0.2%.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-C. Sun, J. Kim, S. Liu, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek), V. Seregin, F. Zou, W. Pu, R. Joshi, M. Karczewicz (Qualcomm), “CE1 : Tests A.1 : Extended copy above mode to the first line (1.1-1.5),” Document of Joint Collaborative Team on Video Coding, JCTVC-T0036, Feb. 2015.

- J. Kim, Y.-C. Sun, S. Liu, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei, V. Seregin, F. Zou, W. Pu, R. Joshi, M. Karczewicz, “CE1 : Tests A.1 : Extended copy above mode to the first line,” Document of Joint Collaborative Team on Video Coding, JCTVC-T0037, Feb. 2015.

- M. Karczewicz, W. Pu, R. Joshi, V. Seregin (Qualcomm), “Non CE1: Grouping Palette Indices At Front,” Document of Joint Collaborative Team on Video Coding, JCTVC-T0065, Feb. 2015.

- J. Kim, P. Lai, S. Liu, S. Lei (MediaTek), “CE1-related : simplification for index map coding in palette mode,” Document of Joint Collaborative Team on Video Coding, JCTVC-T0078, Feb. 2015.

Table 1.  Lossy results of the proposed method. 
Anchor: SCM-3.0. Test: Proposed harmonization method.