| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T1003 |
| --- | --- |

| Title: | Verification test plan for interlaced video and format range extensions |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne, V. Baroncini A. Tourapis, G. Barroux, M. Naccari | Email: | chris.rosewarne@cisra.canon.com.au vittorio@fub.it atourapis@apple.com guillaume.b@jp.fujitsu.com matteo.naccari@bbc.co.uk |
| Source: | JCT-VC |  |  |

_____________________________

# Abstract

This contribution provides the test plan for the HEVC interlace verification and the HEVC Range extensions verification tests. A set of test sequences and a set of encoding conditions are presented.

# Test conditions description

## General description

The HEVC and AVC performance will be compared by encoding selected sequences at selected bit-rates using software representing the typical applications of these two standards.

## Test material

The test sequences to be used for the HEVC interlace verification test are listed in Table 1-1 while the test sequences to be used for the HEVC Range extensions verification test are listed in Table 1-2.

Table 1-1 Test sequences to be used in the HEVC interlace verification testing

| Class | Test sequence |
| --- | --- |
| YCbCr 4:2:0i | CBS Tennis |
|  | CBS Basketball |
|  | ParkJoy_1920x1080_420_50i_8bit.yuv |
|  | NewMobCal_1920x1080_420_50i_10bits.yuv |
|  | MusicHD1 |

Table 1-2 Test sequences to be used in the HEVC Range Extension verification testing

| Class | Test sequence |
| --- | --- |
| YCbCr 4:2:2 | EBUHorse_1920x1080_50_10bit_422.yuv |
|  | EBUKidsSoccer_1920x1080_50_10bit_422.yuv |
|  | EBUWaterRocksClose_1920x1080_50_10bit_422.yuv |
| YCbCr 4:4:4 2 | BirdsInCage_1920x1080_60_10bit_444.yuv |
|  | CrowdRun_1920x1080_50_10bit_444.yuv |
|  | EBULupoCandlelight_1920x1080_50_10bit_444.yuv |

## Software to be used

### HEVC encoding

HM-16.2 software is used with the patch ‘HM-16.2-interlace.patch’ included in the directory comprising this document and which provides the following features:

- Set delta-QP for POC #1 setting using the ‘SecondFieldDeltaQP’ configuration option.

- increment GOP ‘depth’ based on POC and GOP size using the “SecondFieldBias” configuration option.

### AVC encoding

JM-18.6 software is used with no additional patch

## Bit-rates

### Interlace verification

- RA 4:2:0i 8-bit in HEVC: 1.5Mbps, 3Mbps, 5Mbps, 8Mbps (in AVC, double these bit-rates).

- LD: 4:2:0i 8-bit in HEVC: 3Mbps, 6Mbps, 10Mbps, 16Mbps (in AVC, double these bit-rates).

To match these bitrates, the quantization values as specified in Table 1-3 and Table 1-4 are used:

Table 1-3 QP values to use for interlace testing in RA mode

| Class | Test sequence | QP value used for HEVC | QP value used for AVC |
| --- | --- | --- | --- |
| YCbCr 4:2:0i | CBS Tennis | 23.9 | 21.4 |
|  |  | 25.8 | 23.7 |
|  |  | 28.5 | 25.1 |
|  |  | 32.6 | 29.5 |
|  | CBS Basketball | 27.6 | 25 |
|  |  | 30.1 | 28 |
|  |  | 33.4 | 31.7 |
|  |  | 38.3 | 36.5 |
|  | ParkJoy_1920x1080_420_50i_8bit.yuv | 31.7 | 30.1 |
|  |  | 34.2 | 32.6 |
|  |  | 36.8 | 34.7 |
|  |  | 40.3 | 37.4 |
|  | NewMobCal_1920x1080_420_50i_10bits.yuv | 22 | 19 |
|  |  | 23.5 | 21.6 |
|  |  | 25.7 | 23.3 |
|  |  | 29.6 | 26 |
|  | MusicHD1 | 26.9 | 24 |
|  |  | 29.8 | 27.4 |
|  |  | 32.9 | 30 |
|  |  | 37.8 | 35.8 |

Table 1-4 QP values to use for interlace testing in LD mode

| Class | Test sequence | QP value used for HEVC | QP value used for AVC |
| --- | --- | --- | --- |
| YCbCr 4:2:0i | CBS Tennis | 21.4 | 20 |
|  |  | 23.2 | 21.4 |
|  |  | 25.1 | 23.4 |
|  |  | 28.1 | 26 |
|  | CBS Basketball | 24.8 | 24.8 |
|  |  | 26.8 | 25.1 |
|  |  | 29.3 | 28 |
|  |  | 33.4 | 32.1 |
|  | ParkJoy_1920x1080_420_50i_8bit.yuv | 28.8 | 28.3 |
|  |  | 31.3 | 29.4 |
|  |  | 33.8 | 32.1 |
|  |  | 37.2 | 36.5 |
|  | NewMobCal_1920x1080_420_50i_10bits.yuv | 19.6 | 19.7 |
|  |  | 20.9 | 20 |
|  |  | 22.5 | 21.3 |
|  |  | 24.7 | 24 |
|  | MusicHD1 | 23.4 | 23 |
|  |  | 25.7 | 24.4 |
|  |  | 28.6 | 27.6 |
|  |  | 32.5 | 31.3 |

### Range extension verification

- RA 4:2:2 10-bit in HEVC: 5Mbps, 8Mbps, 12Mbps, 16Mbps1

- RA 4:4:4 10-bit in HEVC: Same bit-rates as used in RA 4:2:2.

To match these bitrates, the quantization values as specified in Table 1-5 and Table 1-6 are used:

Table 1-5 QP values to use for Range Extension testing with 4:2:2 content

| Class | Test sequence | QP value used for HEVC | QP value used for AVC |
| --- | --- | --- | --- |
| YCbCr 4:2:2 | EBUHorse_1920x1080_50_10bit_422.yuv | 23.6 | 20 |
|  |  | 24.6 | 21.1 |
|  |  | 26.3 | 23.1 |
|  |  | 28.7 | 25.1 |
|  | EBUKidsSoccer_1920x1080_50_10bit_422.yuv | 25.5 | 22 |
|  |  | 27 | 23.5 |
|  |  | 29 | 25.2 |
|  |  | 31.4 | 28.1 |
|  | EBUWaterRocksClose_1920x1080_50_10bit_422.yuv | 35.5 | 32.1 |
|  |  | 37.5 | 34 |
|  |  | 40.1 | 37.8 |
|  |  | 42.9 | 40 |

Table 1-6 QP values to use for Range Extension testing with 4:4:4 content

| Class | Test sequence | QP value used for HEVC | QP value used for AVC |
| --- | --- | --- | --- |
| YCbCr 4:4:4 | BirdsInCage_1920x1080_60_10bit_444.yuv | 22.9 | 19.8 |
|  |  | 23.6 | 20 |
|  |  | 24.5 | 21.5 |
|  |  | 25.5 | 22 |
|  | CrowdRun_1920x1080_50_10bit_444.yuv | 30.7 | 27.1 |
|  |  | 32.5 | 29.5 |
|  |  | 35.2 | 32.1 |
|  |  | 38.4 | 35 |
|  | EBULupoCandlelight_1920x1080_50_10bit_444.yuv | 17.2 | 14.4 |
|  |  | 18.1 | 15.1 |
|  |  | 19.7 | 17.2 |
|  |  | 21.6 | 18 |

Note:

1. As 4:2:2 and 4:4:4 is used in contribution, higher bit-rates are defined and a narrow spread of subjective scores is expected.

## Software configuration

Selected configurations:

- For RA, 1 second GOP selected.

- For LD, 1 intra-picture encoded.

- Note that the Sequence Adaptive Frame Field coding (‘SAFF’) tool (which can be applied to HM) can cause artefacts at switchover points. Moreover, current test material has no scene changes (except for MusicHD1 which has a fading scene change), so there is a low expectation of switching occurring in current test material. Thus, SAFF is not used in these tests.
Instead, field only mode will be used for the encoding of interlace sequences. Frame mode will be used for Range extension coding tests.

- When encoding interlace sequences, the JM software will usethe MBAFF and PicAff tools.

- Refer to the configuration files included in this contribution for full specification of the encoder settings.

Table 1-7 lists the configuration files to be used for producing bitstreams. Per-sequence settings to be overridden as required for each test sequence.

Table 1-7. Configuration files for JM and HM.

| Test condition | JM configuration file | HM configuration file |
| --- | --- | --- |
| Range extensions - Random access | encoder_JM_RA_B_HE.cfg | encoder_randomaccess_main_rext.cfg |
| Interlace - Random access | encoder_JM_RA_B_HE_Field.cfg | encoder_randomaccess_main_Field.cfg |
| Interlace - Low delay B | encoder_JM_LB_HE_Field.cfg | encoder_lowdelay_main_Field.cfg |

## Viewing procedure

The viewing will be done at FUB with about 24 naïve viewers. A report on the subjective test results is to be issued by Vittorio Baroncini.

### Test method

The evaluation will be done using the DSIS (Variant I) test method.

This method implies the adoption of a Basic Test Cell (BTC) where, at first, the uncompressed reference sample (SRC) of a video test sequence is shown, followed by the compressed video sequences (PVS).

The timing of a DSIS BTC is depicted below, where the message “A” announces the 10 second SRC video clips, followed by the message “B” announcing the compressed video sequence (PVS); then the message “Vote n” is presented on the screen for 5 seconds inviting the viewers to write their scores on the scoring sheets.

### Test sessions

The adoption of the DSIS test method leads to a total of 27” length for each BTC.

A test session is made of a set of BTC evaluating the PVS and a “stabilization phase”, made of three BTC selected from those actually included in the test session, including a low, a high and a mid-quality PVS. The insertion of the “stabilization phase” at the beginning of a test session allows the viewing subjects to have an immediate sensation of the whole range of quality they will see during the test session.

Furthermore in each test session at least two BTC where the SRC is compare with itself (SRC vs. SRC) are shown; this allows verifying the level of attention and in general the reliability of the viewing subjects.

The design of the test for the Interlaced Video verification will require the evaluation of 80 PVS; this will lead to two test sessions of 32 BTCs (length = 14’ 24”) and one test session of 31 BTCs (length = 13’ 57”)

The design of the test for the REXT verification will require the evaluation of 48 PVS; this will lead to two test sessions of 29 BTCs (length = 13’ 3”).

### Viewing equipment

A monitor capable of displaying up to 4:4:4 at 10-bits will be used.

### Deinterlacing process

For interlace testing, YUV sequences resulting from decoding bitstreams are to be de-interlaced using an ‘offline’ de-interlacer. The FFmpeg software [2] will be used for offline de-interlacing. Its ‘yadif’ de-interlacer will be called without the option ‘mcdeint’. The used argument will be '-vf yadif=1:0:0'

### Viewing subjects

The test will be done hiring 24 naïve viewing subjects selected trying to have an equal gender participation of university students aging between 18 and 30 years.

The viewing subjects will be screened for visual acuity and color blindness (Snellen chart and Ishihara table). A post screening of the subjects will be done analysing their score to exclude outliers.

### Test schedule

The five test sessions necessary to complete the evaluation will be scheduled across four working days according to the following schedule.

| Day 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09:00 | 10:00 | 10:30 | 11:00 | 11:30 | 12:00 | 12:30 | lunch | 13:30 | 14:00 | 14:30 | 15:00 |
| G1 | TR | S-01 |  | S-02 |  | S-03 |  |  | S-04 |  | S-05 |  |
| G2 |  |  | S-05 |  | S-04 |  | S-03 |  |  | S-02 |  | S-01 |
| Day 2 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 09:00 | 10:00 | 10:30 | 11:00 | 11:30 | 12:00 | 12:30 | lunch | 13:30 | 14:00 | 14:30 | 15:00 |
| G3 | TR | S-02 |  | S-03 |  | S-04 |  |  | S-05 |  | S-01 |  |
| G4 |  |  | S-04 |  | S-03 |  |  | S-02 |  | S-01 |  | S-05 |
| Day 3 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 09:00 | 10:00 | 10:30 | 11:00 | 11:30 | 12:00 | 12:30 | lunch | 13:30 | 14:00 | 14:30 | 15:00 |
| G5 | TR | S-03 |  | S-04 |  | S-05 |  |  | S-01 |  | S-02 |  |
| G6 |  |  | S-03 |  | S-02 |  | S-01 |  |  | S-05 |  | S-04 |
| Day 4 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 09:00 | 10:00 | 10:30 | 11:00 | 11:30 | 12:00 | 12:30 | lunch | 13:30 | 14:00 | 14:30 | 15:00 |
| G7 | TR | S-04 |  | S-05 |  | S-01 |  |  | S-02 |  | S-03 |  |
| G8 |  |  | S-02 |  | S-01 |  | S-05 |  |  | S-04 |  | S-03 |

G1 trough G8 represent the eight groups of 3 subjects each that alternate in front of the monitor being seated in central, left and right position.

The S01 to S05 are the five test sessions which will be run; while “TR” refers to the time dedicated to the training of the two groups of subjects that will be done all together at the beginning of each day.

- Test scope

- The test set is defined as:

  - 4:2:0 Interlace:

    - 4 bit-rates * 1 config (RA) * 5 (4:2:0 8-bit sequences) = 20 streams

    - 4 bit-rates * 1 config (LD) * 5 (4:2:0 8-bit sequences) = 20 streams

  - RExt:

    - 4 bit-rates * 1 config (RA) * 3 (4:2:2 10-bit sequences) = 12 streams

    - 4 bit-rates * 1 config (RA) * 3 (4:4:4 10-bit sequences) = 12 streams

  - Total: 64 streams

# References

- C. Rosewarne, A. Tourapis, G. Barroux, M. Naccari “Draft verification test plan for interlaced video and format range extensions” JCTVC-S1003, 19th JCT-VC Meeting, Strasbourg, FR, October 2014.

- “FFmpeg”, http://www.ffmpeg.org.