| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-T1102_r2 |
| --- | --- |

| Title: | Description of Core Experiment 2 (CE2): Intra block copy memory access |  |  |
| --- | --- | --- | --- |
| Status: | Output Document of JCT-VC |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Christophe Gisquet Jani Lainema Xiaozhong xu | krapaka@qti.qualcomm.com christophe.gisquet@crf.canon.fr jani.lainema@nokia.com xiaozhong.xu@mediatek.com |  |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides the description of Core Experiment 2 (CE2) on intra block copy memory access.

# Introduction

At the 20th JCT-VC meeting, the aspects related to intra block copy (IBC) memory bandwidth was discussed. A BoG activity was set-up during the meeting cycle to discuss and detail the test methods to evaluate the memory bandwidth reductions when IBC is enabled. It was asserted that IBC bandwidth increases over HEVC v1 inter mode due to the need store additional unfiltered samples that are used for IBC mode prediction. A general interest was shown to keep the memory bandwidth of IBC mode within the limits of bandwidth of HEVC v1 inter mode.

The purpose of this core experiment is to analyse memory accesses (memory capacity, bandwidth, etc.) and evaluate different aspects related to memory accesses of IBC mode.

# Participant list

| Company/Organization | Participant Name | Email Address |
| --- | --- | --- |
| Canon | Patrice Onno Guillaume Laroche Christophe Gisquet | Patrice.Onno@crf.canon.fr Guillaume.LAROCHE@crf.canon.fr Christophe.Gisquet@crf.canon.fr |
| Nokia | Jani Lainema | jani.lainema@nokia.com |
| Qualcomm | Krishna Rapaka Chao Pang Vadim Seregin Ted Hsieh | krapaka@qti.qualcomm.com cpang@qti.qualcomm.com vseregin@qti.qualcomm.com thsieh@qti.qualcomm.com |
| MediaTek | Xiaozhong Xu Polin Lai Tzu-Der Chuang Shan Liu Peter Chuang Yu-Wen Huang Yu-Chen Shih-Ta Hsiang | xiaozhong.xu@mediatek.com polin.lai@mediatek.com peter.chuang@mediatek.com shan.liu@mediatek.com peter.chuang@mediatek.com yuwen.huang@mediatek.comyc.sun@mediatek.com shih-ta.hsiang@mediatek.com |
| Microsoft | Bin Li Jizheng Xu | libin@microsoft.com jzxu@microsoft.com |
| Broadcom | Minhua Zhou | minhua@broadcom.com |
| Samsung | Elena Alshina Ankur Saxena Madhukar Budagavi | elena_a.alshina@samsung.com a.saxena1@samsung.com m.budagavi@samsung.com |
| Sharp | Kiran Misra Seung-Hwan Kim Tomohiro Ikai | misrak@sharplabs.com kimse@sharplabs.com ikai.tomohiro@sharp.co.jp |
| Ericsson | Andrey Norkin | andrey.norkin@ericsson.com |
| NCTU | Wen-Hsiao Peng Chun-Chi Chen Liao-Ru Ling, | wpeng@cs.nctu.edu.tw cheerchen.cs98g@g2.ncu.edu.tw jlliao.cs02g@nctu.edu.tw |
| Tongji University | Tao Lin | lintao@tongji.edu.cn |
| InterDigital | Xiaoyu Xiu Yuwen He | Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com |

# Software

The software to be used in this CE is SCM-4.0.

# Test conditions

The common conditions for screen content coding tests from JCTVC-T1015 shall be used for these experiments.

Additional test configuration with CTU size of 16x16 and same local search area size in terms of samples may be optionally provided.

Expert subjective analysis shall be performed for the methods that change filtering operations (Deblock/SAO) at higher QP’s (in the range of 37 including LD-P configuration).

# Tests

## Category A: Signalling of IBC source block

### Test 1: IBC source block signalled at CTU level (JCTVC-T0045/JCTVC-T0051/JCTVC-S0145)

This test evaluates the average memory bandwidth reduction when CTUs are marked as source CTUs for intra block copy operations.

## Category B: Filtering constraints to reduce memory bandwidth

### Test 2: Intra block copy constraints on filtering – Frame level encoding decision (JCTVC-T0051)

This test evaluates the signaling of CTUs which can be used as reference for IBC prediction. When this flag is true, the deblocking filter and SAO are disabled. The current CTU and the previous one are considered as available for IBC prediction. This implies a delay of 2 CTUs between the CTU reconstruction and the CTU filtering. The decision is based on analysing the block vector value of the Intra block copy PU after the encoding of the whole frame.

.

### Test 3: Intra block copy constraints on filtering – Single-pass encoding decisions (JCTVC-T0045)

This test evaluates effectiveness of local CTU level encoder decisions to define which CTUs are marked as source CTUs for intra block copy operations. The decision is based on analysing how large area of the CTU is coded in either intra block copy or in palette mode.

### Test 4: Disabling filtering when IBC enabled at sequence and adaptively at picture level (meeting discussion)

This test evaluates effectiveness disabling filtering at sequence level and picture level.

## Category C:  IBC prediction constraints to reduce memory bandwidth

### Test 5: Intra block copy constraints on Bi-prediction samples with use of local cache (JCTVC-R0188)

This test evaluates restricting the IBC prediction from bi-predicted samples. In addition this method combines other methods in this section (esp. signalling of the CTU flag based to specify whether there are Bi-predicted samples in the CTU and constraining IBC prediction for the CTU).

# Cross-check

|  | Proponent | Cross-checker |
| --- | --- | --- |
| Test 1 | Nokia/Canon/Qualcomm | xxx |
| Test 2 | Nokia | Qualcomm |
| Test 3 | Canon | Nokia |
| Test 4 | xxx |  |
| Test 5 | Qualcomm | Canon |

# Complexity analysis

Analysis on memory bandwidth impact of increase search area for IBC are to be provided:

- Both read and write memory access needs to be taken into account.

- The software that is used in [2] will be used for evaluating the memory bandwidth increase of IBC.

- All the tests shall provide both average and worst case bandwidth usage analysis as used in [2]

# Timeline

T1: Release of SCM-4.0, 3 weeks (US Pacific Time)

T2: Finalization of CE description, T2 = T1 + 1 week,

T3: Release of Memory bandwidth measurement module, T3 = T2 + 2 weeks

T4: Release of CE software packages and start crosschecking, T4 = T3 + 2 weeks,

T5: Release of simulation results, T5 = T4 + 1 weeks,

T6: Release of Memory analysis numbers T6 = T5 + 1 weeks,

T7: Release of text specifications, T7 = T4 + 2 weeks,

# References

[1] E. François, A. Tabatabai, and E. Alshina “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTVC-L0440, Geneva, CH, Jan. 2013.

[2] J. Sole, S. Liu “HEVC Screen Content Coding Core Experiment 1 (SCCE1): Intra Block Copying Extensions”, JCTVC-Q1121, Valencia, April. 2013.