| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0011 |
| --- | --- |

| Title: | JCT-VC AHG report: SHVC text editing (AHG11) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JCT-VC |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Jianle Chen Qualcomm Incorporated Jill Boyce Vidyo Yan Ye InterDigital Miska Hannuksela Nokia Corporation Gary J. Sullivan Microsoft Corp. Ye-Kui Wang Qualcomm Incorporated | Email: | cjianle@qti.qualcomm.com jill@vidyo.com Yan.Ye@InterDigital.com miska.hannuksela@nokia.com garysull@microsoft.com yekuiw@qti.qualcomm.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document reports the work of the JCT-VC ad hoc group on SHVC text editing (AHG11) between the 20th JCT-VC meeting in Geneva, Switzerland (10–18 Feb. 2015) and the 21th JCT-VC meeting in Warsaw, Poland (19–26 June 2015).

# Mandate

At the 20th meeting of the ITU-T/ISO/IEC Joint Collaborative Team on Video Coding (JCT-VC), AHG11 on SHVC text editing was established with the following mandates:

- Produce and finalize JCTVC-T1007 SHVC Test Model 9 (SHM 9) text.

- Coordinate with AHG12 on SHVC software development to address issues relating to mismatches between software and text.

# Summary of Activities

During this period, the editorial team worked to improve the ISO/IEC 23008-2 document (HEVC version 2 specification text) prior to its publication. The following refinements and bug fixes have been included:

- Fix: remove one inference rule of sub_layer_tier_flag, there are two inference rules for of sub_layer_tier_flag in current spec (this fix was already integrated into the published ITU-T HEVC version 2)

- Editorial fix: adding a missing "the" and removing an unnecessary and confusing “less than or” in the semantics of syntax element part_mode in 7.4.9.5 Coding unit semantics

- Fix: adding missing inference rule for the syntax element collocated_ref_idx

- Clarification fix: replacing "chroma residual samples" by "chroma prediction samples" in 8.5.3.3 Decoding process for inter prediction samples

- Fix: wrong the inputs (passing samples) of deblocking process invocation in 8.7.2.5.3 Decision process for luma block edges

- Fix: replacement of "buffering period" with "active parameter sets" and inserting a missing “access unit with" in an informative NOTE in D.3.3 (Picture timing SEI message semantics)

- Typo fix: fixing titles of clauses F.6, G.6, H.6 to correspond to titles in subclauses of clause 6

- Typo fix: missing changes from RExt specification in F.7.3.2.2.1 General sequence parameter set RBSP syntax

- Editorial fix: missing changes from RExt specification in F.3.7.6.1 General slice segment header syntax

- Minor typo fix in F.7.3.2.3.3 Picture parameter set multilayer extension syntax

- Fix: removal of "for each access unit" in the note on BaseLayerOutputFlag which is not access unit specific, in F.8.1.2 CVSG decoding process

- Fix: wrong index for the derivation of LayerInitializedFlag in F.8.1.3 Common decoding process for a coded picture

- Fix: changing " discardable_flag equal to 1" to " discardable_flag equal to 0" in one place in F.14.3.2.2 Buffering period SEI message semantics for multi-layer extensions

- Minor fix: wrong reference (F.8.5 -> G.8.4) in G.8.1.2 Decoding process for a coded picture with nuh_layer_id greater than 0

- Editorial fix: replacing “dependent layer” by "reference layer" in the Multiview Main and Scalable Main profile definition, since “dependent layer” is not a defined term

- Fix: Removed misleading redundant constraint expressions for SPS and PPS extension flag values in G.11.1.1 and H.11.1.1.

- Typo fix: index typo H.8.1.4.2 Resampling process of picture motion and mode parameters

- Typo fix: remove wrong brackets in equation H-85 and equation H-86

The editorial team did not work on the Scalable HEVC (SHVC) Test Model text during this period.

# Recommendations

The AHG recommends to:

- Use SHVC bug-tracker (https://hevc.hhi.fraunhofer.de/trac/shvc) to report issues related to SHVC Draft and Test Model text.

- Compare the SHVC documents with the SHVC software and resolve any discrepancies that may exist, in collaboration with the SHVC Software AHG.

- Continue to improve the quality of the SHVC test model document.