| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0021 |
| --- | --- |

| Title: | CE1: Summary report of CE on palette mode improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | PoLin Lai (Primary) Vadim Seregin Xiaoyu Xiu Christophe Gisquet | Email: | polin.lai@mediatek.com vseregin@qti.qualcomm.com xiaoyu.xiu@interdigital.com Christophe.Gisquet@crf.canon.fr |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the Core Experiment 1 (CE1) on palette mode improvement.

# Introduction

In the 20th JCT-VC meeting in Geneva, Core Experiment 1 (CE1) [1] was formed with six tests, to further investigate improvements of the existing palette mode in SCM-4.0. This contribution describes test conditions, test results, and comparisons.

# Test conditions

All tests were conducted under SCM-4.0 common test condition [2].

# Test descriptions and results

  - Test A.1 – Extended copy above mode to the first line with index adjustment bits

- Proponent: MediaTek and Qualcomm, JCTVC-U0061

- Crosschecker: ITRI International, JCTVC-U0121

- Description:

This test evaluates palette mode using the nearest row of the above CU or the nearest column of the left CU to predict the pixel values of the current CU. With palette indices grouped at front, index adjustment bits is introduced to signal whether it is an actual max index value when the parsed index is Max max index value - 1.

  - Test A.2 –Extended copy above mode to the first line with grouping the palette_run_type_flag in front

- Proponent: MediaTek, JCTVC-U0062

- Crosschecker: Qualcomm, JCTVC-U0147

- Description:

This test evaluates palette mode using the nearest row of the above CU or the nearest column of the left CU to predict the pixel values of the current CU. Syntax elements palette_run_type_flag are grouped and moved in front of the grouped palette indices. (The syntax of the palette run types, the palette indices, the palette run lengths, and the escape pixels are grouped respectively.) Palette index range adjustment does not depend on above position.

- Lossy, 444, full-frame IBC

| 444, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | -1.5% | -1.6% | -1.6% | -1.4% | -1.5% | -1.5% |
| RGB, mixed, 1440p & 1080p | -0.5% | -0.7% | -0.7% | -0.5% | -0.6% | -0.7% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -1.9% | -1.9% | -2.0% | -1.7% | -1.9% | -1.9% |
| YUV, mixed, 1440p & 1080p | -0.7% | -1.2% | -1.3% | -0.6% | -1.1% | -1.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% (cross-checker) |  |  | 134% |  |  |
| Dec Time[%] | 101% (cross-checker) |  |  | 130% |  |  |

| 444, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | -0.9% | -1.0% | -1.0% | -0.9% | -1.0% | -1.0% |
| RGB, mixed, 1440p & 1080p | -0.4% | -0.5% | -0.5% | -0.4% | -0.5% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| YUV, TGM, 1080p & 720p | -1.1% | -1.2% | -1.4% | -1.0% | -1.2% | -1.4% |
| YUV, mixed, 1440p & 1080p | -0.6% | -1.1% | -1.4% | -0.5% | -0.9% | -1.1% |
| YUV, Animation, 720p | -0.1% | -0.3% | -0.1% | -0.1% | -0.2% | -0.1% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% |
| Enc Time[%] | 99% (cross-checker) |  |  | 126% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 122% |  |  |

| 444, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | -0.9% | -0.8% | -0.8% | -0.8% | -0.8% | -0.9% |
| RGB, mixed, 1440p & 1080p | -0.2% | -0.4% | -0.3% | -0.2% | -0.5% | -0.2% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | -0.1% | 0.0% | -0.1% | 0.0% | -0.1% |
| YUV, TGM, 1080p & 720p | -1.0% | -1.1% | -1.1% | -0.9% | -1.0% | -1.1% |
| YUV, mixed, 1440p & 1080p | -0.2% | -0.7% | -1.1% | -0.1% | -0.6% | -0.8% |
| YUV, Animation, 720p | 0.0% | 0.4% | 0.3% | 0.0% | -0.4% | 0.1% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 99% (cross-checker) |  |  | 128% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 118% |  |  |

- Lossy, 444, 4x1-CTU IBC

| 444, 4x1-CTU IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | -1.8% | -1.8% | -1.9% | -1.7% | -1.8% | -1.8% |
| RGB, mixed, 1440p & 1080p | -0.6% | -0.8% | -0.9% | -0.6% | -0.8% | -0.8% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -2.2% | -2.2% | -2.4% | -2.1% | -2.0% | -2.2% |
| YUV, mixed, 1440p & 1080p | -0.9% | -1.4% | -1.4% | -0.8% | -1.3% | -1.3% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 106% (cross-checker) |  |  | 130% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 127% |  |  |

| 444, 4x1-CTU IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | -1.1% | -1.2% | -1.2% | -1.0% | -1.1% | -1.1% |
| RGB, mixed, 1440p & 1080p | -0.5% | -0.6% | -0.6% | -0.4% | -0.6% | -0.6% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -1.3% | -1.5% | -1.7% | -1.2% | -1.4% | -1.5% |
| YUV, mixed, 1440p & 1080p | -0.6% | -1.2% | -1.4% | -0.5% | -0.9% | -1.1% |
| YUV, Animation, 720p | 0.0% | 0.2% | 0.0% | 0.0% | -0.1% | 0.0% |
| YUV, cam-captured, 1080p | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.1% |
| Enc Time[%] | 106% (cross-checker) |  |  | 123% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 119% |  |  |

| 444, 4x1-CTU IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| RGB, RGM, 1080p & 720p | -0.9% | -1.0% | -0.9% | -0.8% | -1.0% | -0.9% |
| RGB, mixed, 1440p & 1080p | -0.2% | -0.3% | -0.4% | -0.3% | -0.4% | -0.4% |
| RGB, Animation, 720p | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| RGB, cam-captured, 1080p | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM, 1080p & 720p | -1.0% | -1.0% | -1.3% | -0.9% | -1.0% | -1.0% |
| YUV, mixed, 1440p & 1080p | -0.3% | -1.2% | -0.9% | -0.1% | -1.0% | -1.2% |
| YUV, Animation, 720p | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | 0.2% |
| YUV, cam-captured, 1080p | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 103% (cross-checker) |  |  | 130% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 121% |  |  |

- Lossy, 420, full-frame IBC

| 420, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| All Intra | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, TGM, 1080p & 720p | -0.5% | -0.3% | -0.4% | -0.5% | -0.4% | -0.5% |
| YUV, mixed, 1440p & 1080p | -0.4% | -0.6% | -0.5% | -0.3% | -0.6% | -0.5% |
| YUV, Animation, 720p & 768p | -0.1% | 0.1% | 0.2% | 0.0% | 0.1% | 0.3% |
| Enc Time[%] | 104% (cross-checker) |  |  | 140% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 129% |  |  |

| 420, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, TGM, 1080p & 720p | -0.4% | -0.2% | -0.2% | -0.3% | -0.1% | -0.2% |
| YUV, mixed, 1440p & 1080p | -0.2% | -0.5% | -0.6% | -0.1% | -0.4% | -0.3% |
| YUV, Animation, 720p & 768p | 0.0% | 0.2% | 0.0% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 102% (cross-checker) |  |  | 133% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 123% |  |  |

| 420, full-frame IBC | A.1 |  |  | A.2 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Low Delay B | G/Y | B/U | R/V | G/Y | B/U | R/V |
| YUV, TGM, 1080p & 720p | -0.3% | -0.5% | -0.7% | -0.2% | 0.1% | -0.4% |
| YUV, mixed, 1440p & 1080p | -0.2% | -0.8% | -0.3% | -0.3% | -1.1% | -0.3% |
| YUV, Animation, 720p & 768p | -0.1% | -0.5% | 0.3% | 0.0% | -0.2% | 0.5% |
| Enc Time[%] | 101% (cross-checker) |  |  | 140% |  |  |
| Dec Time[%] | 100% (cross-checker) |  |  | 130% |  |  |

  - Test B – Simplification for index map coding in palette mode

- Proponent: MediaTek, JCTVC-U0050

- Crosschecker: Sharp, JCTVC-U0070

- Description:

This test evaluates removing dependency introduced by index range adjustment. In order to apply the methods to the current palette index coding in SCM4.0, syntax reordering with “grouping palette run_type flag at front” is utilized in the tests. The following are presented:

Result 1:	Group run_type in front, no dependency from the above index
			(remove index range adjustment when previous pixel is COPY_ABOVE mode).

Result 2:	Group run_type in front, no dependency from the left index
			(remove index range adjustment when previous pixel is COPY_INDEX mode).

Results 3-1:	Without grouping run_type in front, completely remove index range adjustment.

Results 3-2:	Group run_type in front, completely remove index range adjustment.

Supplementary: Group run_type in front.

- Lossy, 444, full-frame IBC

- Lossy, 444, 4x1-CTU IBC

- Lossy, 420, full-frame IBC

  - Test C.1 – Improved palette run-length coding with palette reverse scan order

- Proponent: InterDigital and Qualcomm, JCTVC-U0101

- Crosschecker: Canon, JCTVC-U0059

- Description:

This test evaluates the palette reverse scan as proposed in JCTVC-T0119. The proposed method signals one additional flag at CU level. If the flag is equal to 1, the palette indices of the current CU are scanned in reverse scan order; otherwise, the palette indices of the current CU are scanned in original scan order. The following illustrate the reverse scan order when horizontal scan is applied to one palette-coded CU.

  - Test C.2 – Palette run-length coding with rotated index scans (scan from bottom-right)

- Proponent: MediaTek, JCTVC-U0088

- Crosschecker: InterDigital, JCTVC-U0108

- Description:

This test evaluates rotated palette index scanning order as reported in JCTVC-T0174. A CU-level flag is used to indicate whether to apply the rotation. Constraint the rotation on CU size is also tested.

Fig. 1 	(a) Horizontal scan	  (b) Reverse horizontal scan (C.1)    (c) Rotated horizontal scan (C.2)

Fig. 2 	(a) Vertical scan	    (b) Reverse vertical scan (C.1)	(c) Rotated vertical scan (C.2)

RD2: Using the RDO method of CE.1 Test C.1 released on Apr. 8, 2015

RD3: Using the RDO method of CE.1 Test C.1 released on May 30, 2015

- Lossy, 444, full-frame IBC

- Lossy, 444, 4x1-CTU IBC

- Lossy, 420, full-frame IBC

  - Test D – Modification of palette run coding

- Proponent: Qualcomm, JCTVC-U0122

- Crosschecker: MediaTek, JCTVC-U0117

- Description:

If the current pixel is the first pixel in the line, a flag is signalled into the bitstream, indicating whether it ends at the last pixel in certain line. If so, the number of whole lines (L) the current run spans is coded into the bitstream using coefficient coding function (xWriteCoefRemainExGolomb) with order 0. Instead of directly signalling L, a mapping process is used to map maximal feasible value to zero and shift L up by 1 if it is less than the maxima. Otherwise, it falls back to normal palette run length coding with an independent set of context models.

If the current pixel is not the first pixel in the line, run length L is coded into the bitstream using xWriteCoefRemainExGolomb with order 0. Instead of signalling L directly, if the current run ends at one line below the current line, a mapping procedure is used to reorder the candidate positions in the line according to their horizontal distance between themselves and the current pixel.

- Lossy, 444, full-frame IBC

- Lossy, 444, 4x1-CTU IBC

- Lossy, 420, full-frame IBC

- Palette-related documents

JCTVC-U0036, “On Zero Maximum Palette Size”

JCTVC-U0052, “On the palette escape pixel coding”

JCTVC-U0063, “CE1-related: Colour-plane-based escape pixel coding”

JCTVC-U0064, “CE1-related: Palette coding with inter-prediction”

JCTVC-U0066, “CE1-related: Row-based copy pixel from neighbouring CU”

JCTVC-U0084, “On palette predictor initialization for Screen Content Coding”

- (Cross-check) JCTVC-U0130, “Crosscheck for palette predictor initialization for Screen Content Coding (JCTVC-U0084)”

JCTVC-U0086, “CE1-related: Simplification on coding NumPaletteIndices”

- (Cross-check) JCTVC-U0131, “Crosscheck for simplification on coding numPaletteIndices (JCTVC-U0086)”

JCTVC-U0087, “Syntax Cleanup for Palette Mode”

JCTVC-U0089, “Constraints on palette syntax elements”

JCTVC-U0090, “CE1-related: Palette Mode Context and Codeword Simplification”

- (Cross-check) JCTVC-U0126, “Crosscheck report of JCTVC-U0090”

JCTVC-U0091, “CE1-related: On the mismatch of SCM4.0 and HEVC SCC working draft”

- (Cross-check) JCTVC-U0124, “Crosscheck of JCTVC-U0091, CE1-related: On the mismatch of SCM4.0 and HEVC SCC working draft”

JCTVC-U0092, “Draft Text modifications on palette”

JCTVC-U0093, “Non-CE1: On Number of Palette Indices coding”

- (Cross-check) JCTVC-U0125, “Cross-verfication of JCTVC-U0093: Non-CE1: On Number of Palette Indices Coding”

JCTVC-U0094, “Non-CE1: Palette Run Hiding”

JCTVC-U0096, “CE1 Related: Improved palette encoder”

- (Cross-check) JCTVC-U0145, “Cross-verification of JCTVC-U0096: Improved palette encoder”

JCTVC-U0097, “Non-CE1: On maximum palette predictor size”

- (Cross-check) JCTVC-U0143, “Crosscheck report of JCTVC-U0097”

JCTVC-U0099, “Modified method for sending number of palette indices”

- (Cross-check) JCTVC-U0144, “Cross-verification of JCTVC-U0099: Modified method for sending number of palette indices”

JCTVC-U0105, “Non-CE1: Simplification on deriving NumPaletteIndices for palette mode”

JCTVC-U0109, “Non-CE1: Simplification for parsing the index of the most significant bit in the paletteRun binarization”

- (Cross-check) JCTVC-U0136, “Cross-checking of JCTVC-U0109: Non-CE1: Simplification for parsing the index of the most significant bit in the paletteRun binarization”

JCTVC-U0110, “Bugfixes and cleanup for the text of palette mode in HEVC SCC draft”

JCTVC-U0114, “Generate the palette predictor initializers with low delay”

- (Cross-check) JCTVC-U0141, “Crosscheck of Generate the palette predictor initializers with low delay (JCTVC-U0114)”

JCTVC-U0116, “Non-CE1: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding”

JCTVC-U0127, “CE1: Additional results for CE1 subtest C”

JCTVC-U0133, “Comment on signalling the palette_transpose_flag after last_palette_run_type_flag (JCTVC-U0090)”

JCTVC-U0146, “CE1 Related: Combination of JCTVC-U0101 reverse scan and JCTVC-U0096 palette encoder improvement”

JCTVC-U0148, “Non-CE1: Context model unification for palette run type flags in HEVC SCC”

JCTVC-U0149, “Non-CE1: Index Map Splitting (IMS) Mode for Palette Coding in HEVC SCC”

JCTVC-U0151, “Cross-check report of U0116: Enhancement to Palette Coding by Palette with Pixel Copy (PPC) Coding”

- References

- P. Lai, V. Seregin, X. Xiu, and C. Gisquet (CE coordinators), “Description of Core Experiment 1 (CE1): Palette mode improvement,” Document of Joint Collaborative Team on Video Coding, JCTVC-T1101, Feb. 2015.

- H. Yu, R. Cohen, K. Rapaka, and J. Xu (editors), “Common Test Conditions for Screen Content Coding,” Document of Joint Collaborative Team on Video Coding, JCTVC-T1015, Feb. 2015.