| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 20th Meeting: Geneva, CH, 10–18 Feb. 2015 | Document: JCTVC-U0022 |
| --- | --- |

| Title: | CE2: Summary report for Core Experiment 2 on Intra block copy memory access |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Krishna Rapaka Christophe Gisquet Jani Lainema Xiaozhong xu |  | krapaka@qti.qualcomm.com christophe.gisquet@crf.canon.fr jani.lainema@nokia.com xiaozhong.xu@mediatek.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document summarizes the activities in Core Experiment 2 on Intra block copy memory access. Various coding tools targeting towards reduction of memory access were evaluated. The evaluated tools and the test conditions are described in document JCTVC-T1102. Additionally memory access analysis is evaluated as defined in JCTVC-L0440

# Introduction

At the 20th JCT-VC meeting, the aspects related to intra block copy (IBC) memory bandwidth was discussed. A BoG activity was set-up during the meeting cycle to discuss and detail the test methods to evaluate the memory bandwidth reductions when IBC is enabled. It was asserted that IBC bandwidth increases over HEVC v1 inter mode due to the need to store additional unfiltered samples that are used to reconstruct predicted samples for IBC mode.

This core experiment was therefore established to further evaluate the performance and complexity of different aspects with regard to IBC and inter harmonization.

# List of participants

| Company/Organization | Participant Name | Email Address |
| --- | --- | --- |
| Canon | Patrice Onno Guillaume Laroche Christophe Gisquet | Patrice.Onno@crf.canon.fr Guillaume.LAROCHE@crf.canon.fr Christophe.Gisquet@crf.canon.fr |
| Nokia | Jani Lainema | jani.lainema@nokia.com |
| Qualcomm | Krishna Rapaka Chao Pang Vadim Seregin Ted Hsieh | krapaka@qti.qualcomm.com cpang@qti.qualcomm.com vseregin@qti.qualcomm.com thsieh@qti.qualcomm.com |
| MediaTek | Xiaozhong Xu Polin Lai Tzu-Der Chuang Shan Liu Peter Chuang Yu-Wen Huang Yu-Chen Shih-Ta Hsiang | xiaozhong.xu@mediatek.com polin.lai@mediatek.com peter.chuang@mediatek.com shan.liu@mediatek.com peter.chuang@mediatek.com yuwen.huang@mediatek.comyc.sun@mediatek.com shih-ta.hsiang@mediatek.com |
| Microsoft | Bin Li Jizheng Xu | libin@microsoft.com jzxu@microsoft.com |
| Broadcom | Minhua Zhou | minhua@broadcom.com |
| Samsung | Elena Alshina Ankur Saxena Madhukar Budagavi | elena_a.alshina@samsung.com a.saxena1@samsung.com m.budagavi@samsung.com |
| Sharp | Kiran Misra Seung-Hwan Kim Tomohiro Ikai | misrak@sharplabs.com kimse@sharplabs.com ikai.tomohiro@sharp.co.jp |
| Ericsson | Andrey Norkin | andrey.norkin@ericsson.com |
| NCTU | Wen-Hsiao Peng Chun-Chi Chen Liao-Ru Ling, | wpeng@cs.nctu.edu.tw cheerchen.cs98g@g2.ncu.edu.tw jlliao.cs02g@nctu.edu.tw |
| Tongji University | Tao Lin | lintao@tongji.edu.cn |
| InterDigital | Xiaoyu Xiu Yuwen He | Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com |

# List of CE contributions

| Doc No. | Title | Participants | Type |
| --- | --- | --- | --- |
| JCTVC-U0034 | CE2: Test 3 on intra block copy constraints on filtering (single-pass encoding decisions) | J. Lainema, M. M. Hannuksela (Nokia) | Proposal |
| JCTVC-U0053 | CE2 Test 2: Intra block copy constraints on filtering | G. Laroche, G. Malard, C. Gisquet, P. Onno (Canon) | Proposal |
| JCTVC-U0078 | CE2 : Test 5 on intra block copy constraints on prediction | K. Rapaka, V. Seregin, R. Joshi, T. Hsieh, M. Karczewicz (Qualcomm) | Proposal |
| JCTVC-U0035 | CE2: Crosscheck of Test 2 on intra block copy constraints on filtering (frame level encoding decisions) | Nokia | Crosscheck |
| JCTVC-U0060 | CE2: Cross check of Test 5 on Intra block copy constraints on Bi-prediction samples with use of local cache | Canon | Crosscheck |
| JCTVC-U0082 | CE2: cross check of CE2 Test 1: Intra block copy constraints on filtering | Qualcomm | Crosscheck |

# Summary of CE proposals

## Filtering constraints to reduce memory bandwidth

### Test 2: Intra block copy constraints on filtering – Frame level encoding decision (JCTVC-U0053)

This test evaluates the signaling of CTUs which can be used as reference for IBC prediction. When this flag is true, the deblocking filter and SAO are disabled. The current CTU and the previous one are considered as available for IBC prediction. This implies a delay of 2 CTUs between the CTU reconstruction and the CTU filtering. The decision is based on analysing the block vector value of the Intra block copy PU after the encoding of the whole frame.

.

### Test 3: Intra block copy constraints on filtering – Single-pass encoding decisions (JCTVC-U0034)

This test evaluates effectiveness of local CTU level encoder decisions to define which CTUs are marked as source CTUs for intra block copy operations. The decision is based on analysing how large area of the CTU is coded in either intra block copy or in palette mode.

## Test 5: Prediction constraints to reduce memory bandwidth (JCTVC-U0078)

### Test 5.1: Intra block copy constraints on Bi-prediction CTUs

This test evaluates restriction of intra block copy from predicting from Bi-Prediction CTU’s.

### Test 5.2: Bi-prediction Restriction when IBC is enabled

This test evaluates the coding performance of restricting 8x8 bi-prediction when intra block copy is enabled.

### Test 5.3: Intra block copy constraints on Bi-prediction CTUs

This test evaluates restriction of intra block copy from predicting from Bi-Prediction CTU’s when use_integer_mv flag is not set to 1 for the current slice.

### Test 5.4: Bi-prediction Restriction when IBC is enabled

This test evaluates the coding performance of restricting 8x8 bi-prediction when intra block copy is enabled when use_integer_mv flag is not set to 1 for the current slice.

# Summary of CE test results

The proposed methods were implemented on top of SCM-4.0 [2] and tested under the SCC common test conditions defined in JCTVC-S1015 [3].

## 444 lossy results - Full frame IBC CTC

The 444 lossy results are summarized as follows:

|  | 444 Lossy All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 3 | Test 2 | Test 5.1 | Test 5.2 | Test 5.3 | Test 5.4 |
| RGB, TGM | 1.3% | 0.6% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, MC | 2.1% | 1.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Ani. | 0.5% | 0.3% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, CC | 0.1% | 0.3% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM | 1.7% | 1.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, MC | 2.3% | 1.3% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Ani. | 1.0% | 0.7% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, CC | 0.1% | 0.5% | 0.0% | 0.0% | 0.0% | 0.0% |

|  | 444 Lossy Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 3 | Test 2 | Test 5.1 | Test 53.2 | Test 3.3 | Test 3.4 |
| RGB, TGM | 2.6% | 0.7% | 3.2% | 0.4% | 0.4% | 2.1% |
| RGB, MC | 1.2% | 0.5% | 2.0% | 0.6% | 0.6% | 2.0% |
| RGB, Ani. | 0.1% | 0.1% | 0.0% | 1.1% | 1.1% | 0.0% |
| RGB, CC | 0.1% | 0.1% | 0.0% | 0.8% | 0.8% | 0.0% |
| YUV, TGM | 3.0% | 1.1% | 2.8% | 0.3% | 0.3% | 1.8% |
| YUV, MC | 1.4% | 0.8% | 2.0% | 0.4% | 0.4% | 2.0% |
| YUV, Ani. | 0.4% | 0.2% | 0.0% | 0.7% | 0.7% | 0.0% |
| YUV, CC | 0.1% | 0.2% | 0.0% | 0.8% | 0.8% | 0.0% |

|  | 444 Lossy Low-delay B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| RGB, TGM | 3.8% | 0.9% | 4.8% | 0.4% | 0.3% | 2.5% |
| RGB, MC | 1.7% | 0.4% | 2.8% | 1.1% | 1.0% | 2.6% |
| RGB, Ani. | 0.0% | 0.0% | -0.1% | 1.0% | 1.0% | -0.1% |
| RGB, CC | 0.0% | 0.0% | 0.0% | 0.8% | 0.8% | 0.0% |
| YUV, TGM | 3.8% | 1.2% | 4.5% | 0.4% | 0.3% | 2.3% |
| YUV, MC | 2.0% | 0.8% | 2.7% | 0.7% | 0.7% | 2.6% |
| YUV, Ani. | 0.2% | 0.1% | -0.1% | 0.6% | 0.6% | -0.1% |
| YUV, CC | 0.0% | 0.0% | 0.0% | 0.4% | 0.4% | 0.0% |

## 444 lossless results - Full frame IBC CTC

The 444 lossless results are summarized as follows:

|  | 444 Lossless All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| RGB, TGM | 0.1% | NA | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, MC | 0.2% | NA | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Ani. | 0.0% | NA | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, CC | 0.0% | NA | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM | 0.2% | NA | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, MC | 0.7% | NA | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Ani. | 0.0% | NA | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, CC | 0.0% | NA | 0.0% | 0.0% | 0.0% | 0.0% |

|  | 444 Lossless Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| RGB, TGM | 0.5% | NA | 8.2% | 0.6% | 0.6% | 8.2% |
| RGB, MC | 0.3% | NA | 0.6% | 0.3% | 0.3% | 0.6% |
| RGB, Ani. | 0.0% | NA | 0.0% | 0.5% | 0.5% | 0.0% |
| RGB, CC | 0.0% | NA | 0.0% | 0.1% | 0.1% | 0.0% |
| YUV, TGM | 0.6% | NA | 8.9% | 0.7% | 0.7% | 8.8% |
| YUV, MC | 0.4% | NA | 0.6% | 0.4% | 0.4% | 0.6% |
| YUV, Ani. | 0.0% | NA | 0.0% | 0.7% | 0.7% | 0.0% |
| YUV, CC | 0.0% | NA | 0.0% | 0.3% | 0.3% | 0.0% |

|  | 444 Lossless Low-delay B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| RGB, TGM | 0.4% | NA | 8.2% | 0.2% | 0.2% | 8.1% |
| RGB, MC | 0.4% | NA | 0.9% | 0.3% | 0.3% | 0.9% |
| RGB, Ani. | 0.0% | NA | 0.0% | 0.4% | 0.4% | 0.0% |
| RGB, CC | 0.0% | NA | 0.0% | 0.2% | 0.2% | 0.0% |
| YUV, TGM | 0.6% | NA | 9.1% | 0.3% | 0.3% | 9.1% |
| YUV, MC | 0.5% | NA | 0.9% | 0.4% | 0.4% | 0.9% |
| YUV, Ani. | 0.0% | NA | 0.0% | 0.5% | 0.5% | 0.0% |
| YUV, CC | 0.0% | NA | 0.0% | 0.4% | 0.4% | 0.0% |

## 420 lossy results - Full frame IBC CTC

The 420 lossy results are summarized as follows:

|  | 420 Lossy All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| TGM, 720p | 2.4% | 1.3% | 0.0% | 0.0% | 0.0% | 0.0% |
| MC, 480p | 2.4% | 1.4% | 0.0% | 0.0% | 0.0% | 0.0% |
| Ani., 768p | 1.0% | 0.6% | 0.0% | 0.0% | 0.0% | 0.0% |

|  | 420 Lossy Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| TGM, 720p | 3.4% | 1.4% | 2.6% | 0.3% | 0.3% | 1.7% |
| MC, 480p | 1.8% | 1.0% | 2.0% | 0.3% | 0.3% | 2.0% |
| Ani., 768p | 0.6% | 0.5% | 0.1% | 0.4% | 0.4% | 0.1% |

|  | 420 Lossy Low-delay B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| TGM, 720p | 4.4% | 1.6% | 3.9% | 0.4% | 0.3% | 2.2% |
| MC, 480p | 2.1% | 0.8% | 2.4% | 0.4% | 0.4% | 2.2% |
| Ani., 768p | 0.3% | 0.3% | 0.1% | 0.4% | 0.4% | 0.1% |

## 420 lossless results -Full frame IBC CTC

The 420 lossless results are summarized as follows:

|  | 420 Lossless All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| TGM, 720p | 0.2% | NA | 0.0% | 0.0% | 0.0% | 0.0% |
| MC, 480p | 1.8% | NA | 0.0% | 0.0% | 0.0% | 0.0% |
| Ani., 768p | 0.1% | NA | 0.0% | 0.0% | 0.0% | 0.0% |

|  | 420 Lossless Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| TGM, 720p | 0.7% | NA | 6.4% | 0.5% | 0.5% | 6.3% |
| MC, 480p | 0.6% | NA | 0.7% | 0.4% | 0.4% | 0.7% |
| Ani., 768p | 0.0% | NA | 0.0% | 0.5% | 0.5% | 0.0% |

|  | 420 Lossless Low-delay B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| TGM, 720p | 0.7% | NA | 6.4% | 0.2% | 0.1% | 6.4% |
| MC, 480p | 0.6% | NA | 1.0% | 0.4% | 0.4% | 1.0% |
| Ani., 768p | 0.0% | NA | 0.0% | 0.4% | 0.4% | 0.0% |

## 444 lossy results – 1x4 CTC

The 444 lossy results are summarized as follows:

|  | 444 Lossy All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| RGB, TGM | 0.8% | 0.4% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, MC | 1.4% | 0.8% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, Ani. | 0.4% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| RGB, CC | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, TGM | 1.3% | 0.8% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, MC | 1.8% | 1.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, Ani. | 0.9% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| YUV, CC | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |

|  | 444 Lossy Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| RGB, TGM | 1.4% | 0.4% | 1.0% | 0.5% | 0.5% | 0.5% |
| RGB, MC | 0.6% | 0.3% | 0.4% | 0.6% | 0.6% | 0.4% |
| RGB, Ani. | 0.1% | 0.0% | 0.0% | 1.0% | 1.0% | 0.0% |
| RGB, CC | 0.1% | 0.1% | 0.0% | 0.8% | 0.8% | 0.0% |
| YUV, TGM | 1.8% | 0.7% | 0.8% | 0.3% | 0.3% | 0.5% |
| YUV, MC | 1.1% | 0.6% | 0.5% | 0.4% | 0.4% | 0.5% |
| YUV, Ani. | 0.4% | 0.1% | 0.0% | 0.8% | 0.8% | 0.0% |
| YUV, CC | 0.1% | 0.1% | 0.0% | 0.8% | 0.8% | 0.0% |

|  | 444 Lossy Low-delay B |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tests | Test 1 | Test 2 | Test 3.1 | Test 3.2 | Test 3.3 | Test 3.4 |
| RGB, TGM | 1.9% | 0.6% | 1.6% | 0.5% | 0.5% | 0.8% |
| RGB, MC | 0.9% | 0.5% | 0.6% | 1.1% | 1.1% | 0.5% |
| RGB, Ani. | 0.1% | 0.0% | 0.0% | 1.0% | 1.0% | 0.0% |
| RGB, CC | 0.0% | 0.0% | 0.0% | 0.8% | 0.8% | 0.0% |
| YUV, TGM | 2.1% | 0.8% | 1.4% | 0.4% | 0.4% | 0.6% |
| YUV, MC | 1.0% | 0.5% | 0.5% | 0.6% | 0.6% | 0.5% |
| YUV, Ani. | 0.2% | 0.1% | 0.0% | 0.8% | 0.8% | 0.0% |
| YUV, CC | 0.1% | 0.0% | 0.1% | 0.5% | 0.5% | 0.1% |

# Complexity analysis

The proposed methods were implemented on top of SCM-4.0 [2] and tested under the condition defined in JCTVC-L0440 [4].

In the table below, Average memory BW refers to average memory read and write access averaged across all frames under SCC CTC [3].

Maximum memory BW refers to maximum memory read and write access for a frame among all frames under SCC CTC [3].

It was also observed that for a particular memory pattern (4x2), the additional worst case increase to memory BW due to storing of IBC unfiltered samples is about 10%.

## Test 1

|  |  |  | Average |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| Lossy | 444 | ai | 74% | 74% | 74% | 74% | 73% | 72% | 99% | 99% |
|  |  | ra | 98% | 98% | 98% | 98% | 98% | 97% | 100% | 100% |
|  |  | lb | 98% | 98% | 98% | 98% | 98% | 97% | 100% | 100% |
|  | 444/4x1 | ai | 75% | 75% | 74% | 75% | 73% | 72% | 100% | 100% |
|  |  | ra | 98% | 98% | 98% | 98% | 98% | 97% | 100% | 100% |
|  |  | lb | 98% | 98% | 98% | 98% | 98% | 97% | 100% | 100% |
|  | 420 | ai | 71% | 71% | 70% | 71% | 70% | 69% | 99% | 98% |
|  |  | ra | 98% | 98% | 98% | 98% | 97% | 96% | 99% | 99% |
|  |  | lb | 98% | 99% | 98% | 98% | 97% | 96% | 100% | 100% |
| Lossless | 444 | ai | 74% | 74% | 75% | 75% | 76% | 77% | 99% | 99% |
|  |  | ra | 97% | 97% | 97% | 97% | 96% | 95% | 100% | 100% |
|  |  | lb | 97% | 97% | 97% | 97% | 96% | 95% | 100% | 100% |
|  | 420 | ai | 71% | 71% | 72% | 73% | 73% | 72% | 98% | 98% |
|  |  | ra | 96% | 97% | 95% | 96% | 94% | 92% | 100% | 100% |
|  |  | lb | 97% | 97% | 96% | 96% | 94% | 92% | 100% | 100% |

|  |  |  | Maximum |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| Lossy | 444 | ai | 75% | 75% | 74% | 74% | 74% | 74% | 98% | 98% |
|  |  | ra | 100% | 99% | 99% | 99% | 89% | 83% | 104% | 101% |
|  |  | lb | 100% | 100% | 100% | 100% | 91% | 97% | 96% | 96% |
|  | 444/4x1 | ai | 78% | 78% | 75% | 74% | 74% | 74% | 100% | 99% |
|  |  | ra | 100% | 100% | 99% | 98% | 91% | 84% | 101% | 100% |
|  |  | lb | 100% | 99% | 98% | 99% | 93% | 97% | 100% | 99% |
|  | 420 | ai | 76% | 75% | 71% | 71% | 71% | 71% | 100% | 100% |
|  |  | ra | 101% | 101% | 99% | 97% | 86% | 82% | 104% | 103% |
|  |  | lb | 100% | 99% | 98% | 97% | 88% | 84% | 100% | 99% |
| Lossless | 444 | ai | 74% | 74% | 75% | 75% | 77% | 78% | 99% | 99% |
|  |  | ra | 100% | 100% | 95% | 95% | 88% | 86% | 100% | 100% |
|  |  | lb | 96% | 95% | 94% | 94% | 87% | 86% | 99% | 99% |
|  | 420 | ai | 75% | 76% | 78% | 79% | 80% | 80% | 101% | 100% |
|  |  | ra | 100% | 97% | 92% | 92% | 87% | 87% | 100% | 99% |
|  |  | lb | 94% | 93% | 92% | 91% | 88% | 87% | 99% | 98% |

## Test 2

|  |  |  | Average |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| Lossy | 444 | ai | 81% | 81% | 82% | 82% | 83% | 84% | 100% | 100% |
|  |  | ra | 74% | 75% | 75% | 76% | 76% | 78% | 100% | 100% |
|  |  | lb | 72% | 72% | 73% | 73% | 74% | 76% | 100% | 100% |
|  | 444/4x1 | ai | 72% | 72% | 73% | 73% | 74% | 75% | 100% | 100% |
|  |  | ra | 74% | 74% | 75% | 75% | 75% | 76% | 100% | 100% |
|  |  | lb | 71% | 71% | 72% | 72% | 73% | 74% | 100% | 100% |
|  | 420 | ai | 85% | 85% | 86% | 86% | 87% | 88% | 100% | 100% |
|  |  | ra | 74% | 75% | 75% | 75% | 77% | 78% | 100% | 100% |
|  |  | lb | 71% | 72% | 73% | 73% | 74% | 76% | 100% | 100% |
| Lossless | 444 | ai | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |
|  |  | ra | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |
|  |  | lb | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |
|  | 420 | ai | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |
|  |  | ra | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |
|  |  | lb | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |

|  |  |  | Maximum |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| Lossy | 444 | ai | 88% | 87% | 87% | 87% | 86% | 85% | 100% | 100% |
|  |  | ra | 77% | 79% | 85% | 86% | 88% | 86% | 102% | 101% |
|  |  | lb | 77% | 79% | 83% | 84% | 87% | 88% | 97% | 96% |
|  | 444/4x1 | ai | 79% | 79% | 79% | 79% | 79% | 78% | 100% | 100% |
|  |  | ra | 76% | 78% | 81% | 81% | 81% | 81% | 100% | 101% |
|  |  | lb | 76% | 77% | 79% | 80% | 82% | 81% | 100% | 100% |
|  | 420 | ai | 88% | 88% | 88% | 88% | 88% | 88% | 100% | 100% |
|  |  | ra | 80% | 82% | 86% | 87% | 88% | 88% | 101% | 101% |
|  |  | lb | 78% | 80% | 86% | 87% | 88% | 89% | 97% | 98% |
| Lossless | 444 | ai | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |
|  |  | ra | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |
|  |  | lb | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |
|  | 420 | ai | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |
|  |  | ra | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |
|  |  | lb | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |

## Test 3.1

|  |  |  | Average |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| Lossy | 444 | ai | 64% | 64% | 64% | 64% | 64% | 65% | 100% | 100% |
|  |  | ra | 75% | 76% | 76% | 76% | 76% | 76% | 100% | 99% |
|  |  | lb | 73% | 74% | 74% | 74% | 75% | 75% | 99% | 99% |
|  | 444/4x1 | ai | 62% | 62% | 62% | 62% | 63% | 63% | 100% | 100% |
|  |  | ra | 75% | 75% | 76% | 76% | 76% | 76% | 100% | 100% |
|  |  | lb | 73% | 74% | 74% | 74% | 75% | 75% | 100% | 100% |
|  | 420 | ai | 61% | 61% | 61% | 62% | 62% | 62% | 100% | 100% |
|  |  | ra | 75% | 75% | 75% | 76% | 75% | 75% | 100% | 99% |
|  |  | lb | 73% | 74% | 74% | 74% | 74% | 74% | 99% | 99% |
| Lossless | 444 | ai | 76% | 76% | 76% | 76% | 75% | 75% | 100% | 100% |
|  |  | ra | 77% | 77% | 78% | 78% | 77% | 76% | 99% | 99% |
|  |  | lb | 76% | 76% | 77% | 77% | 76% | 76% | 99% | 99% |
|  | 420 | ai | 76% | 76% | 76% | 76% | 76% | 76% | 100% | 100% |
|  |  | ra | 75% | 75% | 75% | 75% | 74% | 74% | 99% | 98% |
|  |  | lb | 74% | 75% | 75% | 75% | 75% | 75% | 99% | 98% |

|  |  |  | Maximum |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| Lossy | 444 | ai | 100% | 99% | 99% | 99% | 98% | 96% | 100% | 100% |
|  |  | ra | 100% | 97% | 91% | 90% | 92% | 92% | 103% | 101% |
|  |  | lb | 90% | 87% | 81% | 80% | 76% | 81% | 96% | 94% |
|  | 444/4x1 | ai | 99% | 99% | 100% | 99% | 100% | 99% | 100% | 100% |
|  |  | ra | 100% | 98% | 91% | 89% | 90% | 92% | 99% | 98% |
|  |  | lb | 88% | 84% | 80% | 79% | 77% | 80% | 98% | 97% |
|  | 420 | ai | 99% | 99% | 99% | 99% | 99% | 99% | 100% | 100% |
|  |  | ra | 98% | 93% | 87% | 87% | 92% | 91% | 103% | 102% |
|  |  | lb | 90% | 88% | 84% | 82% | 83% | 89% | 103% | 102% |
| Lossless | 444 | ai | 99% | 99% | 99% | 99% | 99% | 99% | 100% | 100% |
|  |  | ra | 100% | 100% | 93% | 92% | 99% | 101% | 99% | 98% |
|  |  | lb | 100% | 100% | 99% | 99% | 100% | 100% | 96% | 96% |
|  | 420 | ai | 99% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 99% | 97% | 97% | 96% | 100% | 100% | 100% | 99% |
|  |  | lb | 100% | 98% | 100% | 99% | 101% | 101% | 99% | 98% |

## Test 3.2

|  |  |  | Average |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| Lossy | 444 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 99% | 99% | 99% | 99% | 98% | 97% | 99% | 99% |
|  |  | lb | 99% | 99% | 99% | 99% | 98% | 97% | 100% | 100% |
|  | 444/4x1 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 99% | 99% | 99% | 99% | 98% | 97% | 99% | 99% |
|  |  | lb | 99% | 99% | 99% | 99% | 98% | 96% | 100% | 100% |
|  | 420 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 100% | 99% | 99% | 99% | 98% | 98% | 99% | 99% |
|  |  | lb | 100% | 99% | 99% | 99% | 98% | 98% | 100% | 100% |
| Lossless | 444 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 99% | 98% | 97% | 97% | 95% | 93% | 98% | 97% |
|  |  | lb | 99% | 98% | 97% | 96% | 95% | 92% | 98% | 97% |
|  | 420 | ai | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |
|  |  | ra | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |
|  |  | lb | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. | N.A. |

|  |  |  | Maximum |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| Lossy | 444 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 100% | 99% | 98% | 97% | 94% | 96% | 100% | 98% |
|  |  | lb | 99% | 99% | 98% | 98% | 94% | 96% | 94% | 94% |
|  | 444/4x1 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 100% | 100% | 98% | 97% | 93% | 96% | 99% | 98% |
|  |  | lb | 99% | 98% | 97% | 97% | 97% | 105% | 97% | 97% |
|  | 420 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 99% | 98% | 97% | 95% | 97% | 97% | 98% | 96% |
|  |  | lb | 100% | 99% | 97% | 96% | 94% | 96% | 98% | 97% |
| Lossless | 444 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 100% | 100% | 96% | 95% | 96% | 98% | 96% | 96% |
|  |  | lb | 98% | 97% | 96% | 95% | 97% | 97% | 98% | 98% |
|  | 420 | ai | #DIV/0! | ##### | ###### | ###### | ##### | ##### | ##### | ##### |
|  |  | ra | #DIV/0! | ##### | ###### | ###### | ##### | ##### | ##### | ##### |
|  |  | lb | #DIV/0! | ##### | ###### | ###### | ##### | ##### | ##### | ##### |

## Test 3.3

|  |  |  | Average |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| Lossy | 444 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 99% | 99% | 99% | 99% | 98% | 97% | 99% | 99% |
|  |  | lb | 99% | 99% | 99% | 99% | 98% | 97% | 100% | 100% |
|  | 444/4x1 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 99% | 99% | 99% | 99% | 98% | 97% | 99% | 99% |
|  |  | lb | 99% | 99% | 99% | 99% | 98% | 96% | 100% | 100% |
|  | 420 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 100% | 99% | 99% | 99% | 98% | 98% | 99% | 99% |
|  |  | lb | 100% | 99% | 99% | 99% | 98% | 98% | 100% | 100% |
| Lossless | 444 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 99% | 99% | 97% | 97% | 95% | 93% | 98% | 97% |
|  |  | lb | 99% | 98% | 97% | 96% | 95% | 92% | 98% | 97% |
|  | 420 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 99% | 99% | 98% | 97% | 96% | 94% | 98% | 97% |
|  |  | lb | 99% | 99% | 98% | 97% | 97% | 96% | 99% | 98% |

|  |  |  | Maximum |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| Lossy | 444 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 100% | 99% | 98% | 97% | 94% | 96% | 100% | 98% |
|  |  | lb | 100% | 100% | 99% | 98% | 94% | 96% | 94% | 94% |
|  | 444/4x1 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 100% | 100% | 98% | 97% | 93% | 96% | 99% | 98% |
|  |  | lb | 98% | 98% | 96% | 96% | 93% | 98% | 97% | 97% |
|  | 420 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 99% | 98% | 97% | 95% | 97% | 97% | 98% | 96% |
|  |  | lb | 99% | 99% | 98% | 98% | 94% | 96% | 98% | 97% |
| Lossless | 444 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 100% | 100% | 96% | 95% | 96% | 98% | 96% | 96% |
|  |  | lb | 98% | 97% | 96% | 95% | 97% | 97% | 98% | 98% |
|  | 420 | ai | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 100% | 97% | 96% | 95% | 97% | 97% | 94% | 94% |
|  |  | lb | 96% | 94% | 98% | 97% | 97% | 97% | 97% | 97% |

## Test 3.4

|  |  |  | Average |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| Lossy | 444 | ai | 64% | 64% | 64% | 64% | 64% | 65% | 100% | 100% |
|  |  | ra | 80% | 81% | 81% | 81% | 81% | 80% | 100% | 99% |
|  |  | lb | 79% | 80% | 80% | 80% | 80% | 80% | 99% | 99% |
|  | 444/4x1 | ai | 62% | 62% | 62% | 62% | 63% | 63% | 100% | 100% |
|  |  | ra | 80% | 80% | 81% | 81% | 81% | 80% | 100% | 100% |
|  |  | lb | 79% | 80% | 80% | 80% | 80% | 80% | 100% | 100% |
|  | 420 | ai | 61% | 61% | 61% | 62% | 62% | 62% | 100% | 100% |
|  |  | ra | 81% | 81% | 81% | 81% | 81% | 80% | 100% | 99% |
|  |  | lb | 80% | 81% | 81% | 81% | 81% | 80% | 99% | 99% |
| Lossless | 444 | ai | 76% | 76% | 76% | 76% | 75% | 75% | 100% | 100% |
|  |  | ra | 77% | 77% | 78% | 78% | 77% | 76% | 99% | 99% |
|  |  | lb | 81% | 82% | 82% | 82% | 80% | 79% | 99% | 99% |
|  | 420 | ai | 76% | 76% | 76% | 76% | 76% | 76% | 100% | 100% |
|  |  | ra | 81% | 81% | 80% | 81% | 79% | 78% | 99% | 98% |
|  |  | lb | 81% | 82% | 81% | 82% | 80% | 80% | 99% | 98% |

|  |  |  | Maximum |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| Lossy | 444 | ai | 100% | 99% | 99% | 99% | 98% | 96% | 100% | 100% |
|  |  | ra | 100% | 97% | 91% | 90% | 92% | 92% | 103% | 101% |
|  |  | lb | 90% | 87% | 81% | 80% | 86% | 91% | 96% | 94% |
|  | 444/4x1 | ai | 99% | 99% | 100% | 99% | 100% | 99% | 100% | 100% |
|  |  | ra | 100% | 98% | 91% | 89% | 90% | 92% | 99% | 98% |
|  |  | lb | 88% | 84% | 81% | 80% | 96% | 104% | 98% | 97% |
|  | 420 | ai | 99% | 99% | 99% | 99% | 99% | 99% | 100% | 100% |
|  |  | ra | 98% | 93% | 87% | 87% | 92% | 91% | 103% | 102% |
|  |  | lb | 87% | 83% | 83% | 83% | 83% | 89% | 103% | 102% |
| Lossless | 444 | ai | 99% | 99% | 99% | 99% | 99% | 99% | 100% | 100% |
|  |  | ra | 100% | 100% | 93% | 92% | 99% | 101% | 99% | 98% |
|  |  | lb | 100% | 100% | 99% | 99% | 100% | 100% | 96% | 96% |
|  | 420 | ai | 99% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |
|  |  | ra | 99% | 97% | 97% | 96% | 100% | 100% | 100% | 99% |
|  |  | lb | 100% | 98% | 100% | 99% | 101% | 101% | 99% | 98% |

# List of CE related proposals

| Doc No. | Title | Proponents | Type |
| --- | --- | --- | --- |
| JCTVC-U0065 | CE2-related: Intra block copy searching constraints for reducing worst case bandwidth | T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei (MediaTek) | Proposal |
| JCTVC-U0077 | On chroma motion vector derivation for intra block copy | X. Xu, S. Liu, S. Lei (MediaTek) | Proposal |
| JCTVC-U0079 | HLS : On Intra block copy signaling control | K. Rapaka, V. Seregin, R. Joshi, M. Karczewicz, Y.-K. Wang (Qualcomm), A. Tourapis, Y. Su, D. Singer (Apple) | Proposal |
| JCTVC-U0080 | Intra block copy chroma interpolation for Non-444 | K. Rapaka, V. Seregin, R. Joshi, M. Karczewicz (Qualcomm) | Proposal |
| JCTVC-U0103 | On chroma derivation of intra block copy for non-444 video | X. Xiu, Y. Ye, Y. He (InterDigital) | Proposal |

# Other IBC related proposals

| Doc No. | Title | Proponents | Type |
| --- | --- | --- | --- |
| JCTVC-U0054 | Non-CE2: On interpretation of the IBC vectors | C. Gisquet, G. Laroche, P. Onno (Canon) | Proposal |
| JCTVC-U0056 | Unification of the Adaptive Motion Resolution signalling for both IBC and Inter modes | G. Laroche, G. Malard, C. Gisquet, P. Onno (Canon) | Proposal |
| JCTVC-U0076 | On unification of motion vector resolution for screen content coding | X. Xu, S. Liu, S. Lei (MediaTek) | Proposal |
| JCTVC-U0081 | On unification of adapative motion vector resolution | K. Rapaka, V. Seregin, C. Pang, M. Karczewicz, R. Joshi (Qualcomm) | Proposal |
| JCTVC-U0102 | On constrained intra prediction for the unification framework of intra block copy | X. Xiu, Y. Ye, Y. He (InterDigital) | Proposal |
| JCTVC-U0104 | On unification framework of intra block copy | X. Xiu, Y. Ye, Y. He (InterDigital) | Proposal |
| JCTVC-U0107 | On stored decoded motion vector resolution | X. Xiu, Y. Ye, Y. He (InterDigital) | Proposal |
| JCTVC-U0111 | Non-CE: MV resolution unification for MV derivation in Intra-Block Copy search | J.-S. Tu, C.-C. Lin, C.-L. Lin, Y.-J. Chang (ITRI) | Proposal |
| JCTVC-U0118 | On intra block copy bitstream constraints | V. Seregin, K. Rapaka, R. Joshi, M. Karczewicz, Y.-K. Wang (Qualcomm) | Proposal |

# References

- K. Rapaka, C. Gisquet, J. Lainema, X. Xu, “Description of Core Experiment 2 (CE2): Intra block copy memory access, Geneva, CH, Feb. 2015.

- SCM-4.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0/

- K. Rapaka, C. Gisquet, J. Lainema, X. Xu, “Common conditions for screen content coding tests,” JCTVC-T1015, Geneva, CH, Feb. 2015.

- E. François, A. Tabatabai, and E. Alshina “BoG report: Methodology for evaluating complexity of combined and residual prediction methods in SHVC”, JCTVC-L0440, Geneva, CH, Jan. 2013.