| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0031 |
| --- | --- |

| Title: | Discrepancy between Version 2 text and software for High Throughput Profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | K. Sharman, N. Saunders Sony BPRL Jays Close, Viables, Basingstoke RG22 4SB, UK | Tel: Email: | +44 1256 355011 karl.sharman@eu.sony.com nick.saunders@eu.sony.com |
| Source: | Sony Europe Ltd. |  |  |

_____________________________

# Abstract

A discrepancy between the software and the HEVC Version 2 text has been identified that affects the CABAC alignment tool present in the high throughput profile. This error was first reported on the bug-tracker (ticket #1391), where it has been indicated that the software implements the intended function.

Results are presented to compare the implementation in the standardized text relative to the implementation in the software. The results indicate that the implementation in the standardized text is ~0.05% less efficient in terms of BD-rate for low bit rate conditions, and less than 0.01% increase in BD-rate and bit rates for high bit rate and lossless test conditions.

It is proposed that the JCT-VC considers whether to amend the software or the standardized text to resolve this discrepancy.

# Introduction

The CABAC alignment tool was included into the RExt standard, and is used in the “High Throughput 4:4:4 16 Intra” profile. This tool was designed to allow equi-probable (EP) bin data to be extracted with more ease, by allowing such bins to be represented in raw form in the bit stream. The actual process is to merely set ivlCurrRange to 256.

The tool applies an alignment process prior to the coding of the EP bins for each 4x4 coefficient sub-block of the transform block. The alignment process itself introduces a bit-cost of ~0.5 bits per alignment, and for the high data rates that the profile is designed for this introduces an overhead of 0.5% (see test b3a, AHG18 test conditions, JCTVC-P0060).

However, in order to mitigate the losses if the profile were used at lower operating points (e.g. due to rate control), the application of the alignment process was made conditional. The condition reduced the losses incurred by alignment from 2.1% to 0.9% for the lower operating points defined in the historic AHG5 test conditions (see JCTVC-P0060, comparing experiment B1 with experiment B3a).

It is in this conditional test where there is a discrepancy between the text and the software.

The intended operation, described in JCTVC-O0046, which is implemented in the software, was to only apply the alignment process if any coefficient in the coefficient group had not had its magnitude fully described by the preceding CABAC bins. i.e., if any of the coefficients in the coefficient group required the coding of coeff_abs_level_remaining data. This lead to a variable called escapeDataPresent to be defined, which should be reset to 0 at the start of each 4x4 coefficient group, and then gets set to 1 for any coefficient that requires coeff_abs_level_remaining data to be coded. Only if the value of escapeDataPresent is equal to1 prior to coding the sign bits is the alignment process applied. This caps the maximum number of conventionally coded EP bins to 16.

However, the text does not reset escapeDataPresent to 0 at the start of each 4x4 coefficient group, but at the start of the transform block itself. This means that once a block encounters any coefficient that requires coeff_abs_level_remaining data to be coded, all subsequent blocks will also align data.

The result is that the system will align more often than originally intended.

To aid discussions, this contribution includes results from the two systems, described in the next section.

# Systems tested

Two systems are presented in this document, and the common test conditions (CTC) for intra configurations using HM16.5 as the basis have been simulated. These configurations are the 4:2:0 Version 1 CTC, the RExt “AHG5” CTC, the RExt screen content “AHG8” lossy and lossless CTC and the high bit depth “AHG18” lossy and lossless CTC.

Only intra configurations have been tested since the High Throughput 4:4:4 16 Intra profile is intra only.

Although the High Throughput 4:4:4 16 Intra profile is targeted at bit rates commonly seen in the AHG18 test conditions, the purpose of the conditional alignment is to extend the potential operating points down to lower data rates for the benefits of rate control stability. Therefore the lower bit rates seen in, for example, the 4:2:0 CTC illustrate the trends in performance.

The two systems are:

- “H16.5 alignment”: Using the common test conditions (CTC), but setting the profile to the High Throughput 4:4:4 16 Intra  profile and using the alignment scheme present in HM16.5. This was configured by adding the following command line arguments over those used for the CTC:

  - --Profile=high-throughput-RExt

  - --AlignCABACBeforeBypass=1

  - --IntraConstraintFlag=1

- “Text alignment”: As with 1, but changing the source code to match the HEVC Version 2 text. This can be achieved by moving the definition of the escapeDataInGroup flag to outside the group loop.

In addition, results have been included in this contribution that illustrate why conditionality was applied to the alignment process, and results can also be found relative to the HM16.5 base test conditions (not using the High Throughput 4:4:4 16 Intra profile).

# Results

## Version 1 CTC (4:2:0)

The following table shows the results of the Version 1 CTC (4:2:0) of the alignment scheme described in the standardized text relative to the alignment scheme implemented as intended in the HM16.5 software. There is on average a 0.05% increase in BD-rate of the scheme described in the standardized text:

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.07% | 0.07% | 0.09% | 0.08% | 0.09% | 0.11% |
| Class B | 0.04% | 0.08% | 0.09% | 0.05% | 0.08% | 0.05% |
| Class C | 0.03% | 0.05% | 0.02% | 0.04% | 0.03% | 0.05% |
| Class D | 0.03% | 0.04% | 0.07% | 0.04% | 0.05% | 0.07% |
| Class E | 0.06% | 0.09% | 0.09% | 0.05% | 0.07% | 0.08% |
| Class F | 0.01% | -0.02% | 0.01% | 0.02% | 0.00% | 0.05% |
| Overall | 0.04% | 0.05% | 0.06% | 0.05% | 0.05% | 0.06% |

## RExt (AHG5) CTC (4:2:2, 4:4:4)

The following table shows the results of the RExt CTC (4:2:2 and 4:4:4) of the alignment scheme described in the standardized text relative to the alignment scheme implemented as intended in the HM16.5 software. The losses are slightly higher than the results presented in Section 3.1, with an average over all components of 0.11% increase, or 0.07% average increase over the luma or green channels:

|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| RGB 4:4:4 | 0.10% | 0.14% | 0.12% | 0.12% | 0.16% | 0.14% | 0.11% | 0.16% | 0.14% |
| YCbCr 4:4:4 | 0.06% | 0.09% | 0.07% | 0.08% | 0.16% | 0.11% | 0.09% | 0.22% | 0.15% |
| YCbCr 4:2:2 | 0.04% | 0.09% | 0.07% | 0.04% | 0.10% | 0.08% | 0.04% | 0.12% | 0.10% |

## RExt Screen Content (AHG8) CTC

The following tables shows the lossy and lossless results of the RExt Screen Content (AHG8) CTC of the alignment scheme described in the standardized text relative to the alignment scheme implemented as intended in the HM16.5 software. The losses are on average 0.04% increase over all components, or 0.03% increase over just the luma or green channels for the lossy test conditions; the average bit rate increase is 0.01% for the lossless test conditions.

|  | Standardized text vs HM16.5 implementation (lossy) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main-tier |  |  | All Intra High-tier |  |  | All Intra Super-High-tier |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class F | 0.00% | -0.01% | -0.01% | 0.01% | 0.00% | 0.01% | 0.01% | 0.01% | 0.01% |
| Class B | 0.05% | 0.06% | 0.02% | 0.05% | 0.09% | 0.06% | 0.06% | 0.15% | 0.10% |
| RGB 4:4:4 SC | 0.04% | 0.03% | 0.05% | 0.04% | 0.02% | 0.04% | 0.02% | 0.03% | 0.03% |
| RGB 4:4:4 Animation | 0.04% | 0.05% | 0.05% | 0.04% | 0.04% | 0.05% | 0.04% | 0.05% | 0.06% |
| YCbCr 4:4:4 SC | 0.03% | 0.03% | -0.01% | 0.02% | 0.02% | 0.00% | 0.00% | 0.01% | 0.00% |
| YCbCr 4:4:4 Animation | 0.05% | 0.06% | 0.06% | 0.05% | 0.07% | 0.07% | 0.05% | 0.08% | 0.07% |
| RangeExt | 0.04% | 0.08% | 0.05% | 0.06% | 0.15% | 0.10% | 0.09% | 0.21% | 0.17% |
| RGB 4:4:4 SC (Optional) | -0.01% | 0.01% | 0.02% | -0.01% | 0.01% | 0.01% | 0.00% | -0.01% | 0.00% |
| YCbCr 4:4:4 SC (Optional) | 0.00% | -0.02% | -0.03% | 0.01% | -0.01% | -0.01% | 0.01% | 0.00% | -0.01% |

|  | Standardized text vs HM16.5 implementation (lossless) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Total | Avg | Min | Max |
| Class F | 0.01% | 0.01% | 0.00% | 0.02% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% |
| RGB 4:4:4 SC | 0.01% | 0.01% | -0.01% | 0.04% |
| RGB 4:4:4 Animation | 0.03% | 0.03% | 0.01% | 0.06% |
| YCbCr 4:4:4 SC | 0.01% | 0.01% | -0.01% | 0.03% |
| YCbCr 4:4:4 Animation | 0.02% | 0.03% | 0.00% | 0.04% |
| RangeExt | 0.00% | 0.00% | 0.00% | 0.00% |
| RGB 4:4:4 SC (Optional) | 0.01% | 0.01% | 0.00% | 0.02% |
| YCbCr 4:4:4 SC (Optional) | 0.01% | 0.01% | 0.00% | 0.04% |

## High bit depth CTC (RGB 12 and 16-bit)

The following tables shows the lossy and lossless results of the RExt high bit depth (AHG18) CTC of the alignment scheme described in the standardized text relative to the alignment scheme implemented as intended in the HM16.5 software. The losses are on average 0.04% increase over all components, or 0.03% increase over just the luma or green channels for the lossy test conditions; the average bit rate increase is 0.01% for the lossless test conditions.

|  | Standardized text vs HM16.5 implementation (lossy) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Intra 16-bit |  |  | Intra 12-bit |  |  |
|  | Y/G | Cb/B | Cr/R | Y/G | Cb/B | Cr/R |
| SVT | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| RExt RGB | 0.00% | 0.00% | 0.00% | 0.02% | 0.02% | 0.01% |
| HDR | 0.01% | 0.01% | 0.01% | 0.05% | 0.06% | 0.06% |
| 4K from HDR | 0.00% | 0.00% | 0.00% | 0.02% | 0.03% | 0.03% |
| HDR 4:0:0 Medical | 0.01% |  |  | 0.09% |  |  |
| 4:0:0 Medical | 0.00% |  |  | 0.09% |  |  |
| Synthetic RGB HDR (+2 MSBs) | 0.00% | 0.00% | 0.00% | 0.08% | 0.12% | 0.10% |
| Synthetic RGB HDR (+4 MSBs) | 0.02% | 0.02% | 0.02% | 0.09% | 0.18% | 0.13% |
| Synthetic RGB HDR (+6 MSBs) | 0.08% | 0.11% | 0.09% | 0.05% | 0.13% | 0.04% |
| All Classes | 0.01% | 0.02% | 0.02% | 0.05% | 0.08% | 0.05% |

|  | Standardized text vs HM16.5 implementation (lossless) |  |
| --- | --- | --- |
|  | Intra 16-bit | Intra 12-bit |
| SVT | 0.00% | 0.00% |
| RExt RGB | 0.00% | 0.00% |
| HDR | 0.00% | 0.01% |
| 4K from HDR | 0.00% | 0.00% |
| HDR 4:0:0 Medical | 0.00% | 0.00% |
| 4:0:0 Medical | 0.00% | 0.00% |
| Synthetic RGB HDR (+2 MSBs) | 0.00% | 0.00% |
| Synthetic RGB HDR (+4 MSBs) | 0.00% | 0.01% |
| Synthetic RGB HDR (+6 MSBs) | 0.00% | 0.09% |
| All Classes | 0.00% | 0.01% |

# Changes to address the discrepancy

There are two possible changes, as given in the next two sections. The first is to change the standardized text to match the algorithm given in the software. The second is to just update the software to match the algorithm as described in the standardized text.

## Option 1: adjusting the standardized text to match the software

| if( last_sig_coeff_y_prefix > 3 ) |  |
| --- | --- |
| last_sig_coeff_y_suffix | ae(v) |
| lastScanPos = 16 |  |
| lastSubBlock = ( 1 << ( log2TrafoSize − 2 ) ) * ( 1 << ( log2TrafoSize − 2 ) ) − 1 |  |
| escapeDataPresent = 0 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = 16 |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ lastSubBlock ][ 0 ] |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ lastSubBlock ][ 1 ] |  |
| xC = ( xS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << 2 ) + ScanOrder[ 2 ][ scanIdx ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| xS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 0 ] |  |
| yS = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ i ][ 1 ] |  |
| escapeDataPresent = 0 |  |
| inferSbDcSigCoeffFlag = 0 |  |

## Option 2: adjusting the software to match the standardized text

Changes for TDecSbac.cpp

const Int iLastScanSet = uiScanPosLast >> MLS_CG_SIZE; UInt c1 = 1; Uint uiGoRiceParam = 0; Bool escapeDataPresentInGroup = false; // and rename to escapeDataPresent Uint uiSigCoeffGroupFlag[ MLS_GRP_NUM ]; memset( uiSigCoeffGroupFlag, 0, sizeof(Uint) * MLS_GRP_NUM ); Int iScanPosSig = (Int) uiScanPosLast; for( Int iSubSet = iLastScanSet; iSubSet >= 0; iSubSet-- ) { Int iSubPos = iSubSet << MLS_CG_SIZE; uiGoRiceParam = currentGolombRiceStatistic / RExt__GOLOMB_RICE_INCREMENT_DIVISOR; Bool updateGolombRiceStatistics = bUseGolombRiceParameterAdaptation; //leave the statistics at 0 when not using the adaptation system Int numNonZero = 0; Int lastNZPosInCG = -1; Int firstNZPosInCG = 1 << MLS_CG_SIZE; Bool escapeDataPresentInGroup = false; Int pos[1 << MLS_CG_SIZE];

Changes for TEncSbac.cpp

UInt c1 = 1; UInt uiGoRiceParam = 0; Int iScanPosSig = scanPosLast; Bool escapeDataPresentInGroup = false; // and rename to escapeDataPresent for( Int iSubSet = iLastScanSet; iSubSet >= 0; iSubSet-- ) { Int numNonZero = 0; Int iSubPos = iSubSet << MLS_CG_SIZE; uiGoRiceParam = currentGolombRiceStatistic / RExt__GOLOMB_RICE_INCREMENT_DIVISOR; Bool updateGolombRiceStatistics = bUseGolombRiceParameterAdaptation; //leave the statistics at 0 when not using the adaptation system UInt coeffSigns = 0; Int absCoeff[1 << MLS_CG_SIZE]; Int lastNZPosInCG = -1; Int firstNZPosInCG = 1 << MLS_CG_SIZE; Bool escapeDataPresentInGroup = false;

A number of conformance bit streams will also have to be updated accordingly.

# Conclusion

A discrepancy between the software and the standardized text relating to the conditional alignment scheme present only in the “High Throughput 4:4:4 16 Intra” profile has been highlighted. The discrepancy causes the version in the standardized text to align more often than intended, although the principal aim to limit the worst case number of equi-probable (EP) bins per 4x4 coefficient sub-group is still maintained. The two systems have been compared, and although there are increased inefficiencies, these are typically around ~0.05% for the lower bit rate test conditions (main tier, luma, AHG5 RExt CTC), and ~0.01% for the high bit rate and lossless test conditions (16-bit, green component, AHG18 lossy CTC).

It is proposed that the JCT-VC considers whether to amend the software or the standardized text to resolve this discrepancy.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).