| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0032_r1 |
| --- | --- |

| Title: | Proposed addition of transfer characteristics in VUI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y. Nishida, T. Yamashita, and A. Ichigaya 1-10-11 Kinuta, Setagaya-ku, Tokyo 157-8510 Japan T. Shimizu 5-3-6 Akasaka, Minato-ku, Tokyo 107-8006 Japan | Tel: Email: | nishida.y-fe@nhk.or.jp yamashita.t-hq@nhk.or.jp ichigaya.a-go@nhk.or.jp shimizu3@lab.tbs.co.jp |
| Source: | ARIB (Association of Radio Industries and Businesses) and NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

This document proposes a new entry to transfer characteristics of video usability information (VUI) in Annex E of the HEVC specification to support a new opto-electronic transfer function (OETF) for the extended image dynamic range television (EIDRTV) on the basis of a new ARIB Standard STD-B67.

# Introduction

This document proposes a new entry to transfer characteristics of VUI in Annex E of the HEVC specification to support a new OETF for the EIDRTV on the basis of ARIB STD-B67 “Essential parameter values for the extended image dynamic television system (EIDRTV)” [1].

# ARIB STD-B67

## Objectives and scope

ARIB STD-B67 specifies the system parameters essential for the extended image dynamic range television (EIDRTV) including system colorimetry, signal format, and digital representation. For the other system parameters, other television system standards may be referred to. This standard applies to the video equipment for programme production of the extended image dynamic range television.

## System parameter values

System colorimetry is as shown in Table 1 and is identical to that of Recommendation ITU-R BT.2020 [2].

Signal format is as shown in Table 2. Important parameters for the EIDRTV are the reference white and opto-electronic transfer function (OETF, see Figure 1). Luminance and color difference signals are identical to those of Recommendation ITU-R BT.2020.

Digital representation of video signal is as shown in Table 3. Both 10-bit and 12-bit are specified with the same quantization as that of Recommendation ITU-R BT.2020.

TABLE 1

System colorimetry

| Parameter | Values |  |  |
| --- | --- | --- | --- |
| Opto-electronic transfer characteristics before non-linear pre-correction | Assumed linear |  |  |
| Primary colors and reference white1 | Chromaticity coordinates (CIE, 1931) | x | y |
|  | Red primary (R) | 0.708 | 0.292 |
|  | Green primary (G) | 0.170 | 0.797 |
|  | Blue primary (B) | 0.131 | 0.046 |
|  | Reference white (D65) | 0.3127 | 0.3290 |
| 1 The colorimetric values of the picture information can be determined based on the reference RGB primaries and the reference white. |  |  |  |

TABLE 2

Signal format

| Parameter | Values |
| --- | --- |
| Signal format | R', G', B' or Y', C'B, C'R |
| Video level corresponding to reference white level, r | 0.5 |
| Reference non-linear transfer function (opto-electronic transfer function, OETF) | E'=rE0≤E≤1a⋅lnE−b+c1<E where E is voltage normalized by the reference white level and proportional to the implicit light intensity that would be detected with a reference camera color channel R, G, B; E' is the resulting non-linear signal. a=0.17883277,b=0.28466892,c=0.55991073 |
| Derivation of Y’ | Y'=0.2627R'+0.6780G'+0.0593B' |
| Derivation of color difference signals | CB'=B'−Y'1.8814CR'=R'−Y'1.4746 |

TABLE 3

Digital representation

| Parameter |  | Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Coded signal |  | R', G', B' or Y', C'B, C'R |  |  |  |  |
| Sampling lattice – R', G', B', Y' |  | Orthogonal, line and picture repetitive co-sited |  |  |  |  |
| Sampling lattice – C'B, C'R |  | Orthogonal, line and picture repetitive co-sited with each other. The first (top-left) sample is co-sited with the first Y’ samples. |  |  |  |  |
|  |  | 4:4:4 system |  | 4:2:2 system |  | 4:2:0 system |
|  |  | Each has the same number of horizontal samples as the Y' component. |  | Horizontally subsampled by a factor of two with respect to the Y' component. |  | Horizontally and vertically subsampled by a factor of two with respect to the Y' component. |
| Coding format |  | 10 or 12 bits per component |  |  |  |  |
| Quantization of R', G', B', Y', C'B, C'R |  | DR'=INT219×R'+16×2n−8 DG'=INT219×G'+16×2n−8 DB'=INT219×B'+16×2n−8 DY'=INT219×Y'+16×2n−8 DCB'=INT224×CB'+128×2n−8 DCR'=INT224×CR'+128×2n−8 The n represents the bit depth of coding format. INT [X ] is the function that returns the integer closest to X. |  |  |  |  |
| Quantization levels |  |  | 10-bit coding |  | 12-bit coding |  |
| Timing Reference |  |  | 0 - 3 and 1 020 - 1 023 |  | 0 - 15 and 4 080 - 4 095 |  |
| Video data |  |  | 4 - 1 019 |  | 16 - 4 079 |  |
| DR', DG', DB', DY' | Nominal Peak (100%) |  | 940 |  | 3 760 |  |
|  | Reference White (r) |  | 502 |  | 2 008 |  |
|  | Black (0%) |  | 64 |  | 256 |  |
| DC'B, DC'R | Nominal Peak (+50%) |  | 960 |  | 3 840 |  |
|  | Achromatic (0%) |  | 512 |  | 2 048 |  |
|  | Nominal Peak (-50%) |  | 64 |  | 256 |  |

Figure 1

OETF

# Proposed addition to VUI parameter values

The EIDRTV can be signalled with the existing VUI parameter values except for the transfer characteristics. It is thus proposed that a new entry corresponding to the transfer characteristics be included in Table E 4 – Transfer characteristics of the HEVC standard. Change text is copied herein for reader convenience that has been made with respect to the latest HEVC editor’s draft [3].

Add the following entry (#18) to Table E-4 (Transfer Characteristics)

| 18 | V = 0.5 * Lc0.5 for 1 >= Lc >= 0 V = a * ln (Lc – b) + c for Lc > 1 a = 0.17883277, b = 0.28466892, c = 0.55991073 | Association of Radio Industries and Businesses (ARIB) STD-B67 |
| --- | --- | --- |

A suggested delta document for Table E4 and the relevant text in Annex E is provided (JCTVC-U0032_delta_Table_E4_changes_r1.docx) in the attached .zip archive of this proposal.

It should be noted that ARIB is one of the qualified organizations as per Recommendation ITU-T A.5 and its standards can be referred to in ITU-T Recommendations.

# Conclusion

An additional VUI parameter value for transfer characteristics is proposed to signal the EIDRTV as specified in ARIB STD-B67.

# References

[1] Association of Radio Industries and Businesses ARIB STD-B67 Ver. 1.0 (2015), Essential parameter values for the extended image dynamic television system (EIDRTV).

[2] ITU-R BT.2020-1 (2014), Parameter Values for ultra-high definition television systems for production and internal programme exchange

[3] R. Joshi, S. Liu, J. Xu, Y. Ye, “High Efficiency Video Coding (HEVC) Screen Content Coding: Draft 3”, JCTVC-T1005.

# Patent rights declaration(s)

ARIB and NHK do not have any current or pending patent rights relating to the technology described in this contribution.