| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0033 |
| --- | --- |

| Title: | High dynamic range compatibility information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Matteo Naccari Andrew Cotton Sebastian Schwarz Manish Pindoria Marta Mrak Tim Borer Centre House, Block D 56 Wood Lane, W12 7SB London | Tel: Email: | +44 (0)303 0409640 matteo.naccari@bbc.co.uk andrew.cotton@bbc.co.uk sebastian.schwarz@bbc.co.uk manish.pindoria@bbc.co.uk marta.mrak@bbc.co.uk tim.borer@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

The BBC and NHK have jointly proposed a new Opto-Electronic Transfer Function (OETF) for High Dynamic Range (HDR) television production and distribution to ITU-R WP 6C. The same OETF solution also forms the basis of a new ARIB Standard (STD-B67). This HDR OETF has been designed to also deliver compatible images to legacy standard dynamic range displays. To exploit the backwards compatibility in distribution applications, maintaining compatibility with legacy receivers using HEVC Main 10 profile, a new signalling is required to indicate the type of HDR material. This contribution proposes an SEI message to enable this signalling and the distribution of HDR content in broadcasting services.

# Introduction and problem statement

With the advent of high dynamic range displays, content providers have the opportunity to provide viewers with a much more compelling quality of experience when watching television. Broadcasters would like to transmit HDR services while limiting the impact that the required changes will have on the broadcasting chain. Accordingly, the BBC and NHK have jointly proposed a new Opto-Electronic Transfer Function (OETF) for high dynamic range television production and distribution to ITU-R WP 6C. A key benefit of this OETF is that is able to deliver a high quality compatible image to legacy standard dynamic range receivers and displays with only minor colour distortions, which would be acceptable in all but the most critical applications. This OETF has been jointly proposed by the BBC and NHK to ITU-R WP6C for production and exchange of Extended Image Dynamic Range Television (EIDRTV) and forms the basis of a new ARIB Standard STD-B67.

Compatibility with legacy systems and displays is achieved by splicing a log curve on to a traditional “gamma” OETF, in much the same way as a traditional camera “knee”. The combined curve is referred to as a “Hybrid Log-Gamma” transfer characteristics. The logarithmic portion greatly improves the ability to capture specular highlights. But unlike a camera “knee” it is well defined, so can be exploited by a display. The “Hybrid Log-Gamma” OETF is compared with the standard BT.709/BT.2020 OETFs in Figure 1.

Figure 1 - Comparison of BT.709/BT.2020 and Hybrid Log-Gamma OETFs

The “Hybrid Log-Gamma” solution requires no mastering metadata, which proved very difficult to support and unreliable in a “live” television environment. Moreover, the traditional Playout and Presentation operations of mixing, graphics overlays and digital video effects become almost impossible when sources carry different metadata. The only solution would be to transcode all source to a common format, which inevitably degrades the signal and adds unnecessary cost and complexity.

In a distribution environment, when an HEVC bitstream is serving both SDR and HDR receivers, it is important to maintain backward compatibility with legacy displays but, at the same time, allow HDR displays to be able to properly show the content. Backwards compatibility with legacy SDR receivers, is achieved by setting transfer_characteristics syntax element in the VUI to indicate the SDR BT.709, IEC 61966-2-4, or BT.2020 OETFs (i.e. values 1, 6, 11, 14 or 15). Conversely, for HDR receivers, a new signalling is required to recognise that the bitstream is in fact high dynamic range content, encoded using the “Hybrid Log-Gamma” OETF. Moreover, in the same broadcasting Distribution scenario, also the different colour primaries of the SDR and HDR receiver need to be addressed. In fact, even when the HDR and SDR colour primaries are identical, the HDR colour volume is greater than the SDR one as it can achieve higher luminance levels. Therefore, saturated colours in the HDR space may appear slightly de-saturated and suffer from a minor hue shift when viewed on an SDR screen. These colour artefacts can be avoided/minimised by using a colour companding technique implemented in the display and applied before the picture is shown on the screen. The colour companding technique is reversible and hence the full HDR colour volume can be restored by the decoder, using a similar post-processor. The colour volume companding and complimentary expanding algorithms may be implemented using bespoke hardware or perhaps more conveniently within a 3D-LUT, illustrated in Figure 2. Different colour companding techniques can be applied to address the conversion between SDR and HDR colour volumes as well as different colour primaries used for the SDR and HDR displays. Therefore, in order to properly handle the received content, the receiver should be also aware of colour companding to be used.

To address the need of signalling the used OETF, this contribution proposes a new SEI message which carries the necessary information. For backward compatibility, the SEI paradigm is preferred rather than adding a VUI parameter for the following two main reasons:

- An additional entry in the VUI may interfere with Version 1 decoders while the SEI message will be simply ignored by these decoders

- Some applications may require frame accurate dynamic changes to the signalling which are better addressed by sending an SEI message whenever required.

The remainder of this contribution is organised as follows. Section 2 presents the proposed SEI message by describing its syntax and semantics. Section 1 instead details on the design features of a colour companding technique which can be used for the aforementioned purposes.

Figure 2 - Improved SDR compatibility through pre- and post-processing

# Proposed HDR compatibility information SEI message

This section presents the syntax and semantics for the proposed SEI message. All the changes required to the current HEVC draft specification text are highlighted in yellow.

## Proposed HDR compatibility information SEI message syntax

D.2.1	General SEI message syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| ... |  |
| else if( payloadType = = 181 ) |  |
| hdr_compatibility_info( payloadSize ) |  |
| ... |  |
| } |  |

D.2.35	High dynamic range compatibility information SEI message syntax

| hdr_compatibility_info( payloadSize ) { | Descriptor |
| --- | --- |
| hdr_compatibility_cancel_flag | u(1) |
| if( !hdr_compatibility_cancel_flag ) { |  |
| hdr_transfer_characteristics_idc | u(4) |
| } |  |
| } |  |

## Proposed HDR compatibility information SEI message semantics

D.3.35	High dynamic range compatibility information SEI message semantics

This SEI message provides information about the transfer characteristics used to grade the high dynamic range (HDR) video content associated with the CVS.

hdr_compatibility_cancel_flag equal to 1 indicates that the high dynamic range compatibility information SEI message cancels the persistence of any previous high dynamic range compatibility information SEI message in output order. hdr_compatibility_cancel_flag equal to 0 indicates that high dynamic range compatibility information follows.

hdr_transfer_characteristics_idc equal to 0 specifies that the opto-electronic transfer characteristics of the source picture is given by the value of the transfer_characteristics syntax element specified in Annex E. hdr_transfer_characteristics_idc equal to 1 specifies that the opto-electronic transfer characteristics of the source picture is a function of a linear optical intensity Lc in the range of 0 to 1 and given as:

V=r⋅LC0≤LC≤1a⋅lnLC−b+c1<LC

| V = 0.5 * Lc0.5 for 0 <= Lc <= 1 V = a * ln (Lc – b) + c for Lc > 1 a = 0.17883277, b = 0.28466892, c = 0.55991073 | Association of Radio Industries and Businesses (ARIB) STD-B67 |
| --- | --- |

Values greater than 1 for hdr_transfer_characteristics_idc are reserved for purposes to be specified in future versions of this Recommendation | International Standard.

NOTE X – The value specified by hdr_transfer_characteristics_idc is only considered valid when the value of the transfer_characteristics syntax element specified in Annex E is equal to 1, 6, 11, 14 and 15.

# Colour Companding Overview

This section describes a colour companding technique which may be used to convert from different colour primaries associated with SDR and HDR receivers. The principle behind this first colour companding technique, where HDR and SDR colour primaries are the same, is to convert the HDR signal to the true luminance CIE 1976 YU’V’ colour space and then compress the luminance signal so that it never exceeds the maximum allowable luminance in the target SDR BT.709, IEC 61966-2-4, DCI-P3 or BT.2020 colour space for the specific colour of a given pixel. The compression is reversible so the process can be undone in a “post-processing” stage. It should be emphasised that no compression is necessary for HDR monochrome signals. So any loss in signal fidelity arising as a result of the luminance compression and de-compression only occurs in saturated colours, where the eye would appear less sensitive to quantisation effects. Indeed when the HDR OETF is similar to the SDR BT.709/BT.2020 OETF, as is the case with the Hybrid Log-Gamma OETF, it can even be arranged that the process is transparent for monochrome and all but the brightest and most highly saturated pixels.

The HDR input signal is first converted back to linear light by applying the inverse of the signal’s OETF. From there the RGB linear light signals are converted to CIE 1931 XYZ colour space using a matrix transformation. Whilst the Y component from the XYZ colour space can be used directly, all three XYZ components are used to derive the more useful CIE 1976 YU’V’ colour components which are perceptually uniform. U’ and V’ are calculated from XYZ and the Y component remains unchanged.

The high dynamic range true luminance signal, YHDR, is first compressed to standard dynamic range, YSDR, using a function similar to that illustrated in Figure 3. As the compression function’s gradient is always greater than zero, the signals are never “clipped” and they can be de-compressed in the post-processor. As suitable compressor may have a linear section at the bottom so that low brightness signals are unaffected and a logarithmic section for higher value inputs. Alternatively the compressor can be designed to ensure that monochrome signals from the HDR Hybrid Log-Gamma OETF pass through the system unchanged. The compression curve should be carefully designed to ensure that the combined function is continuous and has a continuous first derivative.

Figure 3: Luminance compressor

In parallel a further function uses the U’V’ components to calculate the maximum permitted luminance value (YMAX), for that pixel colour in the target SDR BT.709/IEC 61966-2-4/DCI-P3/BT.2020 colour space.

To determine the maximum luminance value for a given colour, the following workflow is followed:

- Convert from the YU’V’ colour space to the XYZ one

- Write each RGB component as function of XYZ (using the values from the YU’V’ colour space)

- Compute YMAX by finding when one of the RGB is equal to 1.0 (i.e. the maximum value allowed)

For the first step, the X and Z colour coordinates of the XYZ colour space are expressed as:

X=Y⋅9u'4v'Z=12−3u'−20v'4v'.	(1)

Given XYZ components, then RGB components are calculated by pre-multiplying by a 3 × 3 matrix, where the matrix “M” depends on the chosen RGB colour space:

RGB=m11m12m13m21m22m23m31m32m33⋅XYZ.	(2)

Substituting equations (1) into equation (2) yields:

R=Ym119u'4v'+m12+m1312−3u'−20v'4v'G=Ym219u'4v'+m22+m2312−3u'−20v'4v'B=Ym319u'4v'+m32+m3312−3u'−20v'4v',	(3)

which can be re-written as:

R=Y4v'k11u'+k12v'+k13G=Y4v'k21u'+k22v'+k23B=Y4v'k31u'+k32v'+k33	(4)

where the values of the matrix K are defined as:

K=k11k12k13k21k22k23k31k32k33=9m11−3m134m12−20m1312m139m21−3m234m22−20m2312m239m31−3m334m32−20m3312m33.	(5)

Now, as stated above, for maximum luminance at least one of RGB must be 1.0. Therefore, from equation (4) one or more of the following must be true:

YMAX=4v'k11u'+k12v'+k13YMAX=4v'k21u'+k22v'+k23YMAX=4v'k31u'+k32v'+k33,	(6)

hence the maximum luminance is the minimum of the values calculated from equation (6).

Whilst SDR displays supporting BT.2020 colour are beginning to emerge, by far the majority of SDR displays will be BT.709 colour for some time to come. At the same time it is likely that the ITU-R specification for HDR will be based on BT.2020 colour primaries only. So in order for the HDR Production to fully exploit the benefits of a system delivering a compatible SDR image, that compatible image should conform to BT.709 colorimetry.

Conveniently the second form of colour volume companding, where a colour space transformation is required between the HDR and compatible SDR signal, can also be applied in the form of a reversible colour space conversion within the same pre-processor and post-processors. The technique allows a BT.2020 colour HDR signal to be converted to a signal that resembles SDR BT.709, IEC 61966-2-4 or P3D65 prior to HEVC encoding, and returned back to original HDR BT.2020 colour space with very little loss in quality. The same technique can also be used to transform an HDR DCI-P3 (D65) colour signal to a signal that resembles SDR BT.709 or IEC 61966-2-4.

Figure 4: Comparison of BT.709 and BT.2020 colour spaces

Figure 4 is a representation of all colours with red, green and blue shown as vertices in the U’V’ colour space. The wider gamut of BT.2020 is represented by the outer triangle and the narrower gamut of BT.709 by the inner triangle. On any particular line from the centre point shown by D65, a line radially outward from that point represents a single colour of increasing saturation. To bring values from the wider gamut to the narrower gamut a compression function may be used so that, without loss of any information, colours acquired using the wider gamut may be represented appropriately on a narrower gamut display.

The choice of compression function applied to the radial colour components of Figure 4 could be any one of a number of functions provided it does not clip the component. However, the “knee” type compressor illustrated in Figure 3 is particularly well suited as it provides minimum alteration to less saturated colours and a gradual change to the more saturated colours. Within the post-processor, a colour decompressor implements the inverse of the compression function to return the full colour gamut. Any quantisation loss though the compression/decompression process is limited to pure colour components where the eye is less sensitive.

# Patent rights declaration(s)

The British Broadcasting Corporation (BBC) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).