| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0034 |
| --- | --- |

| Title: | CE2: Test 3 on intra block copy constraints on filtering (single-pass encoding decisions) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Miska M. Hannuksela | Tel: Email: | +358 7180 08000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports results of CE2 Test 3 on intra block copy constraints on filtering when using single-pass encoding decisions. The intention of the tested method is to reduce both worst case and average memory bandwidth associated with the intra block copy operations. After encoding a CTU the proposed method decides if the CTU is to be used as a source for future intra block copy operations. In the case it is, deblocking and SAO is switched of for all the samples within the CTU. It is asserted that when operating this way, codec implementations do not need to maintain and write samples to two memory buffers (one for output samples and one for intra block copy source samples). It is reported that the approach has a coding efficiency impact of +1.3% and +2.1% for the 444 lossy all intra RGB text&graphics and mixed content categories, respectively.

# Coding efficiency results

Coding efficiency of the proposed approach was measured following the common test condition for screen content coding and the core experiment description. Full results can be find in the attached spreadsheets. The timing measurements were run on a cluster with identical nodes and can be considered reliable.

# Memory bandwidth results

The tables below summarize the memory bandwidth measurements. The measurements were carried out as described in the code experiment definition document.

Table 1: Average memory bandwidth with respect to the anchor

|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lossy | 444 | ai | 74 % | 74 % | 74 % | 74 % | 73 % | 72 % | 99 % | 99 % |
|  |  | ra | 98 % | 98 % | 98 % | 98 % | 98 % | 97 % | 100 % | 100 % |
|  |  | lb | 98 % | 98 % | 98 % | 98 % | 98 % | 97 % | 100 % | 100 % |
|  | 444 | ai | 75 % | 75 % | 74 % | 75 % | 73 % | 72 % | 100 % | 100 % |
|  | /4x1 | ra | 98 % | 98 % | 98 % | 98 % | 98 % | 97 % | 100 % | 100 % |
|  |  | lb | 98 % | 98 % | 98 % | 98 % | 98 % | 97 % | 100 % | 100 % |
|  | 420 | ai | 71 % | 71 % | 70 % | 71 % | 70 % | 69 % | 99 % | 98 % |
|  |  | ra | 98 % | 98 % | 98 % | 98 % | 97 % | 96 % | 99 % | 99 % |
|  |  | lb | 98 % | 99 % | 98 % | 98 % | 97 % | 96 % | 100 % | 100 % |
| Lossless | 444 | ai | 74 % | 74 % | 75 % | 75 % | 76 % | 77 % | 99 % | 99 % |
|  |  | ra | 97 % | 97 % | 97 % | 97 % | 96 % | 95 % | 100 % | 100 % |
|  |  | lb | 97 % | 97 % | 97 % | 97 % | 96 % | 95 % | 100 % | 100 % |
|  | 420 | ai | 71 % | 71 % | 72 % | 73 % | 73 % | 72 % | 98 % | 98 % |
|  |  | ra | 96 % | 97 % | 95 % | 96 % | 94 % | 92 % | 100 % | 100 % |
|  |  | lb | 97 % | 97 % | 96 % | 96 % | 94 % | 92 % | 100 % | 100 % |

Table 2: Maximum memory bandwidth with respect to the anchor

|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lossy | 444 | ai | 75 % | 75 % | 74 % | 74 % | 74 % | 74 % | 98 % | 98 % |
|  |  | ra | 100 % | 99 % | 99 % | 99 % | 89 % | 83 % | 104 % | 101 % |
|  |  | lb | 100 % | 100 % | 100 % | 100 % | 91 % | 97 % | 96 % | 96 % |
|  | 444 | ai | 78 % | 78 % | 75 % | 74 % | 74 % | 74 % | 100 % | 99 % |
|  | /4x1 | ra | 100 % | 100 % | 99 % | 98 % | 91 % | 84 % | 101 % | 100 % |
|  |  | lb | 100 % | 99 % | 98 % | 99 % | 93 % | 97 % | 100 % | 99 % |
|  | 420 | ai | 76 % | 75 % | 71 % | 71 % | 71 % | 71 % | 100 % | 100 % |
|  |  | ra | 101 % | 101 % | 99 % | 97 % | 86 % | 82 % | 104 % | 103 % |
|  |  | lb | 100 % | 99 % | 98 % | 97 % | 88 % | 84 % | 100 % | 99 % |
| Lossless | 444 | ai | 74 % | 74 % | 75 % | 75 % | 77 % | 78 % | 99 % | 99 % |
|  |  | ra | 100 % | 100 % | 95 % | 95 % | 88 % | 86 % | 100 % | 100 % |
|  |  | lb | 96 % | 95 % | 94 % | 94 % | 87 % | 86 % | 99 % | 99 % |
|  | 420 | ai | 75 % | 76 % | 78 % | 79 % | 80 % | 80 % | 101 % | 100 % |
|  |  | ra | 100 % | 97 % | 92 % | 92 % | 87 % | 87 % | 100 % | 99 % |
|  |  | lb | 94 % | 93 % | 92 % | 91 % | 88 % | 87 % | 99 % | 98 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).