| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0035 |
| --- | --- |

| Title: | CE2: Crosscheck of Test 2 on intra block copy constraints on filtering (frame level encoding decisions) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jani Lainema | Tel: Email: | +358 10 4488000 firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution provides a crosscheck for CE2 Test 2 on intra block copy constraints on filtering when using frame level encoding decisions. The intention of the tested method is to reduce both worst case and average memory bandwidth associated with the intra block copy operations. It is reported that the approach has a coding efficiency impact of +0.6% and +1.0% for the 444 lossy all intra RGB text & graphics and mixed content categories, respectively.

# Coding efficiency results

Coding efficiency of the proposed approach was measured following the common test condition for screen content coding and the core experiment description. Full results can be find in the attached spreadsheets. The timing measurements were run on a cluster with identical nodes and can be considered reliable. Results match to those provided by proponents for CE 2 Test 2.

# Memory bandwidth results

The tables below summarize the memory bandwidth measurements. The measurements were carried out using the software provided by the proponents. Also these results match to those provided by the proponents of CE 2 Test 2.

Table 1: Average memory bandwidth with respect to the anchor

|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lossy | 444 | ai | 81 % | 81 % | 82 % | 82 % | 83 % | 84 % | 100 % | 100 % |
|  |  | ra | 74 % | 75 % | 75 % | 76 % | 76 % | 78 % | 100 % | 100 % |
|  |  | lb | 72 % | 72 % | 73 % | 73 % | 74 % | 76 % | 100 % | 100 % |
|  | 444 | ai | 72 % | 72 % | 73 % | 73 % | 74 % | 75 % | 100 % | 100 % |
|  | /4x1 | ra | 74 % | 74 % | 75 % | 75 % | 75 % | 76 % | 100 % | 100 % |
|  |  | lb | 71 % | 71 % | 72 % | 72 % | 73 % | 74 % | 100 % | 100 % |
|  | 420 | ai | 85 % | 85 % | 86 % | 86 % | 87 % | 88 % | 100 % | 100 % |
|  |  | ra | 74 % | 75 % | 75 % | 75 % | 77 % | 78 % | 100 % | 100 % |
|  |  | lb | 71 % | 72 % | 73 % | 73 % | 74 % | 76 % | 100 % | 100 % |
| Lossless | 444 | ai | 80 % | 80 % | 80 % | 80 % | 78 % | 78 % | 100 % | 100 % |
|  |  | ra | 74 % | 74 % | 75 % | 75 % | 75 % | 75 % | 100 % | 100 % |
|  |  | lb | 72 % | 73 % | 73 % | 74 % | 73 % | 74 % | 100 % | 100 % |
|  | 420 | ai | 86 % | 86 % | 86 % | 86 % | 86 % | 86 % | 100 % | 100 % |
|  |  | ra | 74 % | 75 % | 76 % | 76 % | 77 % | 78 % | 100 % | 100 % |
|  |  | lb | 72 % | 73 % | 73 % | 74 % | 75 % | 77 % | 100 % | 100 % |

Table 2: Maximum memory bandwidth with respect to the anchor

|  |  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Lossy | 444 | ai | 88 % | 87 % | 87 % | 87 % | 86 % | 85 % | 100 % | 100 % |
|  |  | ra | 77 % | 79 % | 85 % | 86 % | 88 % | 86 % | 102 % | 101 % |
|  |  | lb | 77 % | 79 % | 83 % | 84 % | 87 % | 88 % | 97 % | 96 % |
|  | 444 | ai | 79 % | 79 % | 79 % | 79 % | 79 % | 78 % | 100 % | 100 % |
|  | /4x1 | ra | 76 % | 78 % | 81 % | 81 % | 81 % | 81 % | 100 % | 101 % |
|  |  | lb | 76 % | 77 % | 79 % | 80 % | 82 % | 81 % | 100 % | 100 % |
|  | 420 | ai | 88 % | 88 % | 88 % | 88 % | 88 % | 88 % | 100 % | 100 % |
|  |  | ra | 80 % | 82 % | 86 % | 87 % | 88 % | 88 % | 101 % | 101 % |
|  |  | lb | 78 % | 80 % | 86 % | 87 % | 88 % | 89 % | 97 % | 98 % |
| Lossless | 444 | ai | 84 % | 84 % | 84 % | 84 % | 82 % | 81 % | 100 % | 100 % |
|  |  | ra | 75 % | 78 % | 84 % | 85 % | 85 % | 84 % | 100 % | 100 % |
|  |  | lb | 81 % | 82 % | 85 % | 86 % | 84 % | 84 % | 100 % | 100 % |
|  | 420 | ai | 85 % | 85 % | 83 % | 83 % | 82 % | 82 % | 100 % | 100 % |
|  |  | ra | 77 % | 85 % | 86 % | 86 % | 84 % | 85 % | 100 % | 100 % |
|  |  | lb | 82 % | 84 % | 86 % | 87 % | 84 % | 84 % | 100 % | 100 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).