| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0037r1 |
| --- | --- |

| Title: | SHVC verification test draft plan |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yan Ye 9710 Scranton Rd, #250 San Diego, CA 92121, USA Hendry, Ye-Kui Wang 5775 Morehouse Drive San Diego, CA 92121-1714, USA | Tel: Email: | + 1-858-210-4807 Yong.He@InterDigital.com Yan.Ye@InterDigital.com fhendry@qti.qualcomm.com yekuiw@qti.qualcomm.com |
| Source: | InterDigital Communications, LLC, Qualcomm Incorporated. |  |  |

_____________________________

# Introduction

This document contains a proposed draft plan for the video verification test to be conducted to verify the coding performance of the SHVC Scalable Main and Scalable Main 10 profiles. It is suggested that a formal subjective evaluation be conducted comparing the SHVC scalable Main and Main 10 profiles to HEVC simulcast using the HEVC Main and Main 10 profiles, respectively. The suggested test covers a range of video resolutions from 540p to 4K, and various scalability cases, including spatial 1.5x, spatial 2x, SNR and color gamut scalabilities.

# Schedule

The following schedule is suggested for the subjective test:

- Test sequences identified: July, 2015

- Encoding of sequences completed: August, 2015

- Subjective evaluation starts: September, 2015 (this aspect particularly needs to be discussed and agreed with the MPEG Test Subgroup chair)

# Test conditions

The following test conditions are suggested to be used for the SHVC verification test:

- Number of sequences and video resolutions:

  - Two sets of sequences (two layers per set) for each scalability category (Spatial 2x, 1.5x, SNR and CGS scalability))

- Bitstreams

  - Generated with SHM 9.0 simulcast functionality for HEVC bitstreams

  - Generated with SHM 9.0 for SHVC bitstreams

- Encoding parameters

  - Fixed QP

    - 4 bit rate points per sequence covering the whole MOS range as much as possible

  - Bit depth

    - 8 bits for spatial 1.5x, 2x and SNR scalabilities

    - 10 bits for color gamut scalability (CGS)

  - Coding structure:

    - Random access (RA)

      - IRAP picture period at approximately 0.5 second

      - Hierarchical B coding structure

    - Low delay (LD)

      - Only the first picture in the bitstream is IRAP picture

      - Without picture reordering

  - Other settings as in the configuration files in SHM9.0 software cfg subdirectory:

    - cfg/layers.cfg, encoder_lowdelay_scalable.cfg and encoder_randomaccess_scalable.cfg for spatial 1.5x, 2x and SNR scalability;

    - cfg/encoder_randomaccess_scalable10.cfg for CGS.

# Proposed test sequences and test cases

The following test sequences are selected for the subjective test, the relative lower resolution sequences are subsampled by zero phase down sampling filter with 0.9π cut-off frequency [1]. Highlighted sequences are not yet available and need to be obtained.

## Spatial 2x scalability

The following test sequences are suggested for the subjective test for spatial 2x case:

| Sequence | Source [Copyright] | Enhancement layer resolution | Base layer resolution | Frame rate (fps) | Bit depth (bits) | Length (frames) | RA / LD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BT709_Parakeet | Technicolor | 1920x1080 | 960x540 | 50 | 8/8 | 500 | RA |
| Jill_Johnny | Vidyo | 1920x1080 | 960x540 | 30 | 8/8 | 300 | LD |

## Spatial 1.5x scalability

The following test sequences are suggested for the subjective test for spatial 1.5x case:

| Sequence | Source [Copyright] | Enhancement layer resolution | Base layer resolution | Frame rate (fps) | Bit depth (bits) | Length (frames) | RA / LD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BT709_Birthday | Technicolor | 1920x1080 | 1280x720 | 60 | 8/8 | 500 | RA |
| Han | Vidyo | 1920x1080 | 1280x720 | 30 | 8/8 | 300 | LD |

## SNR scalability

The following test sequences are suggested for the subjective test for SNR case:

| Sequence | Source [Copyright] | Enhancement layer resolution | Base layer resolution | Frame rate (fps) | Bit depth (bits) | Length (frames) | RA / LD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Missing |  |  |  |  |  |  |  |
| BT709_Parakeets | Technicolor | 1280x720 | 1280x720 | 50 | 8/8 | 500 | RA |

It is proposed that only the RA configuration is tested for SNR scalability, as SNR scalability does not seem to be widely used for video conferencing applications.

## CGS scalability

Similar to SNR scalability, it is proposed that only random access configuration be tested for CGS.

The following test sequences are selected for the subjective test for CGS case:

| Sequence | Source [Copyright] | Enhancement layer resolution | Base layer resolution | Frame Rate (fps) | Bit depth | Length (frames) | RA/LD | Color gamut |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Missing |  |  |  |  |  |  |  |  |
| BT2020_BirthdayLucy | Technicolor | 1920x1080 (BT.2020) | 1920x1080 (BT.709) | 60 | 10/10 | 600 | RA | BT.2020/BT.709 |

# Preliminary encoding results

Using the proposed test cases as listed in Section 3, we ran preliminary encoding and obtained a set of 48 rate points.

Constant QP was used in each encoding run. The QPs were selected to keep the SHVC bitrate ratio between enhancement layer and the base layer consistent for each resolution scalability (2x, 1.5x and SNR) as shown in Table 1, and the same selected QP was used in simulcast simulation.

The preliminary simulations were run independently by both source companies and thus cross checked.

- SHVC bitrate ratio (EL vs. BL)

|  | CGS 10b (BL/EL) | Spatial 8b 2x (BL/EL) | Spatial 8b 1.5x (BL/EL) | SNR 8b (BL/EL) |
| --- | --- | --- | --- | --- |
| Resolutions | 1920x1080/1920x1080 | 960x540/1920x1080 | 1280x720/1920x1080 | 1280x720/1280x720 |
| SHVC EL/BL bitrate ratio | 30%~40% | 90%~100% | 50%~60% | 30%~40% |
| BL bitrate range | 2Mbps ~ 7Mbps | 400kbps ~ 12Mbps (RA) 100kbps ~ 400kbps (LD) | 2Mbps ~ 7.5Mbps (RA) 150kbps ~ 4Mbps (LD) | 600kbps ~ 3Mbps (RA) |
| SHVC EL bitrate range | 900Kbps ~ 2.5Mbps | 400kbps ~ 2Mbps (RA) 70kbps ~ 180kbps (LD) | 1.2Mbps ~ 3.5Mbps (RA) 70kbps ~ 1.5Mbps (LD) | 200kbps ~ 1.2Mbps (RA) |
| Simulcast EL bitrate range | 3Mbps ~ 8Mbps | 800kbps ~ 3.5Mbps (RA) 100kbps ~ 300kbps (LD) | 3Mbps ~ 8Mbps (RA) 200kbps ~ 5Mbps (LD) | 800kbps ~ 4Mbps (RA) |

The following tables are the summary of the coding gain results with the preliminary selected test points. Detailed R-D data can be found in the accompanying spreadsheet.

- Preliminary encoding results (SHVC vs. simulcast)

|  |  |  |  | Reference:Simulcast |  |  |  | Tested:SHM9.0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Enhancement layer |  |  |  | Enhancement layer |  |  |  | Base layer |  |  |  |
|  |  | BL QP | EL QP | kbps | Y psnr | U psnr | V psnr | kbps | Y psnr | U psnr | V psnr | kbps | Y psnr | U psnr | V psnr |
| RA | Parakeets-SNR | 19 | 17 | 3710.88 | 46.90 | 48.96 | 48.63 | 1216.15 | 46.81 | 48.71 | 48.47 | 2837.94 | 46.23 | 48.31 | 48.12 |
|  | 1280x720x50Hz | 23 | 21 | 2207.09 | 45.47 | 47.60 | 47.57 | 684.15 | 45.26 | 47.25 | 47.31 | 1698.63 | 44.57 | 46.83 | 46.96 |
|  | 1280x720x50Hz | 28 | 26 | 1156.86 | 43.05 | 45.60 | 45.98 | 318.96 | 42.72 | 45.21 | 45.63 | 904.32 | 42.00 | 44.83 | 45.32 |
|  |  | 31 | 29 | 803.59 | 41.45 | 44.50 | 45.02 | 205.54 | 41.03 | 44.24 | 44.77 | 631.34 | 40.33 | 44.01 | 44.56 |
|  | Parakeets-2x | 19 | 22 | 3410.33 | 43.47 | 47.29 | 46.76 | 2105.37 | 43.40 | 47.03 | 46.59 | 1976.85 | 46.32 | 48.20 | 48.35 |
|  | 960x540x50Hz | 24 | 26 | 1943.97 | 42.35 | 45.97 | 45.79 | 1141.84 | 42.22 | 45.60 | 45.50 | 1069.70 | 43.75 | 46.07 | 46.59 |
|  | 1920x1080x50Hz | 27 | 29 | 1332.00 | 41.31 | 44.97 | 45.05 | 749.34 | 41.13 | 44.61 | 44.75 | 736.62 | 41.96 | 44.77 | 45.42 |
|  |  | 31 | 33 | 808.53 | 39.61 | 43.78 | 44.01 | 420.69 | 39.39 | 43.35 | 43.64 | 455.41 | 39.56 | 43.48 | 44.17 |
|  | Birthday-1.5x | 19 | 22 | 8353.88 | 44.16 | 45.44 | 43.79 | 3673.71 | 44.05 | 45.28 | 43.59 | 7632.93 | 45.07 | 46.59 | 44.63 |
|  | 1280x720x60Hz | 23 | 25 | 5602.23 | 42.57 | 44.22 | 42.17 | 2337.44 | 42.33 | 43.98 | 41.89 | 4589.73 | 42.76 | 44.84 | 42.45 |
|  | 1920x1080x60Hz | 27 | 29 | 3463.21 | 40.34 | 42.76 | 40.30 | 1279.71 | 40.01 | 42.49 | 39.99 | 2777.03 | 40.25 | 42.93 | 40.26 |
|  |  | 30 | 31 | 2735.57 | 39.18 | 42.16 | 39.52 | 1188.57 | 38.77 | 41.83 | 39.20 | 1965.30 | 38.35 | 41.92 | 39.13 |
| CGS | Lucy-1x | 23 | 22 | 7846.62 | 42.41 | 44.14 | 45.88 | 2514.23 | 42.32 | 44.06 | 46.87 | 6947.46 | 42.25 | 44.05 | 43.04 |
|  | 1920x1080x60Hz | 25 | 24 | 5640.75 | 41.64 | 43.47 | 44.82 | 1594.16 | 41.51 | 43.34 | 45.85 | 5127.26 | 41.47 | 43.25 | 42.02 |
|  | 1920x1080x60Hz | 29 | 27 | 3676.60 | 40.45 | 42.34 | 43.22 | 1229.08 | 40.20 | 42.21 | 44.17 | 3029.92 | 39.80 | 41.73 | 40.24 |
|  |  | 31 | 29 | 2842.46 | 39.60 | 41.63 | 42.28 | 895.77 | 39.30 | 41.61 | 43.38 | 2341.52 | 38.88 | 41.10 | 39.45 |
| LDB | han-1.5x | 19 | 22 | 5293.00 | 41.42 | 48.04 | 48.85 | 1642.28 | 41.32 | 47.80 | 48.71 | 3801.69 | 43.14 | 49.14 | 49.90 |
|  | 1280x720x30Hz | 26 | 28 | 785.19 | 39.85 | 46.12 | 47.07 | 300.38 | 39.79 | 45.90 | 46.87 | 544.46 | 41.06 | 46.49 | 47.46 |
|  | 1920x1080x30Hz | 27 | 29 | 635.96 | 39.64 | 45.96 | 46.96 | 236.91 | 39.58 | 45.70 | 46.68 | 452.50 | 40.76 | 46.08 | 47.05 |
|  |  | 34 | 36 | 210.99 | 37.77 | 43.84 | 44.99 | 70.47 | 37.59 | 43.48 | 44.66 | 158.16 | 38.26 | 43.53 | 44.74 |
|  | jill-2x | 21 | 24 | 707.10 | 44.69 | 46.27 | 48.49 | 414.20 | 44.60 | 46.21 | 48.39 | 447.45 | 46.02 | 48.14 | 49.79 |
|  | 960x540x30Hz | 26 | 29 | 319.35 | 43.34 | 45.52 | 47.57 | 179.22 | 43.18 | 45.44 | 47.38 | 212.32 | 43.88 | 46.58 | 47.85 |
|  | 1920x1080x30Hz | 32 | 35 | 148.10 | 41.02 | 44.41 | 46.30 | 87.77 | 40.82 | 44.18 | 46.01 | 96.12 | 40.68 | 44.55 | 46.17 |
|  |  | 34 | 37 | 117.78 | 40.05 | 44.20 | 45.94 | 72.99 | 39.86 | 43.89 | 45.64 | 75.17 | 39.53 | 43.83 | 45.39 |

- Overall BD rate reduction (SHVC vs. Simulcast)

|  | RA |  |  | CGS |  |  | LDB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Overall (SHVC vs Simulcast (BL+EL)) | -29.2% | -24.7% | -25.0% | -32.6% | -33.8% | -52.9% | -26.9% | -22.2% | -21.7% |
| EL only (SHVC vs Simulcast) | -55.3% | -52.0% | -52.2% | -67.0% | -67.6% | -76.2% | -50.3% | -47.4% | -46.8% |

# Open issues

In order to proceed with the test plan, besides decisions on the above proposed test conditions and test cases, we seek answers to the following open issues:

- Is SVC performance comparison necessary for spatial 1.5x, 2x and SNR scalability? Due to resource limitation, the companies (InterDigital and Qualcomm) currently involved in bitstream generation for verification test prefer not to include comparison with SVC. This (not including SVC) will make it easier to manage the work load in subjective tests down the road. Further, focusing on the comparison with HEVC simulcast seems to be the most relevant from market adoption point of view.

- For the CGS category, there are two main problems being faced by the verification test work:

  - Lack of sequences: Technicolor has kindly donated one test sequence for use in this category. However, this sequence bears some similarity with sequences already used during CGS development.

  - Lack of testing equipment: displays with wide color gamut capabilities are difficult to find in the market place.

- Request for more test sequences. The need for more sequences is particularly acute for the CGS tests. Also, a general question is how many test sequences are needed for each scalability type.

- Test point selection. The preliminary plan outlined above is based on the following method for rate selection: first select suitable BL rates, and then select the EL rates based on a desirable EL rate to BL rate ratio for each type of scalability, finally find QP values that match the desired BL and EL rates. Currently the EL/BL ratios for each case are listed in Table 1. Any suggestions on other values of desirable EL/BL rate ratio are welcome.

# References:

- ISO/IEC JTC1/SC29/WG11 MPEG2012/M24499, “Downsampling filters for anchor generation for scalable extensions of HEVC”, May, 2012.

Annex A

### Copyright of test sequences

- Technicolor R&D

This copyright document relates to the following video content:

BT709_BirthdayLucy_1920x1080_60_10bit_zerophase_0.9pi.yuv

BT2020_BirthdayLucy_3840x2160_60_10bit.yuv

This video content (“Content”) may only be used for the purpose of developing, testing and promulgating technology standards developed by the MPEG, VCEG or JCT-VC standardization groups (“Purpose”), under Technicolor R&D France SNC (“Technicolor”) owned or controlled copyrights, by individual(s) or organization(s) that participates, contributes or is part of such standardization groups (“User(s)”).

RESTRICTIONS: No license whatsoever is implied except from this expressed limited personal, non-sublicensable, non-transferrable authorization. In particular no right under any patent of Technicolor or its affiliates is granted. Any other use is strictly prohibited. This Content cannot be distributed or otherwise disposed of or made available (via internet or otherwise), broadcasted, disclosed to third parties (except to other User(s) who agreed with the present terms and conditions), used for providing any services to third parties or for any commercial use, nor copied

(except as technically necessary for the Purpose), modified, adapted, translated, exchanged, publicly performed (even for demonstration or educational purposes), integrated, without limitation. This Content and all intellectual property rights, titles and interests therein, are and remain the exclusive property of Technicolor or its affiliates.

NO WARRANTY: This Content is supplied "as is" and without any warranty of any kind, expressed or implied, including but not limited to any warranty for accuracy or performance, fitness for a general or particular purposes, or any warranty arising by statute or by law, or non-infringement of any third parties’ rights. In no event will Technicolor be liable for damages of any kind, including without limitation, any special, indirect, incidental, or consequential damages even if Technicolor has been advised of the possibility of such damages.

This authorization is effective as of the date of use of the Content and shall expire on January, 1, 2020. Technicolor may nevertheless terminate it at any time for any reason upon notice to User(s). It is governed by the laws of France without regard to conflict of laws. It shall be exclusively submitted to the first instance civil court of Paris, France (TGI) in the event of a dispute not settled amicably.

BY USING THIS CONTENT USER(S) AGREES AND ACCEPTS THE ABOVE MENTIONED TERMS AND CONDITIONS. IF USER(S) DOES NOT AGREE TO

THESE TERMS AND CONDITIONS, IT SHALL NOT ACCESS OR OTHERWISE USE THIS CONTENT. DO NOT REMOVE OR MODIFY TECHNICOLOR OR

ITS AFFILIATES’ NAMES, SIGNS, LOGOS, NOTICES OF/IN THE CONTENT.

Copyright © 2012-2014 – Technicolor R&D France, SNC

All Rights Reserved

- Vidyo, Inc.

Vidyo donates the sequences to the public domain