| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0043 |
| --- | --- |

| Title: | Deblocking in HEVC: Some observations from the HDR/WCG CfE |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A.M. Tourapis, Y. Su, D. Singer 1 Infinite Loop Cupertino, CA 95014 USA | Tel: Email: | +1-408-228-7983 atourapis@apple.com |
|  | J. Sole, D. Rusanovskyy, M. Karczewicz 5775 Morehouse Drive San Diego, CA 92121-1714 USA |  |  |
|  | C. Fogg |  |  |
|  | A. Duenas, F. Bossen |  |  |
| Source: | Apple Inc., Qualcomm Inc., Movielabs Inc., NGCodec Inc. |  |  |

_____________________________

# Abstract

One of the primary artefacts that have been observed on the Anchor material generated for the MPEG HDR/WCG CfE [4], was the presence of severe blockiness, especially on material with considerable motion and noise. Such blockiness may also impair the performance of different chroma conversion techniques, i.e. for up-converting from 4:2:0 to a 4:4:4 representation, given the additional, artificial, high frequencies, introduced into the content. This contribution suggests that adding an additional chroma only deblocking filter, after decoding and before upconversion to 4:4:4 could considerably improve both the subjective and objective performance of HDR, and not only, material using Main 10.

# Introduction

During the MPEG HDR/WCG activity, it was observed that several of the anchors, and especially video sequences like FireEater, contained considerable blockiness after compression. After some careful analysis, it was observed that these artefacts mainly occurred in the two colour planes (Figure 1-1), and were an outcome of the deblocking behaviour of the HEVC Main-10 profile. More specifically, in the current HEVC standard it is specified that only chroma edges of intra partitions need to be filtered, i.e. edges associated with deblocking filter strength equal to 2. Unlike luma however, edges that correspond to areas with high/very different motion, i.e. deblocking filter strength equal to 1, are not filtered at all. This can potentially result in considerable blocking artefacts in the colour components, especially when the colour planes have much more significant information in them than some of the material used during the HEVC standardization development.

(a)

(b)

Figure 1-1. U (a) and V (b) components of frame 180 for FireEater Anchor with QP29

# Chroma Post-Decode Deblocking

To resolve this issue, we have explored adding a “post-decode” deblocking filter, which mainly targets chroma components, in the reconstruction process. Such a process is shown in Figure 1-2, where the chroma deblocker was added after the inverse quantization process, to take better advantage of the increased precision. Note, however, that adding the deblocker prior to the inverse quantization process is also feasible, but was not tested.

Figure 1-2. Post-Loop Chroma Deblocking for HDR video reconstruction

We have basically implemented a very simple deblocking scheme, which is based on the weak (bS < 4) deblocking strength used in the MPEG-4 AVC specification. This deblocking filter is applied on all chroma edges regardless of their coding mode, and no analysis of the bitstream is performed. Ideally, if such information was available and the bitstream could be analyzed, then this process could be optimized further. For example, one may wish to deploy a scheme very similar to the derivation process of the bS strength in AVC and HEVC as well as consider the QP parameters for the deblocking threshold parameter derivation. A deblocking scheme may also not necessarily be based on strategies employed in HEVC and/or AVC but use very different techniques altogether. However, that is not the intention of this contribution and it is left to different implementers to decide what schemes they may wish to employ for additional deblocking.

Figure 2-2. Anchor QP 29: Reconstruction performance of the Blue component in FireEater (Frame 180)

Figure 2-2 shows the performance of the reconstruction for the Blue component in FireEater without the additional deblocking. Considerably more blockiness and structured patterns can be observed throughout the image, and especially when moving towards the brighter areas. Figure 2-3 presents the reconstructed B color component after the additional deblocking is performed. Figure 2-4, on the other hand, demonstrates the areas that were impacted by the use of this additional deblocker.

Figure 2-3. Anchor QP 29 + deblocking: Performance of the Blue component in FireEater (Frame 180)

Figure 2-4. Difference of deblocked and non deblocked Blue component for FireEater (Frame 180 – QP29)

For this particular sequence, and using a rather “naïve” deblocking implementation, we have managed to achieve tPSNR Bjontegaard gains of -1.2%, -0.7%, and -3.9% for the X, Y, and Z components respectively (Table 2-1). However, what is much more interesting is looking at the per frame results. Averages, unfortunately, tend to considerably mask the true behavior of a scheme. There could be several frames where there may be little if any improvement, whereas there could be some frames that could benefit a lot from such techniques. On the other hand, there could even be cases where a negative impact may be introduced (at least in terms of objective measurements). Such variations can in fact end up being quite visible temporally and may not be properly represented in the average numbers. Therefore, looking at the per-frame results (Figure 2-5) can actually provide us with some additional useful information with regards to using such schemes in our reconstruction process. In this example, we can see that the post-decode deblocker can provide gains of more than 0.16dB in terms of tPSNR on the Z component for some frames. A more intelligent deblocking scheme should likely be able to achieve even more gains than showing in this contribution.

Table 2-1. Average performance of Anchors (A) and Deblocked (D) data

|  |  |  |  | tPSNR |  |  |  |  |  | BD tPSNR |  |  |  | BD | BD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | QP Slice | kbps | X | Y | Z | XYZ | deltaE | mPsnr | X | Y | Z | XYZ | deltaE | mPSNR |
| FireEater | A | 20 | 1921.6 | 45.43 | 47.02 | 44.98 | 45.72 | 42.73 | 32.69 | -1.2% | -0.7% | -3.9% | -1.9% | -3.6% | -5.5% |
|  |  | 23 | 1259.9 | 44.22 | 45.81 | 44.03 | 44.61 | 42.17 | 32.22 |  |  |  |  |  |  |
|  |  | 26 | 811.8 | 42.81 | 44.38 | 42.93 | 43.31 | 41.52 | 31.62 |  |  |  |  |  |  |
|  |  | 29 | 520.7 | 41.42 | 42.90 | 41.84 | 42.00 | 40.92 | 30.95 |  |  |  |  |  |  |
|  | D | 20 | 1921.6 | 45.465 | 47.042 | 45.083 | 45.781 | 42.786 | 32.771 |  |  |  |  |  |  |
|  |  | 23 | 1259.9 | 44.262 | 45.837 | 44.13 | 44.672 | 42.22 | 32.297 |  |  |  |  |  |  |
|  |  | 26 | 811.8 | 42.846 | 44.401 | 43.021 | 43.364 | 41.57 | 31.7 |  |  |  |  |  |  |
|  |  | 29 | 520.7 | 41.456 | 42.919 | 41.916 | 42.049 | 40.964 | 31.02 |  |  |  |  |  |  |

Figure 2-5. Anchor QP 29: Reconstruction performance of the Blue component in FireEater (Frame 180)

# Recommendation

It is suggested that organizations and/or individuals implementing HEVC decoding solutions consider adding an optional post-decode deblocking engine, primarily targeting chroma components, in their implementations. Without advocating any particular deblocking design, having the ability to potentially use a different deblocker in non-reference pictures, than the one dictated by HEVC, and which might be a bit more aggressive on chroma samples, might also be a good solution.  Such recommendation applies to not only HDR decoding applications but also to any application that may utilize the HEVC video standard.

# Conclusion

This contribution suggests that the consideration of additional deblocking mechanisms, that target mainly the chroma components, after HEVC decoding, could potentially result in an improved subjective as well as objective performance. It is therefore recommended that such mechanisms are potentially considered in future applications that may utilize the HEVC standard.

# References

- ISO/IEC 230082:2014 HEVC Second Edition

- Recommendation ITU-R BT. 2020, “Parameter values for ultra-high definition television systems for production and international programme exchange” (2012).

- SMPTE ST 2084:2014, “High Dynamic Range Electro-Optical Transfer Function of Mastering Reference Displays”, 2014.

- Call for Evidence (CfE) for HDR and WCG Video Coding, MPEG document M36131, Switzerland, Feb.2015

# Patent rights declaration(s)

Apple Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Movielabs Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NGCodec Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

It should be noted, however, that Frame average numbers may also mask region behavior. An image may, for example, have very good quality in one region and rather poor quality in another, which would have a considerably effect in the overall subjective quality.