| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0044r3 |
| --- | --- |

| Title: | HEVC and AVC coding results of noise-reduced 4:2:0 and 4:4:4 HDR/WCG video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) | K. Slavin, isovideo C. Fogg, MovieLabs | Email: | keith@isovideo.com |
| Source: | isovideo LLC, Motion Picture Laboratories Inc. |  |  |

_____________________________

# Abstract

Video clips being studied in the HDR Call for Evidence (CfE) have been noise reduced prior to HEVC encoding in order to provide a visual comparison with the same clips coded without a noise reduction pre-processing step. No changes were applied to the HEVC encoding or decoding steps in this test. Detail and sharpness were maintained by the 16-bit per sample motion-compensated Viarte noise reduction processing from isovideo. Playback of the bitstreams will be available on reference monitors at the Warsaw meeting. Coding results (attached as an .xls spreadsheet) of low noise and noise-reduced CfE content show that the 4:4:4 coded bitrate overhead compared to 4:2:0 is less than 10% for the same Qp target. Slides included in the JCTVC-U0044-v5.zip archive compare different metrics to estimate noise levels, and provide example original vs. noise-reduced image patches.

# Introduction

Equivalent HEVC coding parameters described by the HDR Call for Evidence [1] were applied in the study to provide approximately equal conditions for both original and noise reduced signals.  Viarte motion-compensated filter from isovideo [2] was applied to the 12-bit PQ BT.2020 YCbCr Non-constant luminance 4:4:4 format source video. The output of the noise reducer was then converted to 4:2:0 10-bit format ingested by the Main10 HEVC codecs used in this study. For each input format, both CQP (Constant Qp) and CRF (Constant Rate Factor) runs were performed. Playback of bitstreams will be available on a SIM2 professional HDR monitor in order to show the gains provided by this high accuracy, quality optimized pre-processing.

# Discussion

All noise reduction processing on the test clips was done completely automatically. The objective was to achieve bitrate reduction while maintaining fine detail/sharpness and high contrast.

Of the nine HDR test clips, there are three more noisy ones - “AutoWelding”, “BikeSparklers” and “ShowGirl”, and three intermediate noisy clips - “Magic”, “WarmNight” and “FireEater”, and three relative low-noise clips - “BalloonFestival”, “Market” and “Tibul”.

Our analysis of Constant QP 420 (see 2.1 Tables/Figures below) runs show that denoised compressed versions of the nine sequences either gain on PSNR for a give bitrate, and/or bitrate reduction at a given PSNR. More specifically:

- For a give Y-PSNR(db), the denoised versions achieve siginificant bitrate reductions. For example, the “AutoWelding”, “BikeSparklers” and “ShowGirl” see bitrate reductions of about 13x, 8x, and 5x, respectively at 47db PSNR. “BikeSparklers” also achieves about 4.5x bitrate reduction at 42db PSNR. The other three relative noisy clips “Magic”, and “WarmNight” see about 3x reduction at 47db PSNR. The “fireEater” clip sees about 7x reduction at 52db PSNR.

- For a given bitrate, denoised compressed version gains significant PSNR. For examples, the “AutoWelding”, “BikeSparklers” and “ShowGirl” gains at 3Mbps of about 5db, 3db and 3.5db PSNR, respectively. The other three clips “Magic”, “WarmNight” and “FireEater” gain about 2.5db PSNR also at 3Mbps.

- For the relative low-noise sequences such as “BalloonFestival”, “Market” and “Tibul” no significant bitrate reduction is achieved below 47db PSNR, and only a small reduction above 47db PSNR.

Similar results were obtained for CQP 444, and CRF 420 and 444.

A longer version of this paper is available at http://isovideo.com/jct-vc-denoise_2015.php which also includes AVC results as well as some additional test clips from public domain sources. Any further work will be updated there as well.

# Conclusions

Motion-compensated noise reduction prior to HEVC encoding can be very effective in reducing bitrates and/or increasing PSNR, for a more immersive viewing experience on sources with moderate noise levels (either shot noise or film grain), while maintaining fine detail and sharpness. This type of processing can also be used with a target compression bitrate control to improve visual quality unless there is no noise in the input at all.

# References

[1] m15083, Call for Evidence (CfE) for HDR and WCG Video Coding, Geneva, Feb. 2015

[2] http://isovideo.com