| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0047 |
| --- | --- |

| Title: | HDR Workflow precision and steps |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Fogg MovieLabs 475 Sansome Street, Suite 740 San Francisco, CA 94111 | Email: | chadfogg@gmail.com |
| Source: | Motion Picture Laboratories, Inc. |  |  |

_____________________________

# Abstract

Reported deficiencies with transfer functions may instead be actually caused by content source noise and insufficient process precision in the workflow stages (such as grading) prior to the application of the transfer function to the linear light video signal in some cases, while in other cases, the artifacts arise from stages after the OETF (such as color conversion and chroma difference signal subsampling) but before video encoding the video signal into, for example, AVC or HEVC bitstreams. It is also noted that the performance of one transfer function vs. another is most often predicted by the statistics of each video signal (such as the Probability Distribution Function or histogram), particularly how well the PDF or histogram is matched to the code densities determined by the slope of the OETF (Optical to Electrical Transfer Function).

# Introduction

Source video noise can be estimated without measurement by knowing the difference between the potential dynamic range expressed by the transfer function and bit-depth (such as 10-bit PQ SMPTE ST 2084) and the dynamic range of the electronic camera sensor or film stock.  A majority of the HDR Call for Evidence (CfE) test clips [1] have been captured by sensors or film with a maximum dynamic range of 14 f-stops.  Yet the 10-bit PQ container has a linear light rage of 28 f-stops, calculated by the formula:

Log2( max_light_level / min_light_level )

For 10-bit PQ, the minimum light level above infinite black (which corresponds to 10-bit narrow/legal code level 65, or code level 1 when VUI video_full_range_flag =1) is 0.00005 candelas/meter square (cd/m^2), a.k.a. nits. The maximum light level is generally assumed to be 10,000 cd/m^2 (nits) regardless of bit-depth. Therefore, the potential dynamic range of 10-bit PQ is log2( 10,000 / 0.00005) or approximately 28 f-stops.  Any linear light workflow processing would therefore at least have 28 f-stop’s worth of range, such as 32-bit integer or 16-bit half-float.  Nonetheless, 16-bit half float would introduce occasional saw-tooth quantitation noise from DNORM exponent adjustment steps every time a video signal ramp crosses a DNORM boundary set by the 10-bit mantissa range.

Workflow stages should therefore maintain 32-bits throughout all stages:

Figure 1-1 Workflow stages and precision

The relationship between linear light f-stop and 10-bit PQ bit is illustrated in the Figure 1-2 (generated with Matlab):

Figure 1-2 Alignment between 10-bit PQ and linear f-stop light

To avoid excessive noise levels, dark scenes would captured with a camera set to a wide aperture lens such as depicted in Figure 1-3 below:

Figure 1-3  Example dark scene capture with 14 f-stop sensor or film stock

However, dark scenes in the CfE clips such as StEM “WarmNight” and Dolby “AutoWelder” and “Bike Sparklers” have pixel values that range from approximately 4000 nits to near 0 nits (thus approximately 26 bits of f-stop range), captured with a camera set to a narrower aperture such as depicted in the following figure:

Figure 1-4  Bright scene capture

The alignment of the noise floor, occupying f-stop range f-15 to f-4, in Figure 1-4 with 10-bit PQ suggests the lower 6 least significant bits of 10-bit PQ will contain noise.  The measurement method of JCTVC-U0044 [2] shows a pattern of occasional uncorrelated original – de-noised pixel differences up to code level 60, yet the difference signal has a standard deviation of approximately 5, corresponding to 2.3 bits of average noise.  Ideally, noise should be limited to 0.5 bits average.

It should be noted that in contribution JCTVC-U0044 [2], the Market HDR test clip measured more than 14 f-stops of dynamic range but little noise was indicated by the motion compensated difference method due to perhaps the computational photography algorithms that were applied to combine multiple simultaneous exposures from a dual camera rig in post-processing for that clip, and subsequent re-grading operations to improve the utility of that clip for the CfE.  In single-exposure capture of bright scenes with a 14 f-stop sensor, dark shadow areas would otherwise be expected to be filled with noise.

14 f-stop dynamic range response

Figure 1-5  Kodak Vision3 5219 color negative movie film Sensiometric curve

# References

[1] Call for Evidence

[2] JCTVC-U0044