| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0048 |
| --- | --- |

| Title: | Output Code Map SEI for fractional internal bit-depth increase |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg, MovieLabs 475 Sansome Street, Suite 740 San Francisco, CA 94111 | Email: | chadfogg@gmail.com |
|  | Bill Mandel Universal City Studios LLC 100 Universal City Plaza Universal City, CA 91608 |  | bill.mandel@nbcuni.com |
|  | Alexis Michael Tourapis Yeping Su David Singer 1 Infinite Loop, Cupertino, CA, 95014, U.S.A |  | atourapis@apple.com yeping_su@apple.com singer@apple.com |
| Source: | Motion Picture Laboratories Inc., Universal City Studios LLC, Apple Inc. |  |  |

_____________________________

# Abstract

This proposal requests a new static metadata SEI “Output Code Map” (OCM) to map the output decoded bitstream sample code levels to code levels of potentially higher, but fixed precision code level range with uniform steps. The main goal of this SEI is to insure that a bitstream maximizing the code levels range of a lesser bitdepth always map to predefined levels of a greater bitdepth understood by equipment, connected, for example, by HDMI. A common use would be to map 10-bit full range (video_full_range_flag=1) decoded output samples to fixed, external 12-bit legal range video samples (video_full_range_flag =0). The application of the OCM SEI could be performed during the decoded picture output process of HEVC (described in Annex C), or performed as an external process outside the scope of the HEVC specification decoder if the OCM SEI metadata is also made available to that external process.

# Introduction

OCM map has some similarities to Colour Remapping Information  (CRI) SEI in that a look up table (LUT), one separate table for each of the three {Y,Cb,Cr} components, directly translates decoded sample code levels to output levels indicated by table entries downloaded in the CRI SEI.   CRI however is limited to 33 pivots and does not specify a precise interpolation process between the pivots. For example, out of 33 pivots, the input map values for 4129 code levels in 12-bit video would have to be interpolated, such as by a linear curve.

Except for the bitdepth of the video (conveyed by sequence parameter set (SPS) BitDepth_Y and BitDepth_C), luminance at unity, and range (video usability information video_full_range_flag) which are overridden where applicable for external communication downstream of the decoder, the OCM SEI neither changes nor overrides the container definition of video signal such as colour_primaries, transfer_function, and matrix_coeffs.

# Proposed syntax

| output_code_map ( payloadSize ) { | Descriptor |
| --- | --- |
| ocm_id | ue(v) |
| ocm_cancel_flag | u(1) |
| if( !ocm_ cancel_flag ) | u(1) |
| { |  |
| ocm_persistence_flag | u(1) |
| ocm_num_tables | u(8) |
| for( c = 0; c < 3; c++ ) { |  |
| ocm_cc_table_ptr[ c ] | u(8) |
| } |  |
| for( t= 0; t < ocm_num_tables; t++ ) { |  |
| ocm_table_id[ t ] | u(8) |
| ocm_input_bitdepth[ ocm_table_id[t]] | u(8) |
| ocm_output_bitdepth[ ocm_table_id[t] ] | u(8) |
| ocm_input_zero[ ocm_table_id[t] ] | u(16) |
| ocm_output_zero[ ocm_table_id[t] ] | u(16) |
| ocm_input_unity[ ocm_table_id[t] ] | u(16) |
| ocm_output_unity[ ocm_table_id[t] ] | u(16) |
| ocm_input_luminance_at_unity[ ocm_table_id[t] ] | u(16) |
| ocm_output_luminance_at_unity[ ocm_table_id[t] ] | u(16) |
| ocm_interpolation_method[ ocm_table_id[t] ] | u(8) |
| ocm_num_deltas[ ocm_table_id[t] ] | u(16) |
| if( ocm_num_deltas[ ocm_table_id[t] > 0 ){ |  |
| ocm_init_delta[ ocm_table_id[t] ] | e(v) |
| for( i= 0; i< ocm_num_deltas[ ocm_table_id[t] ]; i++ ) { |  |
| ocm_delta[ ocm_table_id[t] ][i]; | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Proposed semantics

The color component index, c, is 0 for Y, 1 for Cb, and 2 for the Cr color plane.

ocm_id contains an identifying number that may be used to identify the purpose of the output code map information. The value of ocm_id shall be in the range of 0 to 232 − 2, inclusive.

Values of ocm_id from 0 to 255, inclusive, and from 512 to 231 − 1, inclusive, may be used as determined by the application. Values of ocm_id from 256 to 511, inclusive, and from 231 to 232 − 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all output code map information SEI messages containing a value of ocm_id in the range of 256 to 511, inclusive, or in the range of 231 to 232 − 2, inclusive, and bitstreams shall not contain such values.

ocm_cancel_flag equal to 1 indicates that the output code map information SEI message cancels the persistence of any previous output code map information SEI message in output order that applies to the current layer. ocm_cancel_flag equal to 0 indicates that output code map information follows.

ocm_persistence_flag specifies the persistence of the output code map SEI message for the current layer.

ocm_persistence_flag equal to 0 specifies that the output code map information applies to the current picture only.

Let picA be the current picture. ocm_persistence_flag equal to 1 specifies output code map information persists for the current layer in output order until either of the following conditions is true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture picB in the current layer in an access unit containing a output code map information SEI message with the same value of ocm_id and applicable to the current layer is output for which PicOrderCnt( picB ) is greater than PicOrderCnt( picA ), where PicOrderCnt( picB ) and PicOrderCnt( picA ) are the PicOrderCntVal values of picB and picA, respectively, immediately after the invocation of the decoding process for picture order count for picB.

ocm_num_tables  indicates the number of output code delta tables contained within the current SEI.

ocm_cc_table_ptr[ c ] indicates the ocm_table applied for the color component c.

ocm_table_id[ t ]  indicates the OCM table number.

ocm_input_bitdepth[ ocm_table_id[t] ]  indicates the input OCM map bit depth of the table ocm_table_id[t]. ocm_input_bitdepth[ ocm_table_id[t] ]  shall be between 8 and 16. If the value of ocm_input_bitdepth[ ocm_table_id[t] ] differs from the bitdepth indicated in the sequence parameter for the applied color plane, then the bitdepth samples from that color plane shall be increased or decreased to match ocm_input_bitdepth[ ocm_table_id[t] ] when applied in the OCM map process.

ocm_output_bitdepth[ ocm_table_id[t] ]  indicates the output OCM map bit depth of the table ocm_table_id[t]. ocm_output_bitdepth[ ocm_table_id[t] ] shall be between 8 and 16.

ocm_input_zero[ ocm_table_id[t] ]  indicates the input code level that corresponds to the real value zero of the input samples to the OCM map process for ocm_table_id[t].

ocm_output_zero[ ocm_table_id[t] ]  indicates the output code level that corresponds to the real value zero of the output samples from the OCM map process for ocm_table_id[t].

ocm_input_unity[ ocm_table_id[t] ]  indicates the input code level that corresponds to the real value 1.0 (unity) of the input samples to the OCM map process for ocm_table_id[t].

ocm_output_unity[ ocm_table_id[t] ]  indicates the output code level that corresponds to the real value 1.0 (unity) of the output samples from the OCM map process for ocm_table_id[t].

ocm_input_luminance_at_unity[ ocm_table_id[t] ] indicates the luminance in units of 1 candela per meter square of the input samples at unity to the OCM map process for ocm_table_id[t].

ocm_output_luminance_at_unity[ ocm_table_id[t] ] indicates the luminance in units of 1 candela per meter square of the output samples at unity to the OCM map process for ocm_table_id[t].

ocm_interpolation_method[ ocm_table_id[t] ] indicates the interpolation method applied in filling entries of ocm_lut[ ocm_table_id[t] ]. The default value of any ocm_interpolation_method[ ocm_table_id [t] ] is zero.

When ocm_interpolation_method[ ocm_table_id[ t ] ]  equals to 0, the direct linear map is applied on the equivalent primaries code levels, as derived by sequence parameter set and video usability information, according to the boundaries established by ocm_input_bitdepth[ ocm_table_id [t] ], ocm_input_zero[ ocm_table_id[ t ] ], ocm_output_zero[ ocm_table_id[ t ] ]  and ocm_output_bitdepth[ ocm_table_id[ t ] ], ocm_output_zero[ ocm_table_id[ t ]],  ocm_output_unity[ ocm_table_id[ t ] ].

j = ocm_table_id [t]

n = 1<<ocm_input_bitdepth[ j ]

num_input_codes = (ocm_input_unity[ j ] – ocm_input_zero[ j ] + 1)

num_output_codes = (ocm_output_unity[ j ] – ocm_output_zero[ j ] + 1)

for( l=0; l<n; l++){

real_value_V = ( l – ocm_input_zero[ j ] ) / num_input_codes

ocm_lut[ j ][l] = (real_value_V * num_output_codes) + ocm_output_zero[ j ]

}

The equations above utilize fractional (floating point) precision. The u(16) data types are first converted to such fractional precision prior to the equation, while the entries in ocm_lut are converted to integer. The indexes to all tables are integer. Depending upon video usability information, mapping can either be performed on primary signals using appropriate adjustments of levels established by the zero and unity ratios, or the coded video derived from primaries as per matrix_coeffs, and offsets above plus any color space transformation equations.

When ocm_interpolation_method equals 1, the input code level that is mapped to the output code level for the current table t is specified by the following formula:

j = ocm_table_id[ t ]

n = 1<<ocm_input_bitdepth[ j ]

output_level= 0

output_level_delta = ocm_init_delta[ j ]

for( l=0; l<n; l++){

output_level_delta += ocm_delta[ j ][ l ]

output_level += output_level_delta

ocm_lut[ j ][ l ] = output_level

}

ocm_init_delta[ ocm_table_id[t] ] is the initial delta value in the OCM LUT design process.

ocm_num_deltas indicates the number of delta table entries for the current delta table .

ocm_delta[ ocm_table_id[ t ]] ][ i ]  specifies a difference between code levels. The application of ocm_delta[ ocm_table_id[ t ] ][ i ] depends upon ocm_interpolation_method[ ocm_table_id[ t ] ].

# Proposed process

Once ocm_lut[ ocm_table_id[ t ] ][ l ]  is reconstructed, decode output samples from the HEVC decoding processing may be mapped by inputting to the following OCM map process:

output_ocm_sample = ocm_lut[ ocm_cc_table_ptr[ c ] ][ input_ocm_sample ]

where c is the color component plane of the input_ocm_sample to the OCM map process.  c==0 is Y, c==1 is Cb, c==2 is Cr.

Devices utilizing OCM SEI can apply their own interpolation process, guided in part by the OCM syntax information and additional content analysis, to execute smoother mapping if desired.

# Patent rights declaration(s)

Motion Pictures Laboratories Inc. does not have any current or pending patent rights relating to the technology described in this contribution.

Universal City Studios LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).