| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0049r1 |
| --- | --- |

| Title: | Wiener filter for noise reduction pre-processing of HDR video signals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A.M. Tourapis 1 Infinite Loop Cupertino, CA 95014 C. Fogg | Tel: Email: | +1-408-228-7983 atourapis@apple.com chadfogg@gmail.com |
| Source: | Apple Inc., MovieLabs Inc. |  |  |

_____________________________

# Abstract

A noise reduction Wiener has been added to a beta version of the HDRTools C++ source software program being utilized in the HDR/WCG adhoc study group.  The filter is currently intended to operate directly upon .EXR files (half-float samples), typically containing linear light domain video RGB primary video signals.

# Introduction

To enable the Wiener noise-reduction filter (based on the wiener2  algorithm in Matlab Image Processing Toolbox) in HDRTools [1], add the following line to a .cfg:

UseWienerFiltering=1

A second, separable filter is depicted in the figure below, with taps [+1.0,   +14.0,  +62.0,  +102.0,  +62.0,  +14.0,   +1.0]/256.  To enable the filter in HDRConvert tool, set the following lines in the .cfg file:

Use2DSepFiltering=1

2DSepFilteringMode=1

Figure 1-1  Separate filter in HDRConvert tool

# References

[1] m35471, A. Tourapis, D. Singer, “HDRTools: Software Updates”, Lausanne, Switzerland MPEG HDR/WCG Adhoc meeting, December 2014.