| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0050 |
| --- | --- |

| Title: | CE1 Test B : Simplification for index map coding in palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim, Jing Ye, PoLin Lai, Shan Liu, Shawmin Lei 2860 Junction Ave, San Jose, CA, USA | Tel: Email: | Shawmin Lei +886-3-5670766 ext. 25555 { jungsun.kim, jing.ye, polin.lai, shan.liu, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

CE1 Test B proposed to remove condition checks for index adjustment. In this contribution, 3 different options were tested. In Test 1, index adjustment was removed when previous pixel has COPY_ABOVE mode. In Test 2, index adjustment was removed when previous pixel has COPY_INDEX mode. Test 3 removes index adjustment for both cases. Since Test 1 and Test 2 limit index adjustment based on previously decoded run_type, run_type should be signalled before grouped indices are signalled. Test 1 shows average 0.1%, 0.0%, 0.0% loss under AI, RA and LB configuration respectively. Test 2 shows average 0.3%, 0.2%, 0.2% loss under AI, RA and LB configuration respectively. Test 3.1 which doesnot signal run_type before grouped indices shows average 0.3%, 0.1%, 0.1% loss under AI, RA and LB configuration respectively and Test 3.2 which signals run_type before grouped indices shows maximum 0.3%, 0.2%, 0.1% loss under AI, RA and LB configuration respectively.

# Introduction

For palette index which indicates the index of current pixel and consecutive ‘palette_run’ pixels, index adjustment is applied. The index of current pixel (red pixel in Figure 1(a)) cannot be same as above pixel’s (grey pixel in Figure 1 (a)) if the previous pixel has copy above mode (Figure 1 (a)). Also the index of current pixel (red pixel in Figure 1 (b)) cannot be same as previous pixel’s (grey pixel in Figure 1 (b)) if the previous pixel has copy index mode (Figure 1 (b)).

| (a) previous pixel has copy above mode | (b) previous pixel has copy index mode |
| --- | --- |

Figure 1 examples of index adjustment

This property can reduce the number of index candidates by one if the reference pixel (grey pixels in Figure 1) is not ESCAPE and the current pixel is not the first pixel of the block.

# Proposed Method

## Removing index adjustment when previous pixel has COPY_ABOVE

It is proposed to remove index adjustment when previous pixel has COPY_ABOVE (Figure 1 (a)) to avoid referring index of a pixel on the above line. Since it has to check if the previous pixel has COPY_ABOVE, palette_run_type_flag should be signalled before grouped indices signalled.

WD is changed as follows:

palette_coding( x0, y0, nCbS ) {. …. if( MaxPaletteIndex > 0) {. palette_transpose_flag. num_palette_indices_idc. last_palette_run_type_flag. k=0. for( i=0; i < NumPaletteIndices-1; i++ ) {. if(i>0). palette_run_type_flag. palette_run_type_flag[k++] = palette_run_type_flag. if(palette_run_type_flag == COPY_ABOVE_MODE). Palette_run_type_flag[k++] = COPY_INDEX_MODE. }. if(Palette_run_type_flag[k-1]!=last_palette_run_type_flag). Palette_run_type_flag[k] = last_palette_run_type_flag. for( i=0; i < NumPaletteIndices; i++ ) {. palette_index_idc. PaletteIndexIdc[ i ] = palette_index_idc. }. last_palette_run_type_flag. }. CurrNumIndices = 0. PaletteScanPos = 0. while( PaletteScanPos < nCbS * nCbS ) {. …. if( MaxPaletteIndex > 0 && CurrNumIndices < NumPaletteIndices ) {. if( PaletteScanPos >= nCbS && palette_run_type_flag[ xcPrev ][ ycPrev ] != COPY_ABOVE_MODE && PaletteScanPos < nCbS * nCbS – 1) {. palette_run_type_flag[ xC ][ yC ]. }. …. }. runPos = 0. k=0. while ( runPos < = paletteRun ) {. xR = x0 + travScan[ PaletteScanPos ][ 0 ]. yR = y0 + travScan[ PaletteScanPos ][ 1 ]. if(palette_run_type_flag[k++][ xC ][ yC ] = = COPY_INDEX_MODE ) {. PaletteSampleMode[ xR ][ yR ] = COPY_INDEX_MODE. PaletteIndexMap[ xR ][ yR ] = CurrPaletteIndex. } else {. PaletteSampleMode[ xR ][ yR ] = COPY_ABOVE_MODE. PaletteIndexMap[ xR ][ yR ] = PaletteIndexMap[ xR ][ yR − 1 ]. }. runPos++. PaletteScanPos++. }. }. }. palette_index_idc is an indication of an index to the array represented by currentPaletteEntries. The value of palette_index_idc shall be in the range of 0 to adjustedIndexMax, inclusive. When palette_index_idc is not present, it is inferred to be equal to 0. The variable PaletteIndexMap[ xC ][ yC ] specifes a palette index, which is an index to the array represented by CurrentPaletteEntries. The array indices xC, yC specify the location ( xC , yC ) of the sample relative to the top-left luma sample of the picture. The value of PaletteIndexMap[ xC ][ yC ] shall be in the range of 0 to indexMax, inclusive. The variable adjustedRefIndexMax is derived as follows: adjustedRefIndex = indexMax + 1 if( scanPos > 0 ) { if( palette_run_type_flag[xcPrev][ycPrev] != COPY_ABOVE_MODE ) adjustedRefIndex = PaletteIndexMap[xcPrev][ycPrev] (7-1) else adjustedRefIndex = PaletteIndexMap[xC][yC − 1] When palette_run_type_flag[ xC ][ yC ] is equal to COPY_INDEX_MODE, the variable paletteIndex is derived as follows: paletteIndex = palette_index_idc if( palette_index_idc >= adjustedRefIndex ) paletteIndex = palette_index_idc + 1

## Removing index adjustment when previous pixel has COPY_INDEX

It is proposed to remove index adjustment when previous pixel has COPY_INDEX mode (Figure 1 (b)) to avoid referring previous pixel’s index. Since it has to check if previous pixel has COPY_INDEX, palette_run_type_flag should be signalled before grouped indices signalled.

WD is changed as follows:

palette_coding( x0, y0, nCbS ) {. …. if( MaxPaletteIndex > 0) {. palette_transpose_flag. num_palette_indices_idc. last_palette_run_type_flag. k=1. for(i=0 ; i < NumPaletteIndices-1 ; i++) {. if(i>0). palette_run_type_flag. palette_run_type_flag[k++] = palette_run_type_flag. if(palette_run_type_flag == COPY_ABOVE_MODE). Palette_run_type_flag[k++] = COPY_INDEX_MODE. }. if(Palette_run_type_flag[k-1]!=last_palette_run_type_flag). Palette_run_type_flag[k] = last_palette_run_type_flag. for( i=0; i < NumPaletteIndices; i++ ) {. palette_index_idc. PaletteIndexIdc[ i ] = palette_index_idc. }. last_palette_run_type_flag. }. CurrNumIndices = 0. PaletteScanPos = 0. while( PaletteScanPos < nCbS * nCbS ) {. …. if( MaxPaletteIndex > 0 && CurrNumIndices < NumPaletteIndices ) {. if( PaletteScanPos >= nCbS && palette_run_type_flag[ xcPrev ][ ycPrev ] != COPY_ABOVE_MODE && PaletteScanPos < nCbS * nCbS – 1) {. palette_run_type_flag[ xC ][ yC ]. }. …. }. runPos = 0. k=0. while ( runPos < = paletteRun ) {. xR = x0 + travScan[ PaletteScanPos ][ 0 ]. yR = y0 + travScan[ PaletteScanPos ][ 1 ]. if(palette_run_type_flag[k++][ xC ][ yC ] = = COPY_INDEX_MODE ) {. PaletteSampleMode[ xR ][ yR ] = COPY_INDEX_MODE. PaletteIndexMap[ xR ][ yR ] = CurrPaletteIndex. } else {. PaletteSampleMode[ xR ][ yR ] = COPY_ABOVE_MODE. PaletteIndexMap[ xR ][ yR ] = PaletteIndexMap[ xR ][ yR − 1 ]. }. runPos++. PaletteScanPos++. }. }. }. palette_index_idc is an indication of an index to the array represented by currentPaletteEntries. The value of palette_index_idc shall be in the range of 0 to adjustedIndexMax, inclusive. When palette_index_idc is not present, it is inferred to be equal to 0. The variable PaletteIndexMap[ xC ][ yC ] specifes a palette index, which is an index to the array represented by CurrentPaletteEntries. The array indices xC, yC specify the location ( xC , yC ) of the sample relative to the top-left luma sample of the picture. The value of PaletteIndexMap[ xC ][ yC ] shall be in the range of 0 to indexMax, inclusive. The variable adjustedRefIndexMax is derived as follows: adjustedRefIndex = indexMax + 1 if( scanPos > 0 ) { if( palette_run_type_flag[xcPrev][ycPrev] != COPY_ABOVE_MODE ) adjustedRefIndex = PaletteIndexMap[xcPrev][ycPrev] (7-2) else if( palette_run_type_flag[xcPrev][ycPrev] == COPY_ABOVE_MODE ) adjustedRefIndex = PaletteIndexMap[xC][yC − 1] When palette_run_type_flag[ xC ][ yC ] is equal to COPY_INDEX_MODE, the variable paletteIndex is derived as follows: paletteIndex = palette_index_idc if( palette_index_idc >= adjustedRefIndex ) paletteIndex = palette_index_idc + 1

## Removing index adjustment

It is proposed not to apply index adjustment at all to avoid dependency on other syntaxes.

### Without signalling run_type before grouped indices

Since index adjustment is applied to none of cases, previous pixel has COPY_INDEX nor previous pixel has COPY_ABOVE, it is not needed to check run_type of previous pixel. Therefore, it doesnot need to signal run_type before grouped indices.

palette_index_idc is an indication of an index to the array represented by currentPaletteEntries. The value of palette_index_idc shall be in the range of 0 to adjustedIndexMax, inclusive. When palette_index_idc is not present, it is inferred to be equal to 0. The variable PaletteIndexMap[ xC ][ yC ] specifes a palette index, which is an index to the array represented by CurrentPaletteEntries. The array indices xC, yC specify the location ( xC , yC ) of the sample relative to the top-left luma sample of the picture. The value of PaletteIndexMap[ xC ][ yC ] shall be in the range of 0 to indexMax, inclusive. The variable adjustedRefIndexMax is derived as follows: adjustedRefIndex = indexMax + 1 if( scanPos > 0 ) { if( palette_run_type_flag[xcPrev][ycPrev] != COPY_ABOVE_MODE ) adjustedRefIndex = PaletteIndexMap[xcPrev][ycPrev] (7-3) else adjustedRefIndex = PaletteIndexMap[xC][yC − 1] When palette_run_type_flag[ xC ][ yC ] is equal to COPY_INDEX_MODE, the variable paletteIndex is derived as follows: paletteIndex = palette_index_idc if( palette_index_idc >= adjustedRefIndex ) paletteIndex = palette_index_idc + 1

### With signalling run_type before grouped indices

Also, run_type can be signaled before signalling grouped palette indices as 2.1 and 2.2.

WD is changed as follows:

palette_coding( x0, y0, nCbS ) {. …. if( MaxPaletteIndex > 0) {. palette_transpose_flag. num_palette_indices_idc. last_palette_run_type_flag. k=1. for(i=0 ; i < NumPaletteIndices-1 ; i++) {. if(i>0). palette_run_type_flag. palette_run_type_flag[k++] = palette_run_type_flag. if(palette_run_type_flag == COPY_ABOVE_MODE). Palette_run_type_flag[k++] = COPY_INDEX_MODE. }. if(Palette_run_type_flag[k-1]!=last_palette_run_type_flag). Palette_run_type_flag[k] = last_palette_run_type_flag. for( i=0; i < NumPaletteIndices; i++ ) {. palette_index_idc. PaletteIndexIdc[ i ] = palette_index_idc. }. last_palette_run_type_flag. }. CurrNumIndices = 0. PaletteScanPos = 0. while( PaletteScanPos < nCbS * nCbS ) {. …. if( MaxPaletteIndex > 0 && CurrNumIndices < NumPaletteIndices ) {. if( PaletteScanPos >= nCbS && palette_run_type_flag[ xcPrev ][ ycPrev ] != COPY_ABOVE_MODE && PaletteScanPos < nCbS * nCbS – 1) {. palette_run_type_flag[ xC ][ yC ]. }. …. }. runPos = 0. k=0. while ( runPos < = paletteRun ) {. xR = x0 + travScan[ PaletteScanPos ][ 0 ]. yR = y0 + travScan[ PaletteScanPos ][ 1 ]. if(palette_run_type_flag[k++][ xC ][ yC ] = = COPY_INDEX_MODE ) {. PaletteSampleMode[ xR ][ yR ] = COPY_INDEX_MODE. PaletteIndexMap[ xR ][ yR ] = CurrPaletteIndex. } else {. PaletteSampleMode[ xR ][ yR ] = COPY_ABOVE_MODE. PaletteIndexMap[ xR ][ yR ] = PaletteIndexMap[ xR ][ yR − 1 ]. }. runPos++. PaletteScanPos++. }. }. }. palette_index_idc is an indication of an index to the array represented by currentPaletteEntries. The value of palette_index_idc shall be in the range of 0 to adjustedIndexMax, inclusive. When palette_index_idc is not present, it is inferred to be equal to 0. The variable PaletteIndexMap[ xC ][ yC ] specifes a palette index, which is an index to the array represented by CurrentPaletteEntries. The array indices xC, yC specify the location ( xC , yC ) of the sample relative to the top-left luma sample of the picture. The value of PaletteIndexMap[ xC ][ yC ] shall be in the range of 0 to indexMax, inclusive. The variable adjustedRefIndexMax is derived as follows: adjustedRefIndex = indexMax + 1 if( scanPos > 0 ) { if( palette_run_type_flag[xcPrev][ycPrev] != COPY_ABOVE_MODE ) adjustedRefIndex = PaletteIndexMap[xcPrev][ycPrev] (7-4) else adjustedRefIndex = PaletteIndexMap[xC][yC − 1] When palette_run_type_flag[ xC ][ yC ] is equal to COPY_INDEX_MODE, the variable paletteIndex is derived as follows: paletteIndex = palette_index_idc if( palette_index_idc >= adjustedRefIndex ) paletteIndex = palette_index_idc + 1

# Experimental Results

The proposed method is integrated on SCM-4.0. All tests showed were conducted under the common test condition (CTC) [2] for SCC with full frame IBC search. Table 1 shows the results of 2.1 removing index adjustment when previous pixel has COPY_ABOVE. It shows average 0.1%, 0.0%, 0.0% loss under AI, RA and LB configuration respectively. Table 2 shows the results of 2.2 removing index adjustment when previous pixel has COPY_ABOVE. It shows average 0.3%, 0.2%, 0.2% loss under AI, RA and LB configuration respectively with maximum 1% loss. Table 3 and Table 4 show the results of 2.3 removing index adjustment. Without signalling run_type before grouped indices, it shows average 0.3%, 0.1%, 0.1% loss under AI, RA and LB configuration respectively (Table 3) with maximum 0.9% loss. With signalling run_type before grouped indices, it shows 0.3%, 0.2%, 0.1% loss under AI, RA and LB configuration respectively (Table 4) with maximum 1.1% loss. Table 5 shows the performance of signalling run type before grouped indices itself. It shows 0.0%, 0.0%, 0.0% loss under AI, RA and LB configuration respectively. Decoding times may not be accurate.

Table 1 2.1 removing index adjustment when previous pixel has COPY_ABOVE

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.2% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 0.2% | 0.3% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.2% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.1% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 0.2% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.2% | -0.1% |
| YUV, Animation, 720p | -0.1% | -0.2% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.0% | 0.2% |
| RGB, Animation, 720p | -0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | -0.1% | -0.1% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.9% | 0.4% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2 2.2 removing index adjustment when previous pixel has COPY_INDEX

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.7% | 0.8% | 0.7% |
| RGB, mixed content, 1440p & 1080p | 0.2% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 1.0% | 1.0% | 1.2% |
| YUV, mixed content, 1440p & 1080p | 0.4% | 0.6% | 0.7% |
| YUV, Animation, 720p | 0.0% | 0.4% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.4% | 0.4% | 0.4% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.2% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.6% | 0.7% | 0.7% |
| YUV, mixed content, 1440p & 1080p | 0.3% | 0.5% | 0.3% |
| YUV, Animation, 720p | -0.1% | 0.1% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.4% | 0.3% | 0.4% |
| RGB, mixed content, 1440p & 1080p | 0.3% | 0.2% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.4% | 0.5% | 0.9% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.7% | 0.6% |
| YUV, Animation, 720p | -0.1% | 0.3% | 0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 1 2.3.1 removing index adjustment without signalling run_type before grouped indices

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.7% | 0.8% | 0.7% |
| RGB, mixed content, 1440p & 1080p | 0.2% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.9% | 0.9% | 1.1% |
| YUV, mixed content, 1440p & 1080p | 0.4% | 0.6% | 0.7% |
| YUV, Animation, 720p | 0.0% | 0.4% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.4% | 0.4% | 0.4% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.2% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.5% | 0.7% | 0.7% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.3% | 0.3% |
| YUV, Animation, 720p | -0.1% | 0.2% | 0.4% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.4% | 0.4% | 0.4% |
| RGB, mixed content, 1440p & 1080p | 0.3% | 0.1% | 0.2% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | -0.1% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.3% | 0.5% | 0.9% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.7% | 0.4% |
| YUV, Animation, 720p | -0.1% | 0.2% | 0.1% |
| YUV, camera captured, 1080p | -0.1% | 0.1% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

Table 2 2.3.2 removing index adjustment with signalling run_type before grouped indices

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.7% | 0.8% | 0.8% |
| RGB, mixed content, 1440p & 1080p | 0.2% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 1.1% | 1.1% | 1.3% |
| YUV, mixed content, 1440p & 1080p | 0.4% | 0.7% | 0.7% |
| YUV, Animation, 720p | 0.0% | 0.4% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.5% | 0.5% | 0.5% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.3% | 0.3% |
| RGB, Animation, 720p | 0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.6% | 0.8% | 0.8% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 0.5% | 0.3% |
| YUV, Animation, 720p | -0.1% | 0.1% | 0.3% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.4% | 0.5% | 0.5% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.3% | 0.4% |
| RGB, Animation, 720p | 0.0% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.4% | 0.7% | 1.0% |
| YUV, mixed content, 1440p & 1080p | 0.3% | 1.0% | 0.6% |
| YUV, Animation, 720p | -0.1% | -0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |

Table 5 Signalling run_type before grouped indices

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.1% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.2% | 0.1% |
| RGB, Animation, 720p | 0.1% | 0.0% | 0.1% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.2% | 0.0% |
| YUV, Animation, 720p | -0.1% | -0.1% | 0.1% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.1% |
| RGB, mixed content, 1440p & 1080p | 0.1% | -0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.2% |
| RGB, camera captured, 1080p | -0.1% | 0.0% | -0.1% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.5% |
| YUV, mixed content, 1440p & 1080p | 0.2% | 1.1% | 0.7% |
| YUV, Animation, 720p | 0.1% | 0.0% | -0.3% |
| YUV, camera captured, 1080p | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

# References

- J. Kim, P. Lai, S. Liu, S. Lei, “CE1-related : simplification for index map coding in palette mode,” JCTVC-T0078, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 1120th Meeting, Feb. 2015

- P. Lai, V. Seregin, X. Xiu, C. Gisquet, “Description of Core Experiment 1 (CE1): Palette Mode Improvement,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 1120th Meeting, JCTVC-T1101, Feb. 2015.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).