| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0051_r1 |
| --- | --- |

| Title: | Comparison of Compression Performance of HEVC Test Model 16.4 and HEVC Screen Content Coding Extensions Test Model 4 with AVC High 4:4:4 Predictive profile |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Gary J. Sullivan | Email: | libin@microsoft.com jzxu@microsoft.com garysull@microsoft.com |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

This contribution is a study of the relative objective (i.e. PSNR-based) compression performance of HEVC RExt Main 4:4:4 Profile, HEVC Screen Content Coding (SCC) Test Model 4 (SCM 4) and AVC High 4:4:4 Predictive Profile. It builds upon the prior work reported in JCTVC-G399, JCTVC-H0360, JCTVC-I0409, JCTVC-J0236, JCTVC-K0279, JCTVC-L0322, JCTVC-M0329, JCTVC-O0184, JCTVC-P0213, JCTVC-R0101, JCTVC-S0084, and JCTVC-T0042 – updating the results by using the latest available reference software (JM-18.6, HM-16.4, HM-16.4+SCM-4.0), profile and test model designs, and SCC common test conditions (CTC) test sequences. The overall results indicate that for screen content CTC sequences, the HEVC SCC Test Model 4 improves quite substantially over JM-18.6. For example, for RGB text and graphics with motion (TGM) 1080p&720 sequences, HEVC SCC Test Model 4 saves 86%, 80%, and 78% bits for AI, RA and LB lossy coding over JM-18.6, respectively.

# Performance Comparison

We used HM-16.4, HM-16.4+SCM-4.0 and JM-18.6 in the tests. The JM software was configured with the same coding structure and referencing structure as HM using the “HM-like” configurations. High 4:4:4 Predictive profile is used in JM. SCC CTC [13] test conditions (e.g. internal bit depth, intra period, QP setting, etc.) are followed in the comparison.

It should be noted that, for screen test sequences, SCM-4.0 improves quite a lot over JM-18.6. If we still use the same QP values to generate bitstreams and calculate BD-Rate, the PSNR overlapping range may be small (or even not overlapped). So we adjust the QP values to generate JM-18.6 bitstreams. For Desktop, Console, and Webbrowsing, we use QP 12, 17, 22, 27 to generate JM-18.6 bitstreams. For FlyingGraphics, Map, Programming, SlideShow, BasketballScreen, MissionControl2, and MissionControl3, we use QP 17, 22, 27, 32 to generate JM-18.6 bitstreams. For other sequences, we use the QP 22, 27, 32, 37 to generate JM-18.6 bitstreams.

# Experimental result

It should be noted that the encoding/decoding times, especially decoding times, are not so reliable – since the experiments were performed using a computing "cluster".

The results are shown in Tables 1–8 below. Tables 1-4 compare SCM with HM and tables 5-8 compare SCM with JM. For the lossy coding results, the numbers are BD-Rate, and for lossless coding result, the numbers are bitrate change percentages. For both lossy and lossless coding, a negative number means the codec listed on the right is better.

Table 1 – JM18.6 vs. HM-16.4+SCM-4.0 for SCC CTC sequences, lossy coding

Table 2 – JM18.6 vs. HM-16.4+SCM-4.0 for SCC CTC 4:2:0 sequences, lossy coding

Table 3 – JM18.6 vs. HM-16.4+SCM-4.0 for SCC CTC 4:4:4 sequences, lossless coding

Table 4 – JM18.6 vs. HM-16.4+SCM-4.0 for SCC CTC 4:2:0 sequences, lossless coding

Table 5 – HM-16.4 vs. HM-16.4+SCM-4.0 for SCC CTC sequences, lossy coding

Table 6 – HM-16.4 vs. HM-16.4+SCM-4.0 for SCC CTC 4:2:0 sequences, lossy coding

Table 7 – HM-16.4 vs. HM-16.4+SCM-4.0 for SCC CTC 4:4:4 sequences, lossless coding

Table 8 – HM-16.4 vs. HM-16.4+SCM-4.0 for SCC CTC 4:2:0 sequences, lossless coding

# References

- JCTVC-G399, “Comparison of Compression Performance of HEVC Working Draft 4 with AVC High Profile,” Geneva, CH, Nov. 2011.

- JCTVC-H0360, “Comparison of Compression Performance of HEVC Working Draft 5 with AVC High Profile,” San José, US, Feb. 2012.

- JCTVC-I0409, “Comparison of Compression Performance of HEVC Draft 6 with AVC High Profile,” Geneva, CH, 27 April – 7 May 2012.

- JCTVC-J0236, “Comparison of Compression Performance of HEVC Draft 7 with AVC High Profile,” Stockholm, SE, 11–20 July 2012.

- JCTVC-K0279, “Comparison of Compression Performance of HEVC Draft 8 with AVC High Profile and Performance of HM8.0 with Different Delay Characteristics,” Shanghai, CN, 10–19 Oct. 2012.

- JCTVC-L0322, “Comparison of Compression Performance of HEVC Draft 9 with AVC High Profile and Performance of HM9.0 with Temporal Scalability Characteristics,” Geneva, CH, 14–23 Jan. 2013.

- JCTVC-N1005, “High Efficiency Video Coding (HEVC) Range Extensions text specification: Draft 4,” Vienna, AT, 25 July – 2 Aug. 2013.

- JCTVC-O0184, “Comparison of Compression Performance of HEVC Range Extensions Draft 4 with AVC High 4:4:4 Predictive profile,” Geneva, CH, 23 Oct. – 1 Nov. 2013.

- JCTVC-P0213, “Comparison of Compression Performance of HEVC Range Extensions Draft 5 with AVC High 4:4:4 Predictive profile,” Valencia, ES, 27 Mar. – 4 Apr. 2014.

- JCTVC-R0101, “Comparison of Compression Performance of HEVC 4:4:4 Range Extensions Test Model 7 and HEVC Screen Content Coding Extensions Test Model 1 with AVC High 4:4:4 Predictive profile,” Sapporo, JP, 30 June – 9 July 2014.

- JCTVC-S0084, “Comparison of Compression Performance of HEVC 4:4:4 Range Extensions Test Model 8 and HEVC Screen Content Coding Extensions Test Model 2 with AVC High 4:4:4 Predictive profile,” Strasbourg, FR, 17–24 Oct. 2014.

- JCTVC-T0042, “Comparison of Compression Performance of HEVC Test Model 16.2 and HEVC Screen Content Coding Extensions Test Model 3 with AVC High 4:4:4 Predictive profile,” Geneva, CH, 10–18 Feb. 2015.

- JCTVC-T1015, “Common conditions for screen content coding tests,” Geneva, CH, 10–18 Feb. 2015.