| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0052 |
| --- | --- |

| Title: | On the palette escape pixel coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Tel: Email: |  |
| Source: | Microsoft Corp. |  |  |

_____________________________

# Abstract

Escape pixel is introduced in the palette mode. However, the binarization process of escape pixels depends on the CU QP values. It introduces parsing dependency between the palette escape pixel and QP value. This document presents several methods to remove the parsing dependency between the palette escape pixel and CU QP value.

# Introduction

Escape pixel is introduced in the palette mode. However, the binarization process of escape pixels depends on the CU QP values. It introduces parsing dependency between the palette escape pixel and QP value. This document proposes several methods to remove the parsing dependency between the palette escape pixel and CU QP value.

# Solutions

## Constant QP value for palette escape pixel

To remove the parsing dependency between the palette escape pixel and CU QP value, for a slice, the palette escape pixel always use a constant QP value. The QP value for palette escape pixel in a slice is signalled at slice header as QP difference. And the delta QP value is not necessarily signalled for palette escape pixels.

## QP-independent escape pixel binarization

The 2nd method is to use 3rd order Exp-Golomb binarization for escape pixels in lossy coded CUs.

For the escape pixels in the lossless CUs, it is also binarized as fixed-length coding (with cMax is set to the max pixel value).

## CU QP-independent escape pixel binarization

The 3rd method is to use an adaptive order Exp-Golomb binarization for escape pixels in lossy coded CUs. The order is related to the slice QP value. The maximum quantized value of the escape pixel is first calculated using the slice QP value, denoting as maxValue. The order of Exp-Golomb is determined as

order = 0;

while (maxValue > 1) {order++; maxValue >>= 1;}

For the escape pixels in the lossless CUs, it is also binarized as fixed-length coding (with cMax is set to the max pixel value).

# Experimental results

## Constant QP value for palette escape pixel

There is no significant performance change compared with SCM-4.0 anchor. Only several slice header level syntax elements are added.

## QP-independent escape pixel binarization

Table 1 Experimental results for lossy coding (444 sequences)

Table 2 Experimental results for lossy coding (420 sequences)

## CU QP-independent escape pixel binarization

Table 3 Experimental results for lossy coding (444 sequences)

Table 4 Experimental results for lossy coding (420 sequences)

# Patent rights declaration(s)

Microsoft Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Text modifications

The text provided in this section is based on JCTVC-T1005-v2.docx.

## Constant QP value for palette escape pixel

#### 7.3.6.1	General slice segment header syntax

| …… |  |
| --- | --- |
| slice_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| } |  |
| if( pps_slice_act_qp_offsets_present_flag ) { |  |
| slice_act_y_qp_offset | se(v) |
| slice_act_cb_qp_offset | se(v) |
| slice_act_cr_qp_offset | se(v) |
| } |  |
| if ( palette_mode_enabled_flag ) { |  |
| palette_escape_pixel_qp_delta | se(v) |
| if ( ChromaArrayType != 0 ) { |  |
| palette_escape_pixel_cb_qp_offset | se(v) |
| palette_escape_pixel_cr_qp_offset | se(v) |
| } |  |
| } |  |
| if( chroma_qp_offset_list_enabled_flag ) |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| …… |  |

#### 7.3.8.8	Palette syntax

| … |  |
| --- | --- |
| if( palette_escape_val_present_flag ) { |  |
| if( cu_qp_delta_enabled_flag && !IsCuQpDeltaCoded ) { |  |
| cu_qp_delta_palette_abs | ae(v) |
| if( cu_qp_delta_palette_abs ) |  |
| cu_qp_delta_palette_sign_flag | ae(v) |
| } |  |
| if( cu_chroma_qp_offset_enabled_flag && !IsCuChromaQpOffsetCoded ) { |  |
| cu_chroma_qp_palette_offset_flag | ae(v) |
| if( cu_chroma_qp_offset_flag && chroma_qp_offset_list_len_minus1 > 0 ) |  |
| cu_chroma_qp_palette_offset_idx | ae(v) |
| } |  |
| } |  |
| … |  |

#### 7.4.7.1	General slice segment header semantics

palette_escape_pixel_qp_delta specifies the EscQpY of palette escape pixels in the slice. When not present, it is inferred to be equal to 0. EscQpY is derived as follows:

EscQpY = SliceQpY + palette_escape_pixel_qp_delta	(7-XX)

The value of EscQpY shall be in the range of −QpBdOffsetY to +51, inclusive.

palette_escape_pixel_cb_qp_offset specifies a difference to be added to the value of pps_cb_qp_offset when determining the value of the EscQpCb quantization parameter used for the palette escape pixels in the slice. The value of palette_escape_pixel_cb_qp_offset shall be in the range of −12 to +12, inclusive. When palette_escape_pixel_cb_qp_offset is not present, it is inferred to be equal to 0. The value of pps_cb_qp_offset + palette_escape_pixel_cb_qp_offset shall be in the range of −12 to +12, inclusive.

palette_escape_pixel_cr_qp_offset specifies a difference to be added to the value of pps_cr_qp_offset when determining the value of the EscQpCr quantization parameter used for the palette escape pixels in the slice. The value of palette_escape_pixel_cr_qp_offset shall be in the range of −12 to +12, inclusive. When palette_escape_pixel_cr_qp_offset is not present, it is inferred to be equal to 0. The value of pps_cr_qp_offset + palette_escape_pixel_cr_qp_offset shall be in the range of −12 to +12, inclusive.

EscQpCb and EscQpCr are derived as follows:

EscQpCb = Clip3( −QpBdOffsetC, 57, EscQpY + pps_cb_qp_offset + palette_escape_pixel_cb_qp_offset )	(7-XX)

EscQpCr = Clip3( −QpBdOffsetC, 57, EscQpY + pps_cr_qp_offset + palette_escape_pixel_cr_qp_offset )	(7-XX)

#### 7.4.9.6	Palette mode semantics

cu_qp_delta_palette_abs specifies the absolute value of the difference CuQpDeltaVal between the luma quantization parameter of the current coding unit and its prediction.

cu_qp_delta_palette_sign_flag specifies the sign of CuQpDeltaVal as follows:

- If cu_qp_delta_palette_sign_flag is equal to 0, the corresponding CuQpDeltaVal has a positive value.

- Otherwise (cu_qp_delta_palette_sign_flag is equal to 1), the corresponding CuQpDeltaVal has a negative value.

When cu_qp_delta_palette_sign_flag is not present, it is inferred to be equal to 0.

When cu_qp_delta_palette_abs is present, the variables IsCuQpDeltaCoded and CuQpDeltaVal are derived as follows:

IsCuQpDeltaCoded = 1		(7-1)

CuQpDeltaVal = cu_qp_delta_palette_abs * ( 1 − 2 * cu_qp_delta_palette_sign_flag )	(7-2)

The value of CuQpDeltaVal shall be in the range of −( 26 + QpBdOffsetY / 2 ) to +( 25 + QpBdOffsetY / 2 ), inclusive.

cu_chroma_qp_palette_offset_flag, when present and equal to 1, specifies that an entry in the cb_qp_offset_list[ ] is used to determine the value of CuQpOffsetCb and a corresponding entry in the cr_qp_offset_list[ ] is used to determine the value of CuQpOffsetCr. cu_chroma_qp_palette_offset_flag equal to 0 specifies that these lists are not used to determine the values of CuQpOffsetCb and CuQpOffsetCr.

cu_chroma_qp_palette_offset_idx, when present, specifies the index into the cb_qp_offset_list[ ] and cr_qp_offset_list[ ] that is used to determine the value of CuQpOffsetCb and CuQpOffsetCr. When present, the value of cu_chroma_qp_palette_offset_idx shall be in the range of 0 to chroma_qp_offset_list_len_minus1, inclusive. When not present, the value of cu_chroma_qp_palette_offset_idx is inferred to be equal to 0.

When cu_chroma_qp_palette_offset_flag is present, the following applies:

- The variable IsCuChromaQpOffsetCoded is set equal to 1.

- The variables CuQpOffsetCb and CuQpOffsetCr are derived as follows:

- If cu_chroma_qp_palette_offset_flag is equal to 1, the following applies:

CuQpOffsetCb = cb_qp_offset_list[ cu_chroma_qp_offset_idx ]	(7-3)

CuQpOffsetCr = cr_qp_offset_list[ cu_chroma_qp_offset_idx ]	(7-4)

- Otherwise (cu_chroma_qp_palette_offset_flag is equal to 0), CuQpOffsetCb and CuQpOffsetCr are both set equal to 0.

NOTE – When cu_chroma_qp_offset_enabled_flag is equal to 0, CuQpOffsetCb and CuQpOffsetCr are not modified after being initialized to 0 for the slice as specified in clause .

Remove the binarization process of the removed syntax elements.

#### 8.4.4.2.7 Decoding process for palette mode

……

2.	The quantization parameter qP is derived as follows:

–	If cIdx is equal to 0,

qP = max( 0, EscQpY Qp′Y )		(8-5)

–	Otherwise, if cIdx is equal to 1,

qP = max( 0, EscQpCb Qp′Cb )		(8-6)

–	Otherwise (cIdx is equal to 2),

qP = max(0, EscQpCr Qp′Cr )		(8-7)

……

#### 9.3.3.13	Binarization process for palette_escape_val

- Otherwise (cu_transquant_bypass_flag is false) the following ordered steps apply

- The quantization parameter qP is derived as follows:

qP = ( cIdx  = =  0 ) ? EscQpY Qp′Y : ( ( cIdx  = =  1 ) EscQpCb Qp′Cb ? EscQpCr Qp′Cr )	(9-8)

## QP-independent escape pixel binarization

#### 9.3.3.13	Binarization process for palette_escape_val

Input to this process is a request for a binarization for the syntax element palette_escape_val, cu_transquant_bypass_flag and colour component index cIdx.

Output of this process is the binarization of palette_escape_val.

The variable bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC

The binarization of palette_escape_val is derived as follows:

- If cu_transquant_bypass_flag is true, the binarization of palette_escape_val is derived by invoking the FL binarization process specified in clause Error: Reference source not found with the input parameter set to (1<<bitdepth) − 1.

- Otherwise (cu_transquant_bypass_flag is false), the binarization of palette_escape_val is derived by invoking the k-th order Exp-Golomb binarization process specified in clause 9.3.3.3 with the k is set to equal to 3. the following ordered steps apply

- The quantization parameter qP is derived as follows:

qP = ( cIdx  = =  0 ) ? Qp′Y : ( ( cIdx  = =  1 ) Qp′Cb ? Qp′Cr )	(9-9)

- The variable bdShift is derived as follows:

bdShift = 14 + qP / 6		(9-10)

- The list quantScale[ ] is specified as quantScale[ k ] = { 26214, 23302, 20560, 18396, 16384, 14564 } with k = 0..5.

- The maximum parameter cMax for TB binarization process is derived as follows:

cMax = ( ( ( 1<< bitDepth ) − 1 ) *  quantScale[ qP%6 ] + ( 1 << ( bdShift − 1) ) ) >> bdShift	(9-11)

The binarization for the palette_escape_value is derived by invoking the TB binarization process specified in clause Error: Reference source not found with cMax.

## CU QP-independent escape pixel binarization

#### 9.3.3.13	Binarization process for palette_escape_val

Input to this process is a request for a binarization for the syntax element palette_escape_val, cu_transquant_bypass_flag and colour component index cIdx.

Output of this process is the binarization of palette_escape_val.

The variable bitDepth is derived as follows:

bitDepth = ( cIdx  = =  0 ) ? BitDepthY : BitDepthC

The binarization of palette_escape_val is derived as follows:

- If cu_transquant_bypass_flag is true, the binarization of palette_escape_val is derived by invoking the FL binarization process specified in clause Error: Reference source not found with the input parameter set to (1<<bitdepth) − 1.

- Otherwise (cu_transquant_bypass_flag is false) the following ordered steps apply

- The quantization parameter qP is derived as follows:

qP = ( cIdx  = =  0 ) ? Qp′Y : ( ( cIdx  = =  1 ) Qp′Cb ? Qp′Cr )	(9-12)

qP = ( cIdx  = =  0 ) ? SliceQpY: ( ( cIdx  = =  1 ) Clip3( −QpBdOffsetC, 57, SliceQpY + pps_cb_qp_offset + slice_cb_qp_offset )? Clip3( −QpBdOffsetC, 57, SliceQpY + pps_cr_qp_offset + slice_cr_qp_offset ))	(9-13)

- The variable bdShift is derived as follows:

bdShift = 14 + qP / 6		(9-14)

- The list quantScale[ ] is specified as quantScale[ k ] = { 26214, 23302, 20560, 18396, 16384, 14564 } with k = 0..5.

- The maximum parameter cMax for TB binarization process is derived as follows:

cMax = ( ( ( 1<< bitDepth ) − 1 ) *  quantScale[ qP%6 ] + ( 1 << ( bdShift − 1) ) ) >> bdShift	(9-15)

order = 0;

while (cMax > 1) {order++; cMax >>= 1;}

the binarization of palette_escape_val is derived by invoking the k-th order Exp-Golomb binarization process specified in clause 9.3.3.3 with the k is set to equal to order

The binarization for the palette_escape_value is derived by invoking the TB binarization process specified in clause Error: Reference source not found with cMax.