| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0053 |
| --- | --- |

| Title: | CE2 Test 2: Intra block copy constraints on filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Gilles Malard Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports the results of CE2 Test2 on Intra block copy constraints on filtering. The aim is to reduce the worst case cache memory for some decoder implementation when IBC and loop filters are used together. A flag is transmitted at CTU level to know if the deblocking filter and SAO are disabled. The encoder sets the value of this flag by analysing the block vector value after the encoding of the whole frame. The tested method gives similar results in SCM 4.0 as those obtained in SCM3.0. The BDR average for Full frame IBC search configuration is 0.7%, 0.5% and 0.4% for respectively AI, RA, and LB configurations. And 0.4%, 0.3% and 0.3% for 1x4 CTUs search configuration.

# Introduction

For the Intra Block Copy method, the PU predictors come from reconstructed data of the current frame and as all other Intra modes this data is non-loop filtered.

The DBF and SAO loop filtering of HEVC have been defined to be fully parallelized at CU level if all neighboring data are available. So for some implementations, the DBF and SAO are proceeded on the fly and/or in parallel to the other processes of block reconstruction. To keep this ability to apply the SAO and DBF at CU level in parallel, when Intra Block Copy method is enabled, it is needed to store both loop filtered data and non-filtered data. This increases the cache memory needed by one frame for the worst case because a BV in the last CU can point at the very beginning of the image. The alternative is to delay DBF and SAO until the frame is reconstructed, but this then requires saving all DBF and SAO syntaxes for delayed application.

In this CE2 Test 2, the available CTUs for the Intra block copy predictor are signaled in the bitstream. When a CTU is signaled as available for IBC, it is not loop filtered.

# Proposed modifications

## Bitstream and constraint

A flag, use_loop_filtering flag, is inserted in the bitstream. This flag indicates that the current CTU is loop filtered or not. And if not this CTU can be used as reference for IBC prediction. Moreover when this flag is true, the deblocking filter and SAO are enabled. This flag is now a CABAC context coded bin.

In addition, the current CTU and the previous one are always considered as available for IBC prediction.

As a consequence, the cache memory increase for the implementation processing DBF and SAO on the fly is only 2 CTBs. Figure 1 illustrates the proposed method.

Figure 1 Example of the proposed method.

## Encoder selection

In the current SCM4.0, DBF and SAO are processed at picture level. In our encoder algorithm selection, before using DBF and SAO, the encoder sets the value of the flag use_loop_filtering to one if at least one block vector points to this CTU and if the IBC PU is not in the current CTU or following CTU.

In addition, we adapted the early termination of the HM software (JCTVC-I0184) to the proposed method as described in our initial contribution JCTVC-T0051.

# Experimental results

The CE2 Test2 has been tested on top of SCM4.0 for common test conditions, except for lossless configurations where the method is not applied because SAO and DBF are disabled.

## BDR results

Table 1 gives the results of CE2 Test 2 for Full frame IBC search range for 4:4:4 configuration. In average for all classes, 0.7%, 0.5% and 0.4% is obtained for respectively AI, RA, and LB configurations.

Table 1: Lossy results of the proposed method compared to SCM4.0 Anchor for 4:4:4 full frame IBC search configuration.

Table 2 gives the results of CE2 Test 2 for 1x4 CTUs IBC search range for 4:4:4 configuration. In average for all classes, 0.4%, 0.3% and 0.3% is obtained for respectively AI, RA, and LB configurations.

Table 2: Lossy results of the proposed method compared to SCM4.0 Anchor for 4:4:4 1x4 CTU IBC search configuration.

Table 3 gives the results of CE2 Test 2 for 4:2:0 configuration.

Table 3: Lossy results of the proposed method compared to SCM4.0 Anchor for 4:2:0 configuration.

## Memory analysis

Table 4 and Table 5 give respectively the average memory bandwidth and the Maximum memory bandwidth for lossy configurations only. Indeed the proposed method is only applicable for lossy configurations and not for losses. The proposed method reduces the average memory bandwidth and the maximum memory bandwidth.

Table 4: Average memory bandwidth with respect to the anchor for lossy configurations.

|  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 444 | ai | 79% | 79% | 80% | 80% | 81% | 82% | 98% | 98% |
|  | ra | 74% | 75% | 75% | 76% | 76% | 78% | 100% | 100% |
|  | lb | 72% | 72% | 73% | 73% | 74% | 76% | 100% | 100% |
| 444/4x1 | ai | 72% | 72% | 73% | 73% | 74% | 75% | 100% | 100% |
|  | ra | 74% | 74% | 75% | 75% | 75% | 76% | 100% | 100% |
|  | lb | 71% | 71% | 72% | 72% | 73% | 74% | 100% | 100% |
| 420 | ai | 85% | 85% | 86% | 86% | 87% | 88% | 100% | 100% |
|  | ra | 74% | 75% | 75% | 75% | 77% | 78% | 100% | 100% |
|  | lb | 71% | 72% | 73% | 73% | 74% | 76% | 100% | 100% |

Table 5: Maximum memory bandwidth with respect to the anchor for lossy configurations.

|  |  | A8/B8 | A32/B64 | A32/B128 | A64/B128 | A64/B256 | A64/B512 | A64/B256F | A64/B512F |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 444 | ai | 87% | 87% | 87% | 87% | 86% | 84% | 97% | 97% |  |
|  | ra | 77% | 79% | 85% | 86% | 88% | 86% | 102% | 101% |  |
|  | lb | 77% | 79% | 83% | 84% | 87% | 88% | 97% | 96% |  |
| 444/4x1 | ai | 79% | 79% | 79% | 79% | 79% | 78% | 100% | 100% |  |
|  | ra | 76% | 78% | 81% | 81% | 81% | 81% | 100% | 101% |  |
|  | lb | 76% | 77% | 79% | 80% | 82% | 81% | 100% | 100% |  |
| 420 | ai | 88% | 88% | 88% | 88% | 88% | 88% | 100% | 100% |  |
|  | ra | 80% | 82% | 86% | 87% | 88% | 88% | 101% | 101% |  |
|  | lb | 78% | 80% | 86% | 87% | 88% | 89% | 97% | 98% |  |

# Conclusion

In this contribution, we report the results of CE2 Test2 which consists in enabling or disabling the loop filtering at CTU level and constraint the use of the IBC prediction to the CTU where DBF and SAO are disabled. The aim of this proposal is to avoid the storing of both filtered and non-filtered reconstructed frames.

The results obtained over SCM4.0 are similar to those obtained over SCM3.0. The memory analysis of decoding process shows a significant reduction of memory access. According to the significant advantages of the proposed method and the small impact on coding efficiency, we propose to adopt the proposed method.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).