| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0054 |
| --- | --- |

| Title: | Non-CE2: On interpretation of the IBC vectors |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Christophe Gisquet, Guillaume Laroche, Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

At the 22nd meeting held in Geneva (CH) in February 2015, the alignment of Intra Block Copy on Inter signaling, as proposed by JCTVC-T0227, was adopted. Various normative changes were thus introduced, however, it was decided that the motion prediction mechanisms of HEVC v1 would better be kept unmodified, in spite of various proposals reporting gains by adapting these mechanisms. It is asserted that the normative changes from JCTVC-T0227 can be tweaked to recoup some of those gains. It is reported that the proposed changes provide a BDR gain of 0.4% to 0.7% over SCM4.0 for Screen Content classes in the lossy AI scenario, and 0.2% to 0.7% for the lossy RA scenario.

# Description

## High-level view of the current design

Figure 1: simplified 3-parts design of Intra Block Copy and Inter alignment

The current design can be schematized in 3 steps, as depicted in Figure 1 above. Whether a block is coded using Intra Block Copy can be inferred by checking whether the reference frame is the current one. This check is used in the two main normative changes introduced by JCTVC-T0227 [1], in the case the block is coded using IBC:

- The decoded residual is scaled from its full-pel precision to the expected quarter-pel precision of motion vectors;

- During motion compensation, special treatment is applied to the IBC MV:

  - It does not undergo the interpretation of Adaptive Motion Vector Resolution (“Scale MV” in Figure 1 above);

  - Truncation is applied to the chroma MV to retain integer precision satsifying the chroma sampling.

The main rationale for such changes can be summarized as a coding efficiency change. On the other hand, past proposals affecting the motion prediction were not deemed desirable.

## Consequences on coding efficiency

The prediction mechanisms were tuned for Inter and natural motion, and not Screen Content. As a consequence, a number of inefficiencies exist:

- A BV can easily become invalid, due to the causal area and its restrictions, such as described in [2];

- A number of candidates used to fill the merge and AMVP lists are not usable, such as the zero candidates.

However, past proposals affecting the motion prediction stage in Figure 1 were deemed undesirable changes.

## Proposed changes

Figure 1 shows other locations with normative changes introduced by the SCC extension design. These could be exploited to help recoup some of the coding loss due to the inter alignment. The current proposal is, in the adaptive motion vector resolution step, where the MV may be scaled, to interpret specific values of the BV.

It would be possible to catch and interpret several cases, but the trade-off is unlikely to be interesting. To keep the change as minimal and as simple as possible, only the (0, 0) BV is interpreted to be (-N, 0) for an NxN CU, which is a very simple interpretation.

This results in the following specification text changes:

If the reference index refIdxLX is equal to currPic, then:

If  mvLX[ 0 ] == 0 and mvLX[ 1 ]  == 0, then mvLX and mvCLX (where X equals to 0 or 1) are both set as follows:

mvLX[ 0 ] = -( nCbS << 2 )

mvLX[ 1 ] = 0

mvCLX[ 0 ] = -( nCbSwC << 2 )

mvCLX[ 1 ] = 0

Otherwise, when use_integer_mv_flag is equal to 1, mvLX and mvCLX (where X equals to 0 or 1) are modified as follows:

mvLX = Clip3( − 215, 215 − 1, mvLX << 2 )	(8-1)

mvCLX = Clip3( − 215, 215 − 1, mvCLX << 2 )		(8-2)

As can be seen, the proposed change is a trivial one, on top of another already existing normative change compared to HEVC v1.

## Implementation

A global macro, IBC_INTERPRET_BV, is intended to switch on and off the proposal.

### Decoder-side

Decoder-side, the only change is in TComPrediction::xPredInterUni, and just corresponds to the above text change.

### Naïve encoder implementation

There are a number of places to be modified on the encoder side for a naïve implementation.

Indeed, the tests for merge and best AMVP candidates must beware that a (0, 0) BV is now valid, affecting for instance, TEncSearch::predMixedIntraBCInterSearch, TEncSearch::isBlockVectorValid, TEncSearch::predIntraBCSearch, TEncCu::xCheckRDCostIntraBCMerge2Nx2N and finally TEncCu::xCompressCU.

### The case of AMVP

As the (0, 0) BV is now valid, it can and should also be evaluated in AMVP. The corresponding position, (-N, 0), already was evaluated inside of the horizontal 1D search, but proper evaluation of its bit cost is needed.

In addition, the (0, 0) BV may end up also in the cache of best positions, as previous block sizes or neighbour blocks may introduce it in this cache. As a consequence, taking care of it there is also required. Therefore, evaluating it by adding it to that cache is the simplest implementation, and requires further modifying TEncSearch::predIntraBCSearch and TEncSearch::xIntraBCSearchMVChromaRefine.

Finally, properly handling (0, 0) as a valid BV is easier done by disabling a lot of legacy code relating to the “last BV” concept, that JCTVC-T0227 could have disabled. This mainly impacts TComDataCU and TEncCu.

All of the above has been folded under the macro IBC_INTERPRET_AMVP.

### Simplifying the merge evaluation

Finally, the test of (0, 0) as a merge IBC candidate can undergo a full RDO estimation. While providing the most gains, it is actually completely counterproductive: while a runtime can be expected from performing supplementary checks, the 11x% runtime is not warranted here.

It is a better trade-off to actually skip the evaluation of the interpreted BVs (and only them) in TEncCu::xCheckRDCostIntraBCMerge2Nx2N, and do it instead in the SAD-based estimation for merge candidates located in TEncSearch::predMixedIntraBCInterSearch and TEncSearch::predIntraBCSearch.

This is controlled by the macro IBC_INTERPRET_BV_SPEEDUP.

# Results

To simplify testing, only the FullFrame IBC configuration is evaluated..

## 4:4:4 sequences

### Lossy

### Lossless

## 4:2:0 sequences

### Lossy

### Lossless

# References

- JCTVC-T0227, “Non-CE2: Intra block copy and inter signalling unification,” C. Pang, Y.-K. Wang, V. Seregin, K. Rapaka, M. Karczewicz (Qualcomm), X. Xu, S. Liu, S. Lei (MediaTek), B. Li, J. Xu (Microsoft), Geneva, CH, Feb. 2015.

- JCTVC-S0088, “On WPP with palette mode and intra BC mode,” B. Li, J. Xu (Microsoft), Strasbourg, FR, Oct. 2014.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).