| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0055 |
| --- | --- |

| Title: | High-level syntax refinement in monochrome signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Christophe Gisquet, Guillaume Laroche, Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, France | Tel: Email: | +33 299876800 christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon CRF |  |  |

_____________________________

# Abstract

During the editorial integration of JCTVC-T0048, related to palette syntax in the PPS SCC extension, support for monochrome and bitdepths higher than 8 was considered and added. However, it is asserted this syntax may cause non-sensical signaling. This contribution proposes a modification of the PPS-level syntax to make the monochrome information more generic to address this issue. Specifically, the flag monochrome_palette_flag is removed and a more generic flag monochrome_flag is added, keeping the previous meaning of monochrome_palette_flag, but also impacting the signaling of residual adaptive colour transform syntax. It is asserted this solves such ambiguity as having monochrome palette entries while ACT is active.

# Discussion

JCTVC-T0048 introduced a palette predictor initializer transmitted at the PPS level. It was refined in order to achieve support for more color formats in the following way:

- A monochrome_palette_flag which indicates whether the entries of the initializer are monochrome or not, thereby impacting the number of elements to read and the bitdepth information on those entries;

- Bitdepth information for luma and chroma, in order to support deeper than 8 bits data.

This results in the current syntax for the PPS SCC extension:

| pps_scc_extension( ) { | Descriptor |
| --- | --- |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| if( residual_adaptive_colour_transform_enabled_flag ) { |  |
| pps_slice_act_qp_offsets_present_flag | u(1) |
| pps_act_y_qp_offset_plus5 | se(v) |
| pps_act_cb_qp_offset_plus5 | se(v) |
| pps_ act_cr_qp_offset_plus3 | se(v) |
| } |  |
| palette_predictor_initializer_present_flag | u(1) |
| if( palette_predictor_initializer_present_flag ) { |  |
| monochrome_palette_flag | u(1) |
| luma_bit_depth_entry_minus8 | ue(v) |
| if( !monochrome_palette_flag ) |  |
| chroma_bit_depth_entry_minus8 | ue(v) |
| num_palette_predictor_initializer_minus1 | ue(v) |
| numComps = monochrome_palette_flag ? 1 : 3 |  |
| for( i = 0; i <= num_palette_predictor_initializer_minus1; i++ ) |  |
| for( comp = 0; comp < numComps; comp++ ) |  |
| palette_predictor_initializers[ i ][ comp ] | u(v) |
| } |  |
| } |  |

Table 1: PPS SCC extension syntax

However, this can lead to such scenario as ACT being active while initializer entries are monochrome. Bitstream conformance constrains already avoid this in a limited way, but it seems more elegant to account for the root cause of the issue and fix the syntax responsible for this.

The proposal is suggested to be a high-level syntax cleanup/bug-fix for such scenarios. It is proposed to modify the syntax in the following manner to fix said issue:

| pps_scc_extension( ) { | Descriptor |
| --- | --- |
| monochrome_flag | u(1) |
| if( !monochrome_flag ) |  |
| residual_adaptive_colour_transform_enabled_flag | u(1) |
| if( residual_adaptive_colour_transform_enabled_flag ) { |  |
| pps_slice_act_qp_offsets_present_flag | u(1) |
| pps_act_y_qp_offset_plus5 | se(v) |
| pps_act_cb_qp_offset_plus5 | se(v) |
| pps_ act_cr_qp_offset_plus3 | se(v) |
| } |  |
| palette_predictor_initializer_present_flag | u(1) |
| if( palette_predictor_initializer_present_flag ) { |  |
| luma_bit_depth_entry_minus8 | ue(v) |
| if( !monochrome_flag ) |  |
| chroma_bit_depth_entry_minus8 | ue(v) |
| num_palette_predictor_initializer_minus1 | ue(v) |
| numComps = monochrome_flag ? 1 : 3 |  |
| for( i = 0; i <= num_palette_predictor_initializer_minus1; i++ ) |  |
| for( comp = 0; comp < numComps; comp++ ) |  |
| palette_predictor_initializers[ i ][ comp ] | u(v) |
| } |  |
| } |  |

The description for monochrome_palette_flag, in particular bitstream conformance constrains, is kept for monochrome_flag.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).