| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0056 |
| --- | --- |

| Title: | Unification of the Adaptive Motion Resolution signaling for both IBC and Inter modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Gilles Malard Christophe Gisquet Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr christophe.gisquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution is related to the Intra block copy and Inter unification. In the current SCM4.0, the Intra block copy method is signaled as Inter modes by setting the current frame as a reference frame. But the block vectors have systematically the full-pel resolution in opposition to the motion vectors which should be in full-pel or sub-pel resolution according to the value of use_integer_mv_flag in the slice header. In this contribution, it is proposed to unify the use of the adaptive motion vector resolution for IBC in order to unify the usage of full-pel or sub-pel vector for Inter modes for all reference frames. There is no impact on coding efficiency over SCM4.0.

# Introduction

With current adaptive motion vector resolution (JCTVC-S0085) integrated in the SCM3.0, the motion vector resolution is set, for every slice, between full-pel and sub-pel resolutions. In that case, all motion vectors pointing in all reference frames have the same motion vector resolution.

Since the adoption of JCTVC-T0227, the Intra block copy method is signaled as Inter modes by setting the current frame is a reference frame. But the resolution of the vector pointing to this reference frame is systematically the full-pel resolution. So for this reference frame, the value of the use_integer_mv_flag is not taken into account. Consequently, at decoder side, a different process is needed only for the reference frame containing the current frame.

In this contribution, we propose to unify the use of the adaptive motion vector resolution and the coding of the block vector.

# Proposed method

In this contribution, we propose to signal the resolution of vectors for each reference frame. A flag use_integer_mv_flag is transmitted for each reference frame.

For the reference frame containing the current frame, this flag is always set equal to 1, so that the vector resolution for IBC is always the full-pel resolution.

At encoder side, the same algorithm as the one used in the current SCM has been adapted to select the value of the use_integer_mv_flag for each reference frame.

# Experimental results

Table 1 gives the results of the proposed modification over the SCM4.0 for 4:4:4 lossy sequences with full frame IBC search. In average the proposed modification has no impact.

Table 1: Lossy results of the proposed method compared to SCM4.0 Anchor for 4:4:4 full frame IBC search configuration.

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | #VALUE! |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.0% | 0.0% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | #VALUE! |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.0% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.2% |
| YUV, mixed content, 1440p & 1080p | 0.1% | 0.0% | 0.2% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | #VALUE! |  |  |

# Conclusion

In this contribution, we propose a unification of the adaptive motion vector resolution and the signaling of IBC as a reference frame. The modification consists in signaling the resolution of vectors for each reference frame in the slice header. It is mandatory to set the full-pel resolution for the IBC reference frame. The proposed unification has no impact on coding efficiency when the same encoder algorithm implemented in the current SCM to set the motion vector resolution is used for all reference frames. Yet, we should expect additional coding gains with a dedicated algorithm by setting the resolution for each reference frame.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).