| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0061 |
| --- | --- |

| Title: | CE1 : Test A.1 : Extended copy above mode to the first line with index adjustment bits |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun1, Jungsun Kim2, Tzu-Der Chuang1, Yi-Wen Chen1, Shan Liu2, Yu-Wen Huang1, Shawmin Lei1 1 MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 2 MediaTek San Jose: 2860 Junction Ave, San Jose, CA 95134, USA Vadim Seregin, Feng Zou, Wei Pu, Rajan Joshi, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, USA | Tel: Email: | Yu-Chen Sun +886-3-5670766 ext. 24572 {yc.sun, jungsun.kim, peter.chuang, yiwen.chen, shan.liu, yuwen.huang, shawmin.lei}@mediatek.com vseregin@qti.qualcomm.com |
| Source: | MediaTek Inc. Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In CE1 Test A.1, it was proposed for palette mode to use the nearest row of the above CU or the nearest column of the left CU to predict the pixel values of the current CU. The index adjustment bits are introduced for the palette index parsing. Experiments show that, compared with SCM-4.0, 1.5% and 1.9% BD-rate gains are shown under AI configuration, 0.9% and 1.1% BD-rate gains are shown under RA configuration, and 0.9% and 1.0% BD-rate gains are shown under LB configuration.

# Problem statement

In JCTVC-T0036 [1], an extended copy above mode to the first line is proposed to use the pixels in the nearest above row or the nearest left column in neighboring CUs to predict the samples in current CU. As shown in Fig. 1, for the first row of the current CU, if the palette run types are coded by a COPY_ABOVE run, the pixels values of samples in the first row are copied from the neighbouring pixels in the above CUs.

Fig.1. Example of extended copy above mode to the first line

In JCTVC-T0036, for a parsed palette index after a COPY_ABOVE run, if its above sample is directly copied from the sample outside of the current CU, the parsed index will not be adjusted. The AdjustedMaxPaletteIndex is set equal to MaxPaletteIndex. However, in SCM-4.0, the palette indices are grouped at front, before the palette run type flags and the palette runs. When parsing the palette index, whether the above sample is copied from the sample outside of the current CU is unknown.

# Proposed method

In this contribution, an extended copy above mode to the first line with index adjustment bits is proposed. When parsing the palette index, the AdjustedMaxPaletteIndex is set equal to MaxPaletteIndex – 1 except the first index. When parsing the palette run type flags and the palette runs, if a COPY_INDEX_MODE run is decoded, its previous coded run type is COPY_ABOVE run, the above sample is copied from neighboring CU, and the parsed index is equal to the AdjustedMaxPaletteIndex (where the parsed bins are all 1s), an index adjustment bit is decoded to tell whether the current index is equal to MaxPaletteIndex – 1 or MaxPaletteIndex. The index adjustment bit is coded in bypass mode.

# Experimental results

The proposed method is integrated on SCM-4.0 [2]. All tests were conducted under the common test condition (CTC) [3] for SCC with the full picture IBC search range. Table 1 and Table 2 show the lossy coding results of the CE1 Test A.1 for full-frame IntraBC search and 4-CTU IntraBC search in 444 format, respectively. For RGB and YUV text & graphics with motion sequences in full-frame IntraBC search, 1.5% and 1.9% BD-rate gains are shown under AI configuration, 0.9% and 1.1% BD-rate gains are shown under RA configuration, and 0.9% and 1.0% BD-rate gains are shown under LB configuration. The lossless results of the proposed method in 444 format are shown in Table 3. The lossy and lossless results of the proposed method in 420 format are shown in Table 4 and Table 5 respectively.The runtimes are not accurate.

# Conclusions

This contribution reports CE1 Test A.1. It was proposed for palette mode to use the nearest row of the above CU or the nearest column of the left CU to predict the pixel values of the current CU. The index adjustment bits are introduced to solve the palette index parsing ambiguity. Experiments show that, compared with SCM-4.0, 1.5% and 1.9% BD-rate gains are shown under AI configuration, 0.9% and 1.1% BD-rate gains are shown under RA configuration, and 0.9% and 1.0% BD-rate gains are shown under LB configuration.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Y.-C. Sun, J. Kim, S. Liu, T.-D. Chuang, Y.-W. Chen, Y.-W. Huang, S. Lei, V. Seregin, F. Zou, W. Pu, R. Joshi, M. Karczewicz, “CE1 : Tests A.1 : Extended copy above mode to the first line (1.1-1.5),” Document of Joint Collaborative Team on Video Coding, JCTVC-T0036, Feb. 2015.

- SCM-4.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-T1015, Feb. 2015.

Table 1.  Lossy results of the proposed method in 444 format and full frame Intra Block Copy search

Table 2. Lossy results of the proposed method in 444 format and constrained Intra Block Copy seaerch (1x4 CTUs)

Table 3.  Lossless results of the proposed method in 444 format and full frame Intra Block Copy search

Table 4.  Lossy results of the proposed method in 420 format and full frame Intra Block Copy search

Table 5. Lossless results of the proposed method in 420 format and full frame Intra Block Copy search