| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 21st Meeting: Warsaw, PL, 19–26 June 2015 | Document: JCTVC-U0063 |
| --- | --- |

| Title: | CE1-related: Colour-plane-based escape pixel coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shawmin Lei MediaTek Hsinchu: No. 1, Dusing Rd. 1, Hsinchu, Taiwan 30078 | Tel: Email: | Tzu-Der Chuang +886-3-5670766 ext. 24602 { peter.chuang, ching-yeh.chen, yuwen.huang, shawmin.lei }@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In HEVC residual decoding, the decoder decodes the transform block (TB) one by one among three colour components. Therefore, in HEVC decoder architecture, only one coefficient buffer for single colour component is required in entropy decoder. In SCC palette mode coding, there is no residual to be decoded, so the residual coefficient buffer can be reused to store the palette index map information, including palette index and escape values. However, in SCM-4.0 escape pixel coding, the escape values of three components of one sample are coded together. Therefore, three coefficient buffers for three colour components are required in the worst case, which increases the implementation cost and complexity of palette mode. This contribution proposes a colour-plane-based escape pixel coding to reduce the buffer requirement of escape pixels coding. The escape values of the same colour component are grouped together first and then signalled group by group. The entropy decoder can decode the escape values of one colour component at a time, and only one coefficient buffer for single colour component is required. The experimental results reportedly show that there is no BD-rate change.

# Problem statement

In HEVC residual decoding, the coefficients of different colour components are decoded separately. For a transform unit (TU), the coefficients of the Y component TB, the U component TB, and the V component TB are decoded in sequential. The decoder only decodes the TB of one colour component at a time. Therefore, in HEVC decoder architecture, only one coefficient buffer of one colour component is required in CABAC parser. As shown in Figure 1, in parsing stage, the parsed coefficients of the Y-TB are stored in the coefficient buffer in the entropy decoder. After parsing the coefficients of the Y-TB, the coefficient data are passed to the inverse quantization and inverse transform stage. The coefficient buffer in the entropy decoder can store the coefficients of the following U-TB. Only one coefficient buffer of one colour component is required in HEVC residual decoding.

Figure 1.  An example of coefficient decoding in a pipelined HEVC video decoder architecture

In SCC, there is no residual for palette mode. Therefore, the coefficient buffer can be reused to store the palette index map information, which includes palette index and escape values. However, in SCM-4.0 escape pixel coding, the escape values of different colour components are not coded seperately. The escape values of three colour components of one sample are coded together, as shown in Table 1. Therefore, three coefficient buffers are required to store parsed escape values of three colour components in entropy decoder in the worst case. It increases the implementation cost and complexity.

Table 1. The syntax table of escape values parsing in SCM-4.0

| if( palette_escape_val_present_flag ) { |  |
| --- | --- |
| sPos = 0 |  |
| while( sPos < nCbS * nCbS ) { |  |
| xC = x0 + travScan[ sPos ][ 0 ] |  |
| yC = y0 + travScan[ sPos ][ 1 ] |  |
| if( PaletteIndexMap[ xC ][ yC ] = = MaxPaletteIndex ) { |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| if( cIdx = = 0 ¦ ¦ ( xR % 2 = = 0 && yR % 2 = = 0 && ChromaArrayType = = 1 ) ¦ ¦ ( xR % 2 = = 0 && ChromaArrayType = = 2 ) ¦ ¦ ChromaArrayType = = 3 ) { |  |
| palette_escape_val | ae(v) |
| PaletteEscapeVal[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| } |  |
| sPos++ |  |
| } |  |
| } |  |

# Proposed method

In this contribution, a colour-plane-based escape pixel coding is proposed to reduce the buffer requirement in entropy decoder. The escape values of the same colour component are grouped together first, and the escape values groups of the different colour component are signalled group by group. The syntax table of the proposed method is shown in Table 2. In the proposed method, the entropy decoder only decodes the escape values of one colour component at a time, which is similar to the PCM coding in HEVC. By using the same schedule in Figure 1, only one coefficient buffer of one colour component is required for escape pixel coding.

Table 2. The syntax table of the proposed method

| if( palette_escape_val_present_flag ) { |  |
| --- | --- |
| sPos = 0 |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ){ |  |
| while( sPos < nCbS * nCbS ) { |  |
| xC = x0 + travScan[ sPos ][ 0 ] |  |
| yC = y0 + travScan[ sPos ][ 1 ] |  |
| if( PaletteIndexMap[ xC ][ yC ] = = MaxPaletteIndex ) { |  |
| for( cIdx = 0; cIdx < numComps; cIdx++ ) |  |
| if( cIdx = = 0 ¦ ¦ ( xR % 2 = = 0 && yR % 2 = = 0 && ChromaArrayType = = 1 ) ¦ ¦ ( xR % 2 = = 0 && ChromaArrayType = = 2 ) ¦ ¦ ChromaArrayType = = 3 ) { |  |
| palette_escape_val | ae(v) |
| PaletteEscapeVal[ cIdx ][ xC ][ yC ] = palette_escape_val |  |
| } |  |
| } |  |
| sPos++ |  |
| } |  |
| } |  |
| } |  |

# Experimental results

The proposed method is integrated on SCM-4.0 [1]. All tests were conducted under the common test condition (CTC) [2] for SCC with the full picture IBC search range. Table 1 and Table 2 show the lossy coding results of the proposed colour-plane-based escape pixel coding for full-frame IntraBC search and 4-CTU IntraBC search in 444 format, respectively. The lossless results of the proposed method in 444 format are shown in Table 3. The lossy and lossless results of the proposed method in 420 format are shown in Table 4 and Table 5, respectively. The experimental results reportedly show that the BD-rate is not changed. The runtimes are not accurate.

# Conclusions

This contribution proposes a colour-plane-based escape pixel coding to reduce the buffer requirement for escape pixels coding in entropy decoder. The escape values of the same colour component are grouped together first, and the escape values of the different colour component are signalled group by group. The entropy decoder can decode the escape values of one colour component at a time, which is similar to the PCM coding in HEVC. The experimental results reportedly show that the BD-rate is not changed.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- SCM-4.0, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.4+SCM-4.0/

- H. Yu, R. Cohen, K. Rapaka, J. Xu, “Common conditions for screen content coding tests,” Document of Joint Collaborative Team on Video Coding, JCTVC-T1015, Feb. 2015.

Table 1.  Lossy results of the proposed method in 444 format and full frame Intra Block Copy search

Table 2. Lossy results of the proposed method in 444 format and constrained Intra Block Copy seaerch (1x4 CTUs)

Table 3.  Lossless results of the proposed method in 444 format and full frame Intra Block Copy search

Table 4.  Lossy results of the proposed method in 420 format and full frame Intra Block Copy search

Table 5. Lossless results of the proposed method in 420 format and full frame Intra Block Copy search